3GPP TSG-RAN WG4 Meeting #107	R4-2310340

Incheon, South Korea, 22 – 26 May 2023

Source:	Ericsson, Rogers

Title:	TP for TR 37.718-21-11: to include band combination DC_2A-71A_n7A

Agenda item:			7.4.2

Document for:	Approval

# 1. Introduction

A text proposal for TR 37.718-21-11 to add DC_2A-71A_n7A configuration as specified in WID [1].

# 2. Text Proposal

---Start of changes---

## 5.x	DC_2-71_n7

### 5.x.1	Configurations for DC

Table 5.x.1-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-71A_n7A | DC_2A_n7A DC_71A_n7A |

### 5.x.2	Co-existence studies

Based on co-existence studies of DC_2_n7 and DC_71_n7, own Rx impact of the 3rd band is the followings.

-	2nd and 5th order IMD products generated by DC_2_n7 uplink may fall into own Rx of band 71.

### 5.x.3	∆TIB and ∆RIB values

For DC_2-71_n7, the TIB,c and RIB,c values are reused from DC_2-71_n41 are given in the tables below.

Table 5.X.3-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_2-71_n7 | 0.5 | 0.6 | 0.5 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 5.X.3-2: ΔRIB

| nter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_2-71_n7 | - | 0.2 | - |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

### 5.x.4	Reference sensitivity exceptions

Table 5.x.4-1 lists the MSD required for the dual connectivity configuration for the cases that IMD interference fall into the own 3rd Rx frequency band. The MSD values for DC_2A-71A_n41A are reused.

Table 5.x.4-1: MSD for the DC configuration

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A-71A_n7A | 2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | 71 | 676 | 5 | 50 | 630 | 28.7 | IMD24 |
|  | n7 | 2530 | 10 | 50 | 2650 | N/A | N/A |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

---End of changes---

# Reference

[1]		RP-230473, Dual Connectivity (DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL), Huawei