# 3GPP TSG-RAN WG4 Meeting # 107		                           							R4-2310364

# Incheon, KR, May 22 – May 26, 2023

Source:	Huawei, HiSilicon, CATT

Title:	TP for TR 38.718-02-01 to introduce BCS4 and BCS5 for CA_n78-n79

Agenda item:	7.10.2

Document for:	Approval

1 Background

This contribution provides some technical text proposals on the introduction BCS4 and BCS5 of the NR CA band combination CA_n78-n79 as defined in Revised WID: NR inter-band Carrier Aggregation/Dual connectivity for 2 bands DL with x bands UL (x=1,2) [1].

2 Text Proposal

---Start of changes---

5.X		CA_n78-n79

5.X.1	Common for 1 band UL and 2 bands UL CA

5.X.1.1 Operating bands for CA

Table 5.X.1.1-1: CA band combination of band n78+n79

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
| n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

5.X.1.2	Channel bandwidths per operating band for CA

Table 5.X.1.2-1: Supported bandwidths per CA band combination of band n78+n79

| CA operating/channel bandwidth [MHz] |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n78A-n79A | n78A n79A CA_n78A-n79A | n78 | See n78 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n78(2A)-n79A | CA_n78A-n79A | n78 | CA_n78(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |

5.X.1.3	REFSENS exception requirements

Since new minimum channel bandwidth 10MHz was introduced in Rel-18, the new MSD test point due to cross band isolation for band n79 10MHz can be introduced as below for BCS4 and 5 cases to overwrite the case with DL band n79 100MHz.

Table 5.X.1.3-1: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC3 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n783 | n79 | 3750 | 100 | 30 | 270 (RBstart=3) | 4405 | 10 | 2 | >ACLR2 |
| NOTE 3: The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. |  |  |  |  |  |  |  |  |  |

---End of changes---

Reference

[1]	RP-230133, “Revised WID: Rel-18 NR inter-band Carrier Aggregation/Dual connectivity for 2 bands DL with x bands UL (x=1,2)”, ZTE Corporation