3GPP TSG-RAN WG4 Meeting # 107												R4-2310376

Incheon, KR, May 22nd – May 26th , 2023

Title: 	WF on NR-U Full and Partial RB allocations

Agenda Item:	7.31.6

Source: 	Apple, Skyworks

Document for:	Approval

# MPR

## Explanation of Full and Partial MPR

Definition of Full and Partial RB allocations is clarified in the following.

During specification work on PC3 MPR issues were raised on the correct understanding on definition of Full and Partial allocations. The description is found in Table 6.2F.2-1.

Observing Note 2 and Note 3 it can be observed that the Full and Partial RB allocations are defined by:

- ‘Full’ requires all sub-carriers to be occupied over the entire channel.

- In case of wideband operation ‘Full’ is defined by sub-band configuration A in Table 6.2F.2-2.

- ‘Partial’ is at least one RB not occupied (i.e.. interlaced) or is part of configuration B in Table 6.2F.2-2.

The differentiation between configuration A and configuration B for MPR is required due to issues with image rejection under wideband operation.

It is important to note that ‘Partial’ is not only referring to interlaced RB allocations but also includes wideband configurations where all RBs are filled in a 20MHz sub-band. For better understanding the figure below provides several examples for 80MHz channel and wideband setup.

Example breakdown for ‘Full’ according to Figure 1:

- Case a) belongs to ‘Full’ as all RBs allocated in the entire channel.

- Case c) is part of ‘Full’ as all RBs are allocated for wideband operation and it belongs to wideband configuration A

Example breakdown for ‘Partial’:

- The cases b), d), f) and h) belong to ‘Partial’ as the RB configuration is interlaced.

- The examples e) and g) are ‘Partial’: While all RBs are allocated in a 20MHz channel the wideband setup belongs to configuration B.

Figure 1: Showcases for ‘Full’ and ‘Partial’ with 80MHz channel and wideband setup.

## Wording of Full and Partial MPR

Based on the understanding above the wording for ‘Full RB allocation MPR’ and ‘Partial RB allocation MPR’ is updated for easier understanding.

The MPR for Full RB allocation applies

- to all RB’s in all transmitted 20 MHz or larger channels that are fully allocated

- or to all RB’s in all transmitted sub-bands for wideband operation that are fully allocated excluding the wideband configurations of Table 6.2F.2-2.

The MPR for Partial RB allocation applies

- to interlaced allocations with uplink resource allocation type 2 as specified in TS 38.214 [10]

- or to transmitted sub-bands for wideband operation are transmitted according to the wideband configurations of Table 6.2F.2-2.

<Agreement>: Apply wording to Table 6.2F.2-1 and remove wideband configuration A from Table 6.2F.2-2 and Table 6.2F.2A.2-2 as indicated below

Table 6.2F.2-1 Maximum power reduction (MPR) for shared spectrum access UE power class 5

| Pre-coding | Modulation | RB Allocation |  |  |
| --- | --- | --- | --- | --- |
|  |  | Full2 (dB) | Partial3 (dB) | Exception for 100MHz Full5 (dB) |
| DFT-s-ODFM | Pi/2 BPSK4 | ≤ 1.5 | ≤ 2.5 |  |
|  | QPSK | ≤ 1.5 | ≤ 2.5 |  |
|  | 16 QAM | ≤ 2.0 | ≤ 3.0 |  |
|  | 64 QAM | ≤ 3.5 | ≤ 4.5 |  |
|  | 256 QAM | ≤ 5.0 | ≤ 5.5 |  |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 3.5 | ≤ 4.5 |
|  | 16 QAM | ≤ 4.0 | ≤ 4.0 | ≤ 4.5 |
|  | 64 QAM | ≤ 5.5 | ≤ 5.5 |  |
|  | 256 QAM | ≤ 7.0 | ≤ 7.0 |  |
| NOTE 1: The MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: The MPR for Full RB allocation applies to all RB’s in all transmitted 20 MHz or larger channels that are fully allocated or all RB’s in all transmitted sub-bands for wideband operation that are fully allocated excluding the wideband configurations of Table 6.2F.2-2. NOTE 3: The MPR for Partial RB allocation applies to interlaced allocations with uplink resource allocation type 2 as specified in TS 38.214 [10] or transmitted sub-bands for wideband operation are transmitted according to the wideband configurations of Table 6.2F.2-2. NOTE 4: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. NOTE 5: Exception for 100MHz Full RB allocation MPR applies when all RB’s in all sub-bands for 100MHz wideband operation are fully allocated and sub-bands are transmitted according to the wideband configurations of Table 6.2F.2-2. |  |  |  |  |

Table 6.2F.2-2 Exception MPR mapping for wideband operation

| Wideband operation channel bandwidth (MHz) | Sub-band configuration exceptions |
| --- | --- |
| 40 | 10, 01 |
| 60 | None |
| 80 | 1100, 0011, 0100, 0010 |
| 100 | 00111, 11100, 00011, 11000 |
| NOTE 1: The sub-band configuration is represented as a bitmap where ‘1’ indicates that a sub-band is transmitted and ‘0’ indicates a sub-band is not transmitted. The bitmap is ordered with MSB mapped to the lowest frequency sub-band and LSB mapped to highest frequency sub-band within the wideband channel. NOTE 2: Void. |  |

Table 6.2F.2A.2-2 Exception MPR mapping for intra-band CA wideband operation

| Wideband operation channel bandwidth (MHz) | Sub-band configuration exceptions [CC1-CC2] |
| --- | --- |
| 20+20 | 1-0, 0-1 |
| 20+40 | None |
| 20+60 | 0-010, 0-100, 0-011, 1-100 |
| 20+80 | 1-1100, 0-0111 |
| 40+20 | None |
| 40+40 | 00-10, 01-00, 00-11, 11-00 |
| 40+60 | 11-100, 00-111 |
| 40+80 | 11-1000, 00-0111, 01-1000, 00-0110, 00-1000, 00-0100 |
| 60+20 | 001-0, 010-0, 001-1, 110-0 |
| 60+40 | 111-00, 001-11 |
| 60+60 | 111-000, 000-111, 011-000, 000-110, 001-000, 000-100 |
| 60+80 | 111-1000, 000-1111, 011-1000, 000-1110, 111-0000, 000-0111, 011-0000, 000-0110 |
| 80+20 | 1110-0, 0011-1 |
| 80+40 | 1110-00, 0001-11, 0110-00, 0001-10, 0010-00, 0001-00 |
| 80+60 | 1111-000, 0001-111, 0111-000, 0001-110, 1110-000, 0000-111, 0110-000, 0000-110 |
| 80+80 | 1111-1000, 0001-1111, 0111-1000, 0001-1110, 1111-0000, 0000-1111, 0111-0000, 0000-1110, 0011-0000, 0000-1100, 0001-0000, 0000-1000 |