3GPP TSG-RAN WG4 Meeting #107 										R4-2310395

Incheon, KR, May 22nd - May 26th, 2023

Source:	China Telecom

Title:	WF on RedCap HPUE RAN task

Agenda Item:	11.3

Document for:	Approval

- Background

In RAN #99, RAN tasked RAN4 the following work:

- To study how to enable HPUE including PC2 for RedCap in the band agnostic way

- To study the applicable form factor, e.g., sensor, camera

In RAN4 #106-bis-e, RAN4 discussed on the RAN task and reached the following agreements in the WF R4-2306650:

Issue 1: Form factor for PC2 RedCap HPUE

Candidate options for further discussion:

- Option 1: Not restrict the applicable form factor for PC2 RedCap HPUE in the specification

- Option 2: Clearly add the restriction on the applicable form factor for PC2 Redcap in the specification

Issue 2: How to specify HPUE including PC2 for RedCap

Way forward:

- PC2 RedCap HPUE requirements can be specified in the band agnostic way at least for TDD and HD-FDD.

  - The band agnostic way means the PC2 requirements for non-RedCap UE can be re-used for PC2 RedCap UE in a band agnostic way. Only the operating bands for which PC2 on non-RedCap UE has been introduced can be considered for RedCap UE.

- Further discuss whether the band agnostic way can be applied for PC2 FD-FDD bands with 1Rx and 2Rx, based on more analysis on the RF requirements including A-MPR and RSD.

- Note 1: 1Tx architecture is assumed for RedCap PC2.

- Note 2: PC2 for Rel-17 Redcap is considered under this RAN task.

- Way forward

Issue 1-1-1: A-MPR for FD-FDD RedCap PC2 UE

Way forward:

- Further discuss whether or not the A-MPR for non-Redcap PC2 UE can be directly re-used for Redcap PC2 UE

Issue 1-2-1: RSD for FD-FDD RedCap PC2 2Rx UE

Online agreement:

- RSD for non-RedCap 2Rx PC2 UE can be re-used for RedCap PC2 2Rx UE for the bands, on which RSD requirements for non-RedCap 2Rx PC2 are defined based on 1Tx architecture.

Issue 1-2-2: RSD for FD-FDD RedCap PC2 1Rx UE

Way forward:

- Further discuss whether or not the Single Rx antenna port RSD allowance for RedCap PC3 UE can be re-used for RedCap PC2 UE

Issue 1-3-1: RedCap PC2 UE for sensor and camera

Online agreement:

- RedCap PC2 UE is feasible for some form factors, e.g., sensor and camera

- FFS on whether RedCap PC2 UE is feasible for wearable

- FFS on whether the form factor for RedCap will be explicitly listed in the specification