3GPP TSG-RAN WG4 Meeting #107	R4-2310400

Online, April 17th – 26th, 2023

Title: 	WF on UE RF requirements for ATG

Agenda Item:	8.14.6

Source: 	Huawei, HiSilicon

Document for:	Approval

2 Agreement

<Agreement>: Define ATG UE requirements in separate subclause with suffix J in 38.101-1 for ATG UE with omni-direction antenna and with antenna array.

<Agreement>:

for ATG UE with phase antenna array, the Tx RF requirement should be defined on the sum of measurement of all TAB connectors.

for ATG UE with phase antenna array, the Rx RF requirement should be defined on top of each TAB connector.

<Agreement>: The rated maximum output power at maximum modulation order and full PRB configurations is reported by UE via new capability.

<Agreement>: The ATG UE maximum output power shall remain within +2dB and -2dB of the rated maximum output power at maximum modulation order and full PRB configurations indicated by new capability.

<Agreement>: There is no need to specify power class for ATG UE.

<Agreement>: Introduce the configured output power for ATG UE.

<Agreement>: UL 256QAM should be supported for ATG UE as optional feature for ATG.

Issue 1-2-6: ACLR

- Proposals

  - Option 1: to reuse FR1 PC3 ACLR requirement for ATG CPE.

  - Option 2: Others

- Recommended WF

  - Option 1

<Agreement>: Agree on Option 1.

Issue 1-2-7: Additional SEM for ATG CPE

- Proposals

  - Option 1:	propose not to define it unless there are any regulatory requirement if necessary

  - Option 2: Others

- Recommended WF

  - Option 1

<Agreement>: Agree on Option 1.

Issue 1-2-8: UE coexistence requirement for ATG CPE

- Proposals

  - Option 1: propose not to define it.

  - Option 2: Others.

- Recommended WF

  - TBD

<Agreement>: Agree on Option 1.