3GPP TSG-RAN4 Meeting #107	R4-2310434

# Incheon, Korea, 22-26 May 2023

Title: 	WF on additional CBWs for band n71

Agenda Item:	7.26

Source: 	Ericsson, Skyworks

Document for:	Approval

### PC3 REFSENS for 25 and 30MHz symmetric UL/DL CBW

Agreement:

Take the average values as REFSENS for SCS=15kHz

Derive the values for SCS = 30kHz based on values for SCS 15Khz by reducing 0.1dB.

Agreement:

REFSENS values:

| CBW | 25MHz | 30MHz |
| --- | --- | --- |
| 15kHz PC3 REFSENS | -74.8 | -67.1 |
| 30kHz PC3 REFSENS | -74.9 | -67.2 |

### PC2 REFSENS Degradation (RSD)

Agreement:

RSD values:

| CBW | 25MHz | 30MHz |
| --- | --- | --- |
| 1TX PC2 RSD | 2.9 | 3.5 |
| 2TX PC2 RSD | 7.3 | 8.1 |

Note: Above RSD values were calculated based on average of REFSENS for PC3, 1Tx PC2 and 2TX PC2.

The PC2 RSD will be handled in the HPUE FDD WI.

### How to capture the PC3 REFSENS and PC2 RSD into the spec

Agreement:

Adopt the method proposed in R4-2307060 for adding PC3 REFSENS and PC2 RSD requirements for the symmetric 25 and 30MHz CBW into TS 38.101-1.

FFS the exact wording when discussing the CR.

### Single-band UE requirements: A-MPR for NS_35

Agreement:

A-MPR is not needed for NS_35 (according to Table 2.2-1) in Band n71 with 25 MHz and 30 MHz uplink channel bandwidths.

### Capability signalling for n71 symmetric CBWs

Agreement:

For both n8 and n71 there is no need for new signalling to indicate support for symmetric channel bandwidths greater than 20 MHz. The existing signalling is sufficient.