3GPP TSG-RAN WG4 Meeting # 107												R4-2310446

Incheon, KR, May 22nd – May 26th , 2023

Agenda item:			8.20.4

Source:	Moderator (Qualcomm Incorporated)

Title:	Topic summary for [107][307] FS_NR_duplex_evo_Part2

Document for:	Information

# Introduction

Briefly introduce background, the scope of this email discussion (e.g., list of treated agenda items) and provide some guidelines for email discussion if necessary.

# Topic #1: FR1 TPs

We are coving the highlighted agenda items.

8.20 Study on evolution of NR duplex operation [FS_NR_duplex_evo] 8.20.1 General and work plan [FS_NR_duplex_evo] 8.20.2 Study the feasibility of and impact on RF requirements [FS_NR_duplex_evo] 8.20.2.1 Adjacent channel co-existence evaluation [FS_NR_duplex_evo] 8.20.2.2 Implementation feasibility of SBFD [FS_NR_duplex_evo] 8.20.2.2.1 Feasibility of FR1 BS aspects [FS_NR_duplex_evo] 8.20.2.2.2 Feasibility of FR2 BS aspects [FS_NR_duplex_evo] 8.20.2.2.3 Feasibility of FR1 UE aspects [FS_NR_duplex_evo] 8.20.2.2.4 Feasibility of FR2 UE aspects [FS_NR_duplex_evo] 8.20.2.3 Impacts on BS RF requirements [FS_NR_duplex_evo] * R1-2304183 LS on BS noise figure model for duplex evolution 8.20.2.4 Impacts on UE RF requirements [FS_NR_duplex_evo] 8.20.3 Summary of regulatory aspects [FS_NR_duplex_evo] 8.20.4 Moderator summary and conclusions [FS_NR_duplex_evo]

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-23xxxxx | Company A | Proposal 1: Observation 1: |

| R4-2307355 | Apple | Merge with other TPs for FR1 and for FR2 | TP on UE aspects for SBFD | Approval |
| --- | --- | --- | --- | --- |
| R4-2308266 | vivo | Various proposals (merge with other input on FR1) | TP on feasibility of FR1 UE aspects | Approval |
| R4-2308632 | Nokia, Nokia Shanghai Bell | Observation 1: AGC is not modelled if a fixed NF is used as a function of the input power. Proposal 1: Consider the change to the text proposal into the TR 38.858. Proposal 2: For new SBFD aware UE, sub-band filtering is not considered in FR1. Merge input with other TPs | TP to TR 38.858: Feasibility of FR1 UE aspects | Approval |
| R4-2308748 | Ericsson | Merge with other TPs on FR1 | TP to TR 38.858 on Feasibility of FR1 UE aspects | Approval |
| R4-2309018 (Available in inbox due to EOL problem uploading) | Qualcomm Incorporated | Merge with other TPs | TP to TR 38.858 on Feasibility of FR1 UE aspects | Approval |
| R4-2309166 | RICHTEK KOREA | Moderator: I don’t understand this document. It is a TP, and the only change mark is in the document info section at the top. Perhaps there is an error and proponent may want to provide some clarification. | TP to TR 38.858 on Feasibility of FR1 UE aspects | Approval |

## Open issues summary

Companies have proposed TPs for clarification and alignment with agreements in the last meeting.

### FR1 merged TP

Companies have proposed TPs for clarification and alignment with agreements in the last meeting.

Open issues and candidate options before meeting:

Issue 1-1: Aligning company proposals in FR1 TPs

- Recommended WF

  - Mediatek to prepare a tdoc considering the submissions

# Topic #2: FR2 TPs

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations | Title |
| --- | --- | --- | --- |
| R4-2307355 | TP on UE aspects for SBFD | Merge with other TPs for FR1 and for FR2 | TP on UE aspects for SBFD |
| R4-2308267 | vivo | Various updates, merge with other input on FR2 | TP on feasibility of FR2 UE aspects |
| R4-2308633 | Nokia, Nokia Shanghai Bell | Observation 1: AGC is not modelled if a fixed NF is used as a function of the input power. Proposal 1: Consider the change to the text proposal to TR 38.858. Merge with other TPs | TP to TR 38.858: Feasibility of FR2 UE aspects |
| R4-2308749 | Ericsson | Merge with other TPs on FR2 | TP to TR 38.858 on Feasibility of FR2-1 UE aspects |
| R4-2309019 (Available in inbox due to EOL problem uploading) | Qualcomm Incorporated | Merge with other TPs | TP to TR 38.858 feasibility of and impact on FR2 UE RF requirements |

## Open issues summary

Companies have proposed TPs for clarification and alignment with agreements in the last meeting.

### FR2 merged TP

Sub-topic description:

Open issues and candidate options before meeting:

Issue 2-1: Aligning company proposals in FR2 TPs

- Recommended WF

  - Qualcomm to prepare a revision of R4-2309019 considering the proposals in the TPs

# Topic #3: SBFD-aware UE

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations | Title |
| --- | --- | --- | --- |
| R4-2308196 | CMCC | Observation 1: for FR1, in extreme case, Rx interference at LNA input port from Tx sub-band is relatively large if without sub-band RF filtering assumption, e.g., maybe larger than max input power, when aggressor and victim UE are in very close proximity(1m) and aggressor transmit with max output power. Table 2. received power at Rx LNA input port if assume no sub-band RF filtering Received power from Tx sub-band/dB; From calibration date; calculated with 1m separation assumption. 5% of CDF; Average Minimum CL among all companies’ date; Minimum CL among all companies’ data. FR1; -93.4; -40; -22; -21. Observation 2: for FR1, in extreme case, Rx interference in Rx sub-band from Tx sub-band leakage is severe and maybe in the range -121.4~ -39dBm. Sub-band filter could help to reduce interference. Proposal 1: if it’s hard to conduct sub-band filtering for every FR1 SBFD-aware UE, sub-band filtering is not mandatory | study on feasibility at FR1 UE side |
| R4-2308197 | CMCC | Observation 1: Rx interference is less than specified max input power for FR2 even in extreme case when aggressor and victim UE are in very close proximity, i.e., 1m and aggressor transmit with max output power. Observation 2: for FR2, received interference in Rx sub-band from Tx sub-band leakage is in the range -54dBm ~-88dBm, max -4dB larger than thermal noise even in extreme case when aggressor and victim UE are in very close proximity, i.e., 1m and aggressor transmit with max output power. Sub-band filter could help to reduce interference. Proposal 1: if it’s hard to conduct sub-band filtering for every FR2 SBFD-aware UE, sub-band filtering is not mandatory | study on feasibility at FR2 UE side |
| R4-2308268 | vivo | Proposal 1: Clarify whether SBFD-aware UE is in the scope of SI FS_NR_duplex_evo. Proposal 2: Study whether the existing legacy UE interference models can be applied for SBFD-aware UEs case by case. Proposal 3: Further improve the subband/in-channel selectivity for UE co-channel Rx model. | Remaining issues for full duplex at UE side |
| R4-2308630 | Nokia, Nokia Shanghai Bell | Observation 1: Considering the existing requirements, subband filtering may be limited to the maximum channel bandwidths in TS 38.101-1 and TS 38.101-2. Observation 2: In current requirements for FR2-1, the ACS values might be as low as 20 dB. Proposal 1: RAN4 further study whether the level of 23dB (from ACS) can reflect the adjacent channel selectivity performance level for typical FR2 UE implementations. | Discussion on SBFD feasibility study and requirements impact from UE aspects |
| R4-2308632 | Nokia, Nokia Shanghai Bell | Observation 1: AGC is not modelled if a fixed NF is used as a function of the input power. Proposal 1: Consider the change to the text proposal into the TR 38.858. Proposal 2: For new SBFD aware UE, sub-band filtering is not considered in FR1. Merge input with other TPs | TP to TR 38.858: Feasibility of FR1 UE aspects |
| R4-2308747 | Ericsson | Observation 1: Whether to consider sub-filtering for SBFD-aware UE will be determined after co-existence and co-channel studies | Discussion on SBFD UE RF impact |
| R4-2309328 | Samsung | Observation-1: The following two questions shall be answered for non-SBFD-aware UE: - Question-1: In Rel-19 SBFD work item (if any), whether or not the above agreement means RAN4 shall not introduce new RAN4 requirement of sub-band selectivity for “non-SBFD-aware” UE in Rel-19? - Question-2: How RAN4 can conclude the impact on RF requirements by considering the inter-subband CLI, with assumed 33dB and 23dB for subband selectivity performance for non-SBFD-aware FR1 and FR2-1 UE respectively? Moderator: I am sorry I don’t understand the question. Can you clarify? Observation-2: The following two questions shall be answered for SBFD-aware UE: - Question-3: From UE implementation perspective, what is the achievable improvement on subband selectivity for SBFD-aware UE over non-SBFD-aware UE? - Question-4: From inter-subband CLI’s impact on UE performance perspective, what is the minimum subband selectivity performance for SBFD-aware UE? | Impact on UE RF requirements: Further Analysis |

## Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 3-1 Technical feasibility of FR1 UE sub-band filtering

Sub-topic description:

Open issues and candidate options before meeting:

Issue 3-1: Sub-band filtering aspects

- Proposals

  - Proposal 1: Agree for new SBFD aware UE, sub-band RF filtering is not considered in FR1. SAW/BAW/FBAR filter resonator Qs are not high enough for effective filtering. (Nokia 8632)

  - Observation 1 : only under very extreme case, total input power at Rx LNA input may be larger than -25dBm, i.e. when aggressor and victim is 1m distant and aggressor transmit with max power, therefore sub-band filtering is not mandatory (CMCC 8196)

  - Observation 2: A SBFD-aware UE filtering may be limited to the maximum CBW from TS 38.101-1 and -2. (Nokia 8630)

- Recommended WF

  - Companies may want to comment and discuss Proposal 1

  - Companies may want to comment and discuss Observation 1

  - Companies may want to comment and discuss Observation 2

### Sub-topic 3-2 Technical feasibility of FR2 UE sub-band filtering

Sub-topic description:

Open issues and candidate options before meeting:

Issue 3-2: Sub-band filtering aspects

- Proposals

  - Observation 1: total input power at Rx LNA input is always less than specified max input power, therefore sub-band filtering is not necessary/mandatory (CMCC 8197)

  - Observation 2: A SBFD-aware UE filtering may be limited to the maximum CBW from TS 38.101-1 and -2. (Nokia 8630)

- Recommended WF

  - Companies may want to comment and discuss Observation 1

  - Companies may want to comment and discuss Observation 2

### Sub-topic 3-3 SBFD-aware UE part of SI coexistence and co-channel system studies

Sub-topic description

Open issues and candidate options before meeting:

Issue 3-3: Clarify whether SBFD-aware UE is part of coexistence and co-channel system studies for SI FS_NR_duplex_evo (vivo 8268)

- Proposals

  - Option 1: SBFD-aware UEs to be included in coexistence and co-channel system studies during the SI ( note: how to model a SBFD-aware UE is not clear at this time)

  - Option 2: FFS on inclusion of SBFD-aware UEs in coexistence and co-channel system studies until legacy UEs studies are complete (Ericsson 8747)

  - Option 3: SBFD-aware UEs not included in coexistence and co-channel system studies

- Recommended WF

  - Companies to discuss their understanding of the work and the 3 options

### Sub-topic 3-4 Company views rel-19 and SBFD-aware UEs

Deciding anything about rel-19 work is not in the scope of RAN4, it is a plenary decision,  but if companies have any opinions they would like to share on the topic they may do so.

Open issues and candidate options before meeting:

Issue 3-4: Would companies support SBFD-aware UE be part of potential rel-19 WI? This is just an informal poll (motivated by Samsung 9328)

- Proposals

  - Option 1: Would support including SBFD-aware UEs as part of a potential rel-19 WI

  - Option 2: Not sure

- Recommended WF

  - Companies may want to comment on the options. This is an informal poll.

### Sub-topic 3-5  SBFD-aware UE implementation and performance

Samsung 9328 asks some questions on SBFD-aware UE selectivity improvement. Technical analysis discussion and TPs to the TR are welcome on  this topic.

- Recommended WF

  - As before, technical analysis discussion and TP proposals to the TR are welcome on SBFD-UE implementation feasibility.

# Topic #4: Other

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations | Title |
| --- | --- | --- | --- |
| R4-2308630 | Nokia, Nokia Shanghai Bell | Observation 1: Considering the existing requirements, subband filtering may be limited to the maximum channel bandwidths in TS 38.101-1 and TS 38.101-2. Observation 2: In current requirements for FR2-1, the ACS values might be as low as 20 dB. Proposal 1: RAN4 further study whether the level of 23dB (from ACS) can reflect the adjacent channel selectivity performance level for typical FR2 UE implementations. | Discussion on SBFD feasibility study and requirements impact from UE aspects |

## Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 4-1 Value of adjacent channel selectivity for FR2 UE

Sub-topic description: Proponent wants to revisit RAN4 agreement on 23 dB selectivity for adjacent channel

Moderator: Here are some of the agreements on adjacent channel selectivity

RAN4#104bise(R4-2217513) - For FR2-1 use same method as FR1. - 23 dB value agreed as a performance point RAN4#105 (R4-2220245. For expediency chair notes captured that items labeled ‘proposed agreement’ are agreed) - Typical values to be used for selectivity For FR2-1: Pinterference_adjacent_channel_FR2-1 = Pinterferer – (23 dB + 10*log10(max(1,BWinterference /BWvictim_subband)))

Open issues and candidate options before meeting:

Issue 3-1: FR2 adjacent channel selectivity

- Proposals

  - Proposal 1: Reopen RAN4 agreement on adjacent channel ACS value for further discussion (Nokia 8630)

- Recommended WF

  - Keep previous RAN4 agreement of 23 dB for adjacent channel selectivity