3GPP RAN WG4 Meeting #107	R4-2310472

# Seoul, South Korea, May 22 - 26th 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.849 | CR | 0003 | rev | 2 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of new countries with associated NS values and A-MPR back-off |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic_enh-Core |  |  |  |  |  | Date: |  |  | 2023-05-24 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Updated summary of the NS values and associated A-MPR values (for the LPI and VLP mode) for new countries that have released regulatory rules for the 6GHz band. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The NS values and A-MPR values are added for the following countries Region 1: UK, Morocco, UAE, Saudi Arabia, Kenya, Qatar, Jordan Region 2: Canada, Brazil, Costa Rica, Dominican Republic Region 3: Hong Kong, Australia, New Zealand, Japan, Malaysia Adding NS_66 tentativly to US VLP in Table 6.1.1-1 . |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The 6GHz TR will not contain information for new countries that have released regulatory rules for the 6GHz unlicensed band. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1, 6.1.1.2.4(new), 6.1.1.2.5(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 6	RF requirements

## 6.1	UE specific

### 6.1.1	Transmitter characteristics

This section details specific transmitter characteristics for a UE operating in the 6 GHz NR band.

Table 6.1.1-1: Summary of NS values.

| Country | Mode |  |  |
| --- | --- | --- | --- |
|  | SP | LPI | VLP |
| Region 1 |  |  |  |
| EU/CEPT | N/A | NS_58 | NS_64 |
| UK | N/A | NS_01 | NS_65 |
| Morocco | N/A | NS_01 | NS_65 |
| UAE | N/A | NS_01 | N/A |
| Saudi Arabia | N/A | NS_01 | N/A |
| Kenya | N/A | NS_01 | NS_68 |
| Qatar | N/A | NS_01 | NS_65 |
| Jordan | N/A | NS_01 | NS_65 |
| Russian Federation | N/A | NS_01 | NS_68 |
| Region 2 |  |  |  |
| US | NS_54 | NS_53 | [NS_66] |
| Canada | NS_54 | NS_59 | NS_66 |
| Brazil | N/A | NS_53 | NS_67 |
| Peru | N/A | NS_53 | N/A |
| Chile | N/A | NS_53 | N/A |
| Costa Rica | N/A | NS_01 | NS_y2 |
| Colombia | N/A | NS_53 | N/A |
| Dominican Republic | N/A | NS_60 | NS_66 |
| Region 3 |  |  |  |
| South Korea | N/A | NS_60 | [NS_61] |
| Hong Kong | N/A | NS_58 | NS_64 |
| Australia | N/A | NS_01 | NS_68 |
| New Zealand | N/A | NS_01 | NS_68 |
| Malaysia | N/A | NS_01 | NS_65 |
| Japan | N/A | NS_63 | NS_69 |

******************** NEXT CHANGED SECTION ********************

#### 6.1.1.1	A-MPR for a NS(s) for lower 6GHz NR unlicensed operation

#### 6.1.1.1.1	EU/CEPT

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band as detailed in TS 38.101-1.

To meet the additional requirements applicable in EU as given in EN 303 687, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.1.1.1.1-1 for low power indoor (LPI) with PC5 and in Table 6.1.1.1.1-1b for PC3.

Table 6.1.1.1.1-1:  A-MPR for PC5 LPI

| Pre-coding | Modulation | RB Allocation |  |
| --- | --- | --- | --- |
|  |  | Full2 (dB) | Partial3 (dB) |
| DFT-s-ODFM | Pi/2 BPSK4 | ≤ 1.5 | ≤ 2.5 |
|  | QPSK | ≤ 2.0 | ≤ 3.5 |
|  | 16 QAM | ≤ 2.5 | ≤ 4.0 |
|  | 64 QAM | ≤ 3.5 | ≤ 4.5 |
|  | 256 QAM | ≤ 5.0 | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 4.5 |
|  | 16 QAM | ≤ 4.0 | ≤ 4.5 |
|  | 64 QAM | ≤ 5.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. The MPR applies to interlaced allocations with uplink resource allocation type 2 as specified in TS 38.214 [10]. NOTE 2: Full RB allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and sub-bands are transmitted according to configuration A in Table 6.2F.2-2. NOTE 3: Partial RB allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when the transmitted sub-bands for wideband operation are transmitted according to configuration B in Table 6.2F.2-2. NOTE 4: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. NOTE 5: The A-MPR applies instead of MPR for 20 MHz channel centered at the nearest NR-ARFCN corresponding to 5955 MHz, 40 MHz channel at the nearest NR-ARFCN corresponding to 5965 MHz, 60 MHz channel at the nearest NR-ARFCN corresponding to 5975 MHz, and 80 MHz channel at the nearest NR-ARFCN corresponding to 5985 MHz. For all other channels, A-MPR is zero and MPR as specified in Table 6.2F.2-1 applies. |  |  |  |

Table 6.1.1.1.1-1a:  Signal setup

| ID | Modulation | Waveform | Allocation |
| --- | --- | --- | --- |
| 1 | Pi/2 BPSK | DFT-s-OFDM | Full |
| 2 | Pi/2 BPSK | DFT-s-OFDM | Interlaced |
| 3 | Pi/2 BPSK | DFT-s-OFDM | Wideband |
| 4 | Pi/2 BPSK | DFT-s-OFDM | Wideband Interlaced |
| 5 | QPSK | DFT-s-OFDM | Full |
| 6 | QPSK | CP-OFDM | Full |
| 7 | QPSK | DFT-s-OFDM | Interlaced |
| 8 | QPSK | CP-OFDM | Interlaced |
| 9 | QPSK | DFT-s-OFDM | Wideband |
| 10 | QPSK | CP-OFDM | Wideband |
| 11 | QPSK | DFT-s-OFDM | Wideband Interlaced |
| 12 | QPSK | CP-OFDM | Wideband Interlaced |
| 13 | 16QAM | DFT-s-OFDM | Full |
| 14 | 16QAM | CP-OFDM | Full |
| 15 | 16QAM | DFT-s-OFDM | Interlaced |
| 16 | 16QAM | CP-OFDM | Interlaced |
| 17 | 16QAM | DFT-s-OFDM | Wideband |
| 18 | 16QAM | CP-OFDM | Wideband |
| 19 | 16QAM | DFT-s-OFDM | Wideband Interlaced |
| 20 | 16QAM | CP-OFDM | Wideband Interlaced |
| 21 | 64QAM | DFT-s-OFDM | Full |
| 22 | 64QAM | CP-OFDM | Full |
| 23 | 64QAM | DFT-s-OFDM | Interlaced |
| 24 | 64QAM | CP-OFDM | Interlaced |
| 25 | 64QAM | DFT-s-OFDM | Wideband |
| 26 | 64QAM | CP-OFDM | Wideband |
| 27 | 64QAM | DFT-s-OFDM | Wideband Interlaced |
| 28 | 64QAM | CP-OFDM | Wideband Interlaced |
| 29 | 256QAM | DFT-s-OFDM | Full |
| 30 | 256QAM | CP-OFDM | Full |
| 31 | 256QAM | DFT-s-OFDM | Interlaced |
| 32 | 256QAM | CP-OFDM | Interlaced |
| 33 | 256QAM | DFT-s-OFDM | Wideband |
| 34 | 256QAM | CP-OFDM | Wideband |
| 35 | 256QAM | DFT-s-OFDM | Wideband Interlaced |
| 36 | 256QAM | CP-OFDM | Wideband Interlaced |

Figure 6.1.1.1.1-1a: A-MPR simulation results for PC3 LPI in EU/CEPT.

Table 6.1.1.1.1-1b:  A-MPR for PC3 LPI (1TX)

| Pre-coding | Modulation | RB Allocation |  |
| --- | --- | --- | --- |
|  |  | Full2 (dB) | Partial3 (dB) |
| DFT-s-ODFM | Pi/2 BPSK4 | ≤ 1.5 | ≤ 4.5 |
|  | QPSK | ≤ 2.0 | ≤ 4.5 |
|  | 16 QAM | ≤ 2.5 | ≤ 4.5 |
|  | 64 QAM | ≤ 3.0 | ≤ 4.5 |
|  | 256 QAM | ≤ 4.5 | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 5.0 |
|  | 16 QAM | ≤ 4.0 | ≤ 5.0 |
|  | 64 QAM | ≤ 4.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 6.5 | ≤ 7.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. The MPR applies to interlaced allocations with uplink resource allocation type 2 as specified in TS 38.214 [10]. NOTE 2: Full RB allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and sub-bands are transmitted according to configuration A in Table 6.2F.2-2. NOTE 3: Partial RB allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when the transmitted sub-bands for wideband operation are transmitted according to configuration B in Table 6.2F.2-2. NOTE 4: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. NOTE 5: The A-MPR applies instead of MPR for 20 MHz channel centered at the nearest NR-ARFCN corresponding to 5955 MHz, 40 MHz channel at the nearest NR-ARFCN corresponding to 5965 MHz, 60 MHz channel at the nearest NR-ARFCN corresponding to 5975 MHz, 80 MHz channel at the nearest NR-ARFCN corresponding to 5985 MHz. For all other channels, A-MPR is zero and MPR as specified in Table 6.2F.2-1 applies. |  |  |  |

For very low power (VLP) operation the out-of-band emissions and in-band power spectral density requirements are much more restrictive than for LPI.  For PC5 VLP, a comprehensive set of A-MPR simulation results is provided below for 20, 40, 60, and 80 MHz channels.  In Figure 6.1.1.1.1-1 all channels in the band are represented while in Figure 6.1.1.1.1-2 lower edge channels are not illustrated since those are most impacted by the additional spurious emission requirement of -45 dBm/MHz.  The lower edge channels found to be impacted were the ones centered at 5955 MHz for 20 MHz channels, 5965 MHz for 40 MHz channels, 5975 MHz and 5995 MHz for 60 MHz channels, and 5985 MHz for 80 MHz channels.

Table 6.1.1.1.1-2.  Simulation scenarios for all CBW/SCS

| Scenario | Modulation | DFT/CP | Allocation |
| --- | --- | --- | --- |
| 1 | QPSK | CP | Interlace_0 |
| 2 | QPSK | DFT-S | Interlace_0 |
| 3 | QPSK | CP | Full |
| 4 | QPSK | DFT-S | Full |
| 5 | 16QAM | CP | Interlace_0 |
| 6 | 16QAM | DFT-S | Interlace_0 |
| 7 | 16QAM | CP | Full |
| 8 | 16QAM | DFT-S | Full |
| 9 | 64QAM | CP | Interlace_0 |
| 10 | 64QAM | DFT-S | Interlace_0 |
| 11 | 64QAM | CP | Full |
| 12 | 64QAM | DFT-S | Full |
| 13 | 256QAM | CP | Interlace_0 |
| 14 | 256QAM | DFT-S | Interlace_0 |
| 15 | 256QAM | CP | Full |
| 16 | 256QAM | DFT-S | Full |

Figure 6.1.1.1.1-1. Power backoff for 20, 40, 60, and 80 MHz channels

Figure 6.1.1.1.1-2. Power backoff with lower edge channels excluded

Based on these simulation results, the A-MPR table for VLP is provided below in Table 6.1.1.1.1-3.

Table 6.1.1.1.1-3.  PC5 A-MPR table for VLP

| Pre-coding | Modulation | RB Allocation (Note 2) |  | RB Allocation (Note 3) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| DFT-s-ODFM | QPSK | ≤ 12 | ≤ 14 | ≤ 8 | ≤ 10 |
|  | 16 QAM | ≤ 12 | ≤ 15 | ≤ 9 | ≤ 10 |
|  | 64 QAM | ≤ 12 | ≤ 15 | ≤ 11 | ≤ 11 |
|  | 256 QAM | ≤ 13 | ≤ 15 | ≤ 13 | ≤ 14 |
| CP-OFDM | QPSK | ≤ 13 | ≤ 15 | ≤ 10 | ≤ 10 |
|  | 16 QAM | ≤ 13 | ≤ 15 | ≤ 11 | ≤ 10 |
|  | 64 QAM | ≤ 13 | ≤ 15 | ≤ 13 | ≤ 12 |
|  | 256 QAM | ≤ 15 | ≤ 15 | ≤ 15 | ≤ 15 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. NOTE 2: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5955 MHz, 40 MHz channels centered at the nearest NR-ARFCN corresponding to 5965 MHz, 60 MHz channels centered at the nearest NR-ARFCN corresponding to 5975 and 5995 MHz and 80 MHz channels centered at the nearest NR-ARFCN corresponding to 5985 MHz. NOTE 3: Applicable for all valid channels other than those enumerated under NOTE 2. |  |  |  |  |  |

##### 6.1.1.2.2	Australia and New Zealand

Table 6.1.1.2.2-1: Signal setup (LPI).

| ID | Modulation | Waveform | Allocation |
| --- | --- | --- | --- |
| 1 | QPSK | DFT-s-OFDM | Full |
| 2 | QPSK | CP-OFDM | Full |
| 3 | QPSK | DFT-s-OFDM | Interlaced |
| 4 | QPSK | CP-OFDM | Interlaced |
| 5 | QPSK | DFT-s-OFDM | Wideband |
| 6 | QPSK | CP-OFDM | Wideband |
| 7 | QPSK | DFT-s-OFDM | Wideband Interlaced |
| 8 | QPSK | CP-OFDM | Wideband Interlaced |
| 9 | 16QAM | DFT-s-OFDM | Full |
| 10 | 16QAM | CP-OFDM | Full |
| 11 | 16QAM | DFT-s-OFDM | Interlaced |
| 12 | 16QAM | CP-OFDM | Interlaced |
| 13 | 16QAM | DFT-s-OFDM | Wideband |
| 14 | 16QAM | CP-OFDM | Wideband |
| 15 | 16QAM | DFT-s-OFDM | Wideband Interlaced |
| 16 | 16QAM | CP-OFDM | Wideband Interlaced |
| 17 | 64QAM | DFT-s-OFDM | Full |
| 18 | 64QAM | CP-OFDM | Full |
| 19 | 64QAM | DFT-s-OFDM | Interlaced |
| 20 | 64QAM | CP-OFDM | Interlaced |
| 21 | 64QAM | DFT-s-OFDM | Wideband |
| 22 | 64QAM | CP-OFDM | Wideband |
| 23 | 64QAM | DFT-s-OFDM | Wideband Interlaced |
| 24 | 64QAM | CP-OFDM | Wideband Interlaced |
| 25 | 256QAM | DFT-s-OFDM | Full |
| 26 | 256QAM | CP-OFDM | Full |
| 27 | 256QAM | DFT-s-OFDM | Interlaced |
| 28 | 256QAM | CP-OFDM | Interlaced |
| 29 | 256QAM | DFT-s-OFDM | Wideband |
| 30 | 256QAM | CP-OFDM | Wideband |
| 31 | 256QAM | DFT-s-OFDM | Wideband Interlaced |
| 32 | 256QAM | CP-OFDM | Wideband Interlaced |

Figure 6.1.1.2.2-1: A-MPR simulation results for PC5 LPI in Australia.

Table 6.1.1.2.2-2: A-MPR for PC5 LPI in Australia

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz |  | 40 MHz |  | 60 MHz |  | 80 MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| DFT-s-ODFM | QPSK | ≤ 7.0 | ≤ 9.5 | ≤ 6.0 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 16 QAM | ≤ 7.0 | ≤ 9.5 | ≤ 6.0 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 64 QAM | ≤ 7.0 | ≤ 9.5 | ≤ 6.0 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 256 QAM | ≤ 7.0 | ≤ 9.5 | ≤ 6.0 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
| CP-OFDM | QPSK | ≤ 7.0 | ≤ 9.5 | ≤ 6.0 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 16 QAM | ≤ 7.0 | ≤ 9.5 | ≤ 6.0 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 64 QAM | ≤ 7.0 | ≤ 9.5 | ≤ 6.0 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 256 QAM | ≤ 7.0 | ≤ 9.5 | ≤ 6.0 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies. |  |  |  |  |  |  |  |  |  |

##### 6.1.1.2.3	Japan

Table 6.1.1.2.3-1: Signal setup (LPI and VLP).

| ID | Modulation | Waveform | Allocation |
| --- | --- | --- | --- |
| 1 | Pi/2 BPSK | DFT-s-OFDM | Full |
| 2 | Pi/2 BPSK | DFT-s-OFDM | Interlaced |
| 3 | Pi/2 BPSK | DFT-s-OFDM | Wideband |
| 4 | Pi/2 BPSK | DFT-s-OFDM | Wideband Interlaced |
| 5 | QPSK | DFT-s-OFDM | Full |
| 6 | QPSK | CP-OFDM | Full |
| 7 | QPSK | DFT-s-OFDM | Interlaced |
| 8 | QPSK | CP-OFDM | Interlaced |
| 9 | QPSK | DFT-s-OFDM | Wideband |
| 10 | QPSK | CP-OFDM | Wideband |
| 11 | QPSK | DFT-s-OFDM | Wideband Interlaced |
| 12 | QPSK | CP-OFDM | Wideband Interlaced |
| 13 | 16QAM | DFT-s-OFDM | Full |
| 14 | 16QAM | CP-OFDM | Full |
| 15 | 16QAM | DFT-s-OFDM | Interlaced |
| 16 | 16QAM | CP-OFDM | Interlaced |
| 17 | 16QAM | DFT-s-OFDM | Wideband |
| 18 | 16QAM | CP-OFDM | Wideband |
| 19 | 16QAM | DFT-s-OFDM | Wideband Interlaced |
| 20 | 16QAM | CP-OFDM | Wideband Interlaced |
| 21 | 64QAM | DFT-s-OFDM | Full |
| 22 | 64QAM | CP-OFDM | Full |
| 23 | 64QAM | DFT-s-OFDM | Interlaced |
| 24 | 64QAM | CP-OFDM | Interlaced |
| 25 | 64QAM | DFT-s-OFDM | Wideband |
| 26 | 64QAM | CP-OFDM | Wideband |
| 27 | 64QAM | DFT-s-OFDM | Wideband Interlaced |
| 28 | 64QAM | CP-OFDM | Wideband Interlaced |
| 29 | 256QAM | DFT-s-OFDM | Full |
| 30 | 256QAM | CP-OFDM | Full |
| 31 | 256QAM | DFT-s-OFDM | Interlaced |
| 32 | 256QAM | CP-OFDM | Interlaced |
| 33 | 256QAM | DFT-s-OFDM | Wideband |
| 34 | 256QAM | CP-OFDM | Wideband |
| 35 | 256QAM | DFT-s-OFDM | Wideband Interlaced |
| 36 | 256QAM | CP-OFDM | Wideband Interlaced |

Figure 6.1.1.2.3-1: A-MPR simulation results for PC5 LPI in Japan.

Table 6.1.1.2.3-3a: A-MPR for PC5 LPI in Japan (lower edge)

| Pre-coding | Modulation | RB Allocation (Note 2) | RB Allocation (Note 3) |  |
| --- | --- | --- | --- | --- |
|  |  | Full/Partial | Full (dB) | Partial (dB) |
| DFT-s-OFDM | PI/2 BPSK4 | See Table PC5 MPR | ≤ 2.0 | ≤ 3.0 |
|  | QPSK |  | ≤ 2.5 | ≤ 3.5 |
|  | 16 QAM |  | ≤ 3.0 | ≤ 3.5 |
|  | 64 QAM |  | ≤ 3.5 | ≤ 4.5 |
|  | 256 QAM |  | ≤ 5.0 | ≤ 5.5 |
| CP-OFDM | QPSK |  | ≤ 4.5 | ≤ 5.0 |
|  | 16 QAM |  | ≤ 4.5 | ≤ 5.5 |
|  | 64 QAM |  | ≤ 5.5 | ≤ 5.5 |
|  | 256 QAM |  | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. NOTE 2: Applicable for all valid channels and bandwidths other than those enumerated in NOTE 3. NOTE 3: Applicable for 40 MHz channels centered at the nearest NR-ARFCN corresponding to [5965 MHz], 60 MHz channels centered at the nearest NR-ARFCN corresponding to [5975 and 5995 MHz], and 80 MHz channels centered at the nearest NR-ARFCN corresponding to [5985 MHz]. NOTE 4: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. NOTE 5: Channel bandwidth sizes of 60MHz and 100MHz are not applicable for this network signalling. |  |  |  |  |

Table 6.1.1.2.3-3b: A-MPR for PC5 LPI in Japan (upper edge)

| Pre-coding | Modulation | RB Allocation (Note 3) |  |
| --- | --- | --- | --- |
|  |  | Full (dB) | Partial (dB) |
| DFT-s-OFDM | PI/2 BPSK4 | ≤ 2.0 | ≤ 3.0 |
|  | QPSK | ≤ 2.0 | ≤ 3.0 |
|  | 16 QAM | ≤ 2.5 | ≤ 3.5 |
|  | 64 QAM | ≤ 3.5 | ≤ 4.5 |
|  | 256 QAM | ≤ 5.0 | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 4.5 |
|  | 16 QAM | ≤ 4.0 | ≤ 4.5 |
|  | 64 QAM | ≤ 5.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. NOTE 2: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to [6415 MHz], 40 MHz channels centered at the nearest NR-ARFCN corresponding to [6405 MHz] and 80 MHz channels centered at the nearest NR-ARFCN corresponding to [6385 MHz]. NOTE 3: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. NOTE 4: Channel bandwidth sizes of 60MHz and 100MHz are not applicable for this network signalling. |  |  |  |

Figure 6.1.1.2.3-2: A-MPR simulation results for PC5 VLP in Japan.

Table 6.1.1.2.3-3: A-MPR for PC5 VLP in Japan

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz |  | 40 MHz |  | 80 MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| DFT-s-ODFM | PI/2 BPSK2 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.5 | ≤ 6.5 | ≤ 7.0 |
|  | QPSK | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.5 | ≤ 6.5 | ≤ 7.0 |
|  | 16 QAM | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.5 | ≤ 6.5 | ≤ 7.0 |
|  | 64 QAM | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.5 | ≤ 6.5 | ≤ 7.0 |
|  | 256 QAM | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.5 | ≤ 6.5 | ≤ 7.0 |
| CP-OFDM | QPSK | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 8.0 | ≤ 6.5 | ≤ 8.5 |
|  | 16 QAM | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 8.0 | ≤ 6.5 | ≤ 8.5 |
|  | 64 QAM | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 8.0 | ≤ 6.5 | ≤ 8.5 |
|  | 256 QAM | ≤ 7.0 | ≤ 6.0 | ≤ 6.0 | ≤ 8.0 | ≤ 7.0 | ≤ 8.5 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies. NOTE 2: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. NOTE 3: Channel bandwidth sizes of 60MHz and 100MHz are not applicable for this network signalling. |  |  |  |  |  |  |  |

******************** NEXT CHANGED SECTION ********************

#### 6.1.1.2	A-MPR for a NS(s) for the full 6GHz NR unlicensed operation

##### 6.1.1.2.1	Canada

Table 6.1.1.2.1-1: Signal setup (LPI).

|  | ID | Waveform | BW | RB Setup | SCS |
| --- | --- | --- | --- | --- | --- |
| Full Allocation Single CC | 1 | DFT-s-OFDM | 20 | 100RB3 | 15 |
|  | 2 | DFT-s-OFDM | 20 | 100RB0 | 15 |
|  | 3 | DFT-s-OFDM | 20 | 50RB0 | 30 |
|  | 4 | DFT-s-OFDM | 40 | 216RB0 | 15 |
|  | 5 | DFT-s-OFDM | 60 | 162RB0 | 30 |
|  | 6 | DFT-s-OFDM | 80 | 216RB0 | 30 |
|  | 7 | CP-OFDM | 20 | 106RB0 | 15 |
|  | 8 | CP-OFDM | 20 | 51RB0 | 30 |
|  | 9 | CP-OFDM | 40 | 216RB0 | 15 |
|  | 10 | CP-OFDM | 60 | 162RB0 | 30 |
|  | 11 | CP-OFDM | 80 | 217RB0 | 30 |
| Interlaced Allocation Single CC | 12 | DFT-s-OFDM | 20 | 1RB0 every 10RBs (10x) | 15 |
|  | 13 | DFT-s-OFDM | 40 | 1RB0 every 10RBs (20x) | 15 |
|  | 14 | DFT-s-OFDM | 60 | 1RB0 every 10RBs (30x) | 30 |
|  | 15 | DFT-s-OFDM | 80 | 1RB0 every 5RBs (40x) | 30 |
|  | 16 | CP-OFDM | 20 | 1 RB0 every 10RBs (10x) | 15 |
|  | 17 | CP-OFDM | 40 | 1RB0 every 10RBs (22x) | 15 |
|  | 18 | CP-OFDM | 60 | 1RB0 every 10RBs (33x) | 30 |
|  | 19 | CP-OFDM | 80 | 1RB0 every 5RBs (44x) | 30 |

Figure 6.1.1.2.1-1: A-MPR simulation results for PC5 LPI in Canada.

Table 6.1.1.2.1-2: A-MPR for PC5 LPI in Canada

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |
| --- | --- | --- | --- |
|  |  | 20 MHz |  |
|  |  | Full (dB) | Partial (dB) |
| DFT-s-ODFM | PI/2 BPSK | ≤ 3.0 | ≤ 5.5 |
|  | QPSK | ≤ 3.0 | ≤ 5.5 |
|  | 16 QAM | ≤ 3.0 | ≤ 5.5 |
|  | 64 QAM | ≤ 3.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 5.0 | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 5.5 |
|  | 16 QAM | ≤ 4.0 | ≤ 5.5 |
|  | 64 QAM | ≤ 5.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 7.0 | ≤ 7.0 |

Table 6.1.1.2.1-3: Signal setup (VLP).

| ID | Modulation | Waveform | Allocation |
| --- | --- | --- | --- |
| 1 | QPSK | DFT-s-OFDM | Full |
| 2 | QPSK | CP-OFDM | Full |
| 3 | QPSK | DFT-s-OFDM | Interlaced |
| 4 | QPSK | CP-OFDM | Interlaced |
| 5 | QPSK | DFT-s-OFDM | Wideband |
| 6 | QPSK | CP-OFDM | Wideband |
| 7 | QPSK | DFT-s-OFDM | Wideband Interlaced |
| 8 | QPSK | CP-OFDM | Wideband Interlaced |
| 9 | 16QAM | DFT-s-OFDM | Full |
| 10 | 16QAM | CP-OFDM | Full |
| 11 | 16QAM | DFT-s-OFDM | Interlaced |
| 12 | 16QAM | CP-OFDM | Interlaced |
| 13 | 16QAM | DFT-s-OFDM | Wideband |
| 14 | 16QAM | CP-OFDM | Wideband |
| 15 | 16QAM | DFT-s-OFDM | Wideband Interlaced |
| 16 | 16QAM | CP-OFDM | Wideband Interlaced |
| 17 | 64QAM | DFT-s-OFDM | Full |
| 18 | 64QAM | CP-OFDM | Full |
| 19 | 64QAM | DFT-s-OFDM | Interlaced |
| 20 | 64QAM | CP-OFDM | Interlaced |
| 21 | 64QAM | DFT-s-OFDM | Wideband |
| 22 | 64QAM | CP-OFDM | Wideband |
| 23 | 64QAM | DFT-s-OFDM | Wideband Interlaced |
| 24 | 64QAM | CP-OFDM | Wideband Interlaced |
| 25 | 256QAM | DFT-s-OFDM | Full |
| 26 | 256QAM | CP-OFDM | Full |
| 27 | 256QAM | DFT-s-OFDM | Interlaced |
| 28 | 256QAM | CP-OFDM | Interlaced |
| 29 | 256QAM | DFT-s-OFDM | Wideband |
| 30 | 256QAM | CP-OFDM | Wideband |
| 31 | 256QAM | DFT-s-OFDM | Wideband Interlaced |
| 32 | 256QAM | CP-OFDM | Wideband Interlaced |

Figure 6.1.1.2.1-2: A-MPR simulation results for PC5 VLP in Canada.

Table 6.1.1.2.1-4: A-MPR for PC5 VLP in Canada

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz |  | 40 MHz |  | 60 MHz |  | 80 MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| DFT-s-ODFM | QPSK | ≤ 16.0 | ≤ 18.5 | ≤ 12.5 | ≤ 15.5 | ≤ 11.0 | ≤ 14.0 | ≤ 9.5 | ≤ 12.5 |
|  | 16 QAM | ≤ 16.0 | ≤ 18.5 | ≤ 12.5 | ≤ 15.5 | ≤ 11.0 | ≤ 14.0 | ≤ 9.5 | ≤ 12.5 |
|  | 64 QAM | ≤ 16.0 | ≤ 18.5 | ≤ 12.5 | ≤ 15.5 | ≤ 11.0 | ≤ 14.0 | ≤ 9.5 | ≤ 12.5 |
|  | 256 QAM | ≤ 16.0 | ≤ 18.5 | ≤ 12.5 | ≤ 15.5 | ≤ 11.0 | ≤ 14.0 | ≤ 9.5 | ≤ 12.5 |
| CP-OFDM | QPSK | ≤ 16.0 | ≤ 18.5 | ≤ 12.5 | ≤ 15.5 | ≤ 11.0 | ≤ 14.0 | ≤ 9.5 | ≤ 12.5 |
|  | 16 QAM | ≤ 16.0 | ≤ 18.5 | ≤ 12.5 | ≤ 15.5 | ≤ 11.0 | ≤ 14.0 | ≤ 9.5 | ≤ 12.5 |
|  | 64 QAM | ≤ 16.0 | ≤ 18.5 | ≤ 12.5 | ≤ 15.5 | ≤ 11.0 | ≤ 14.0 | ≤ 9.5 | ≤ 12.5 |
|  | 256 QAM | ≤ 16.0 | ≤ 18.5 | ≤ 12.5 | ≤ 15.5 | ≤ 11.0 | ≤ 14.0 | ≤ 9.5 | ≤ 12.5 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies. |  |  |  |  |  |  |  |  |  |

##### 6.1.1.2.2	South Korea

Table 6.1.1.2.2-1: Signal setup (LPI and VLP).

|  | ID | Waveform | BW | RB Setup | SCS |
| --- | --- | --- | --- | --- | --- |
| Full Allocation Single CC | 1 | DFT-s-OFDM | 20 | 100RB3 | 15 |
|  | 2 | DFT-s-OFDM | 20 | 100RB0 | 15 |
|  | 3 | DFT-s-OFDM | 20 | 50RB0 | 30 |
|  | 4 | DFT-s-OFDM | 40 | 216RB0 | 15 |
|  | 5 | DFT-s-OFDM | 60 | 162RB0 | 30 |
|  | 6 | DFT-s-OFDM | 80 | 216RB0 | 30 |
|  | 7 | CP-OFDM | 20 | 106RB0 | 15 |
|  | 8 | CP-OFDM | 20 | 51RB0 | 30 |
|  | 9 | CP-OFDM | 40 | 216RB0 | 15 |
|  | 10 | CP-OFDM | 60 | 162RB0 | 30 |
|  | 11 | CP-OFDM | 80 | 217RB0 | 30 |
| Interlaced Allocation Single CC | 12 | DFT-s-OFDM | 20 | 1RB0 every 10RBs (10x) | 15 |
|  | 13 | DFT-s-OFDM | 40 | 1RB0 every 10RBs (20x) | 15 |
|  | 14 | DFT-s-OFDM | 60 | 1RB0 every 5RBs (30x) | 30 |
|  | 15 | DFT-s-OFDM | 80 | 1RB0 every 5RBs (40x) | 30 |
|  | 16 | CP-OFDM | 20 | 1 RB0 every 10RBs (10x) | 15 |
|  | 17 | CP-OFDM | 40 | 1RB0 every 10RBs (22x) | 15 |
|  | 18 | CP-OFDM | 60 | 1RB0 every 5RBs (33x) | 30 |
|  | 19 | CP-OFDM | 80 | 1RB0 every 5RBs (44x) | 30 |

Figure 6.1.1.2.2-1: A-MPR simulation results for PC5 LPI in South Korea.

Table 6.1.1.2.2-2: A-MPR for PC5 LPI in South Korea

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz |  | 40 MHz |  | 60 MHz |  | 80 MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| DFT-s-ODFM | QPSK | ≤ 6.0 | ≤ 8.5 | ≤ 4.0 | ≤ 5.5 | ≤ 3.5 | ≤ 4.5 | ≤ 3.0 | ≤ 4.5 |
|  | 16 QAM | ≤ 6.0 | ≤ 8.5 | ≤ 4.0 | ≤ 5.5 | ≤ 4.0 | ≤ 5.0 | ≤ 3.5 | ≤ 5.0 |
|  | 64 QAM | ≤ 6.0 | ≤ 8.5 | ≤ 4.0 | ≤ 5.5 | ≤ 4.0 | ≤ 5.0 | ≤ 3.5 | ≤ 5.0 |
|  | 256 QAM | ≤ 6.0 | ≤ 8.5 | ≤ 5.0 | ≤ 5.5 | ≤ 5.0 | ≤ 5.5 | ≤ 5.0 | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 6.0 | ≤ 8.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.0 | ≤ 5.5 | ≤ 4.5 | ≤ 5.5 |
|  | 16 QAM | ≤ 6.0 | ≤ 8.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.0 | ≤ 5.5 | ≤ 4.5 | ≤ 5.5 |
|  | 64 QAM | ≤ 6.0 | ≤ 8.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 6.0 | ≤ 8.5 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 |

Table 6.1.1.2.2-3 contains A-MPR values for the 5945-6425MHz frequency range based on South Korea regulatory requirements.

Table 6.1.1.2.2-3: A-MPR for PC5 VLP in South Korea (5945-6425MHz).

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz |  | 40 MHz |  | 60 MHz |  | 80 MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| DFT-s-ODFM | QPSK | ≤ 7.5 | ≤ 10.0 | ≤ 6.5 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 16 QAM | ≤ 7.5 | ≤ 10.5 | ≤ 6.5 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 64 QAM | ≤ 7.5 | ≤ 10.5 | ≤ 6.5 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 256 QAM | ≤ 7.5 | ≤ 10.5 | ≤ 6.5 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
| CP-OFDM | QPSK | ≤ 7.5 | ≤ 10.0 | ≤ 6.5 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 16 QAM | ≤ 7.5 | ≤ 10.5 | ≤ 6.5 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 64 QAM | ≤ 7.5 | ≤ 10.5 | ≤ 6.5 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 256 QAM | ≤ 7.5 | ≤ 10.5 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies. |  |  |  |  |  |  |  |  |  |

Table 6.1.1.2.2-4 contains A-MPR values for the 5925-5945MHz frequency range based on South Korea regulatory requirements.

Table 6.1.1.2.2-4: A-MPR for PC5 VLP in South Korea (5925-5945MHz).

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz |  | 40 MHz |  | 60 MHz |  | 80 MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| DFT-s-ODFM | QPSK | ≤ 8.5 | ≤ 11.5 | ≤ 7.0 | ≤ 9.0 | ≤ 6.5 | ≤ 7.5 | ≤ 6.5 | ≤ 7.0 |
|  | 16 QAM | ≤ 8.5 | ≤ 11.5 | ≤ 7.0 | ≤ 9.0 | ≤ 6.5 | ≤ 7.5 | ≤ 6.5 | ≤ 7.0 |
|  | 64 QAM | ≤ 8.5 | ≤ 11.5 | ≤ 7.0 | ≤ 9.0 | ≤ 6.5 | ≤ 7.5 | ≤ 6.5 | ≤ 7.0 |
|  | 256 QAM | ≤ 8.5 | ≤ 11.5 | ≤ 7.0 | ≤ 9.0 | ≤ 6.5 | ≤ 7.5 | ≤ 6.5 | ≤ 7.0 |
| CP-OFDM | QPSK | ≤ 11 | ≤ 12.5 | ≤ 9.0 | ≤ 11.0 | ≤ 7.5 | ≤ 9.5 | ≤ 7.0 | ≤ 8.5 |
|  | 16 QAM | ≤ 11 | ≤ 12.5 | ≤ 9.0 | ≤ 11.0 | ≤ 7.5 | ≤ 9.5 | ≤ 7.0 | ≤ 8.5 |
|  | 64 QAM | ≤ 11 | ≤ 12.5 | ≤ 9.0 | ≤ 11.0 | ≤ 7.5 | ≤ 9.5 | ≤ 7.0 | ≤ 8.5 |
|  | 256 QAM | ≤ 11 | ≤ 12.5 | ≤ 9.0 | ≤ 11.0 | ≤ 7.5 | ≤ 9.5 | ≤ 7.0 | ≤ 8.5 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies. NOTE 2: This table is applicable when the channel raster point is extended so that first 20MHz, i.e. 5925-5945MHz, can be used by NR-U. |  |  |  |  |  |  |  |  |  |

Table 6.1.1.2.2-5 provides the A-MPR values for PC3 LPI with 1Tx based on South Korea regulatory requirements.

Table 6.1.1.2.2-5: A-MPR for PC3 LPI with 1Tx in South Korea

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz |  | 40 MHz |  | 60 MHz |  | 80 MHz |  | 100 MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| DFT-s-OFDM | Pi/2 BPSK2 | ≤ 9.5 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 4.5 | ≤ 7.0 | ≤ 3.0 | ≤ 6.0 | ≤ 2.5 | ≤ 5.0 |
|  | QPSK | ≤ 9.5 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 4.5 | ≤ 7.0 | ≤ 3.0 | ≤ 6.0 | ≤ 2.5 | ≤ 5.0 |
|  | 16 QAM | ≤ 9.5 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 4.5 | ≤ 7.0 | ≤ 3.0 | ≤ 6.0 | ≤ 2.5 | ≤ 5.0 |
|  | 64 QAM | ≤ 9.5 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 4.5 | ≤ 7.0 | ≤ 3.0 | ≤ 6.0 | ≤ 2.5 | ≤ 5.0 |
|  | 256 QAM | ≤ 9.5 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 4.5 | ≤ 7.0 | ≤ 4.5 | ≤ 6.0 | ≤ 4.5 | ≤ 5.0 |
| CP-OFDM | QPSK | ≤ 9.5 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 4.5 | ≤ 7.0 | ≤ 4.0 | ≤ 6.0 | ≤ 3.5 | ≤ 5.0 |
|  | 16 QAM | ≤ 9.5 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 4.5 | ≤ 7.0 | ≤ 4.0 | ≤ 6.0 | ≤ 3.5 | ≤ 5.0 |
|  | 64 QAM | ≤ 9.5 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 4.5 | ≤ 7.0 | ≤ 4.0 | ≤ 6.0 | ≤ 4.0 | ≤ 5.0 |
|  | 256 QAM | ≤ 9.5 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 4.5 | ≤ 7.0 | ≤ 5.5 | ≤ 6.5 | ≤ 5.5 | ≤ 6.0 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies NOTE 2: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. |  |  |  |  |  |  |  |  |  |  |  |

Table 6.1.1.2.2-6 provides the A-MPR values for PC3 LPI with 2Tx based on South Korea regulatory requirements.

Table 6.1.1.2.2-6: A-MPR for PC3 LPI with 2Tx in South Korea

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz |  | 40 MHz |  | 60 MHz |  | 80 MHz |  | 100 MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| DFT-s-OFDM | Pi/2 BPSK2 | ≤ 9.5 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 4.5 | ≤ 7.0 | ≤ 3.0 | ≤ 6.0 | ≤ 3.0 | ≤ 5.5 |
|  | QPSK | ≤ 9.5 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 4.5 | ≤ 7.0 | ≤ 3.0 | ≤ 6.0 | ≤ 3.0 | ≤ 5.5 |
|  | 16 QAM | ≤ 9.5 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 4.5 | ≤ 7.0 | ≤ 3.0 | ≤ 6.0 | ≤ 3.0 | ≤ 5.5 |
|  | 64 QAM | ≤ 9.5 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 4.5 | ≤ 7.0 | ≤ 3.0 | ≤ 6.0 | ≤ 3.0 | ≤ 5.5 |
|  | 256 QAM | ≤ 9.5 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 4.5 | ≤ 7.0 | ≤ 4.5 | ≤ 6.0 | ≤ 4.5 | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 9.5 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 5.0 | ≤ 7.0 | ≤ 4.5 | ≤ 6.0 | ≤ 4.5 | ≤ 5.5 |
|  | 16 QAM | ≤ 9.5 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 5.0 | ≤ 7.0 | ≤ 4.5 | ≤ 6.0 | ≤ 4.5 | ≤ 5.5 |
|  | 64 QAM | ≤ 9.5 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 5.0 | ≤ 7.0 | ≤ 4.5 | ≤ 6.0 | ≤ 4.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 9.5 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 5.0 | ≤ 7.0 | ≤ 5.5 | ≤ 6.5 | ≤ 5.5 | ≤ 6.0 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies NOTE 2: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. |  |  |  |  |  |  |  |  |  |  |  |

##### 6.1.1.2.3	Peru and Chile

Table 6.1.1.2.3-1: Signal setup (LPI).

|  | ID | Waveform | BW | RB Setup | SCS |
| --- | --- | --- | --- | --- | --- |
| Full Allocation Single CC | 1 | DFT-s-OFDM | 20 | 100RB3 | 15 |
|  | 2 | DFT-s-OFDM | 20 | 100RB0 | 15 |
|  | 3 | DFT-s-OFDM | 20 | 50RB0 | 30 |
|  | 4 | DFT-s-OFDM | 40 | 216RB0 | 15 |
|  | 5 | DFT-s-OFDM | 60 | 162RB0 | 30 |
|  | 6 | DFT-s-OFDM | 80 | 216RB0 | 30 |
|  | 7 | CP-OFDM | 20 | 106RB0 | 15 |
|  | 8 | CP-OFDM | 20 | 51RB0 | 30 |
|  | 9 | CP-OFDM | 40 | 216RB0 | 15 |
|  | 10 | CP-OFDM | 60 | 162RB0 | 30 |
|  | 11 | CP-OFDM | 80 | 217RB0 | 30 |
| Interlaced Allocation Single CC | 12 | DFT-s-OFDM | 20 | 1RB0 every 10RBs (10x) | 15 |
|  | 13 | DFT-s-OFDM | 40 | 1RB0 every 10RBs (20x) | 15 |
|  | 14 | DFT-s-OFDM | 60 | 1RB0 every 10RBs (30x) | 30 |
|  | 15 | DFT-s-OFDM | 80 | 1RB0 every 5RBs (40x) | 30 |
|  | 16 | CP-OFDM | 20 | 1 RB0 every 10RBs (10x) | 15 |
|  | 17 | CP-OFDM | 40 | 1RB0 every 10RBs (22x) | 15 |
|  | 18 | CP-OFDM | 60 | 1RB0 every 10RBs (33x) | 30 |
|  | 19 | CP-OFDM | 80 | 1RB0 every 5RBs (44x) | 30 |

Figure 6.1.1.2.3-1: A-MPR simulation results for PC5 LPI in Peru and Chile.

Table 6.1.1.2.3-2: A-MPR for PC5 LPI in Peru and Chile

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz |  | 40 MHz |  | 60 MHz |  | 80 MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| DFT-s-ODFM | QPSK | ≤ 9.0 | ≤ 12.0 | ≤ 6.0 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 3.0 | ≤ 5.5 |
|  | 16 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.0 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 3.0 | ≤ 5.5 |
|  | 64 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.0 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 3.0 | ≤ 5.5 |
|  | 256 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.0 | ≤ 8.5 | ≤ 5.0 | ≤ 6.5 | ≤ 5.0 | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 9.0 | ≤ 12.0 | ≤ 6.0 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 4.0 | ≤ 5.5 |
|  | 16 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.0 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 4.0 | ≤ 5.5 |
|  | 64 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.0 | ≤ 8.5 | ≤ 5.5 | ≤ 6.5 | ≤ 5.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 7.0 | ≤ 8.5 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 |

The A-MPR results for PC5 LPI in Peru and Chile are mostly identical to the A-MPR results defined by NS_53 (LPI in US). In some cases, A-MPR results are 0.5-1dB relaxed comparing to NS_53. Thus, accounting for the marginal difference between these A-MPR values and NS_53, the latter can be re-used to support Peru and Chile.

##### 6.1.1.2.4	Brazil

Table 6.1.1.2.4-1: Signal setup (VLP).

| ID | Modulation | Waveform | Allocation |
| --- | --- | --- | --- |
| 1 | QPSK | DFT-s-OFDM | Full |
| 2 | QPSK | CP-OFDM | Full |
| 3 | QPSK | DFT-s-OFDM | Interlaced |
| 4 | QPSK | CP-OFDM | Interlaced |
| 5 | QPSK | DFT-s-OFDM | Wideband |
| 6 | QPSK | CP-OFDM | Wideband |
| 7 | QPSK | DFT-s-OFDM | Wideband Interlaced |
| 8 | QPSK | CP-OFDM | Wideband Interlaced |
| 9 | 16QAM | DFT-s-OFDM | Full |
| 10 | 16QAM | CP-OFDM | Full |
| 11 | 16QAM | DFT-s-OFDM | Interlaced |
| 12 | 16QAM | CP-OFDM | Interlaced |
| 13 | 16QAM | DFT-s-OFDM | Wideband |
| 14 | 16QAM | CP-OFDM | Wideband |
| 15 | 16QAM | DFT-s-OFDM | Wideband Interlaced |
| 16 | 16QAM | CP-OFDM | Wideband Interlaced |
| 17 | 64QAM | DFT-s-OFDM | Full |
| 18 | 64QAM | CP-OFDM | Full |
| 19 | 64QAM | DFT-s-OFDM | Interlaced |
| 20 | 64QAM | CP-OFDM | Interlaced |
| 21 | 64QAM | DFT-s-OFDM | Wideband |
| 22 | 64QAM | CP-OFDM | Wideband |
| 23 | 64QAM | DFT-s-OFDM | Wideband Interlaced |
| 24 | 64QAM | CP-OFDM | Wideband Interlaced |
| 25 | 256QAM | DFT-s-OFDM | Full |
| 26 | 256QAM | CP-OFDM | Full |
| 27 | 256QAM | DFT-s-OFDM | Interlaced |
| 28 | 256QAM | CP-OFDM | Interlaced |
| 29 | 256QAM | DFT-s-OFDM | Wideband |
| 30 | 256QAM | CP-OFDM | Wideband |
| 31 | 256QAM | DFT-s-OFDM | Wideband Interlaced |
| 32 | 256QAM | CP-OFDM | Wideband Interlaced |

Figure 6.1.1.2.4-1: A-MPR simulation results for PC5 LPI in Brazil.

Table 6.1.1.2.4-2:  A-MPR for PC5 LPI in Brazil

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz |  | 40 MHz |  | 60 MHz |  | 80 MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| DFT-s-ODFM | QPSK | ≤ 13.0 | ≤ 15.5 | ≤ 9.5 | ≤ 12.5 | ≤ 8.0 | ≤ 11 | ≤ 6.5 | ≤ 9.5 |
|  | 16 QAM | ≤ 13.0 | ≤ 15.5 | ≤ 9.5 | ≤ 12.5 | ≤ 8.0 | ≤ 11 | ≤ 6.5 | ≤ 9.5 |
|  | 64 QAM | ≤ 13.0 | ≤ 15.5 | ≤ 9.5 | ≤ 12.5 | ≤ 8.0 | ≤ 11 | ≤ 6.5 | ≤ 9.5 |
|  | 256 QAM | ≤ 13.0 | ≤ 15.5 | ≤ 9.5 | ≤ 12.5 | ≤ 8.0 | ≤ 11 | ≤ 6.5 | ≤ 9.5 |
| CP-OFDM | QPSK | ≤ 13.0 | ≤ 15.5 | ≤ 9.5 | ≤ 12.5 | ≤ 8.0 | ≤ 11 | ≤ 6.5 | ≤ 9.5 |
|  | 16 QAM | ≤ 13.0 | ≤ 15.5 | ≤ 9.5 | ≤ 12.5 | ≤ 8.0 | ≤ 11 | ≤ 6.5 | ≤ 9.5 |
|  | 64 QAM | ≤ 13.0 | ≤ 15.5 | ≤ 9.5 | ≤ 12.5 | ≤ 8.0 | ≤ 11 | ≤ 6.5 | ≤ 9.5 |
|  | 256 QAM | ≤ 13.0 | ≤ 15.5 | ≤ 9.5 | ≤ 12.5 | ≤ 8.0 | ≤ 11 | ≤ 6.5 | ≤ 9.5 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies. |  |  |  |  |  |  |  |  |  |