3GPP TSG-RAN WG4 Meeting #107                                                               R4-2310445

Incheon, Korea Republic, May 22nd – 26th, 2023

Agenda item:			8.20.4

Source:	Moderator (Samsung)

Title:	Topic summary for [106bis-e][306] FS_NR_duplex_evo_Part1

Document for:	Information

# Introduction

This thread is on Rel-18 SI for Study on evolution of NR duplex operation, in which the following highlighted agenda items are supposed to be covered:

- Study on evolution of NR duplex operation [FS_NR_duplex_evo] - General and work plan [FS_NR_duplex_evo] - Study the feasibility of and impact on RF requirements [FS_NR_duplex_evo] - Adjacent channel co-existence evaluation [FS_NR_duplex_evo] - Implementation feasibility of SBFD [FS_NR_duplex_evo] - Feasibility of FR1 BS aspects [FS_NR_duplex_evo] - Feasibility of FR2 BS aspects [FS_NR_duplex_evo] - Feasibility of FR1 UE aspects [FS_NR_duplex_evo] - Feasibility of FR2 UE aspects [FS_NR_duplex_evo] - Impacts on BS RF requirements [FS_NR_duplex_evo] * R1-2304183 LS on BS noise figure model for duplex evolution - Impacts on UE RF requirements [FS_NR_duplex_evo] - Summary of regulatory aspects [FS_NR_duplex_evo] - Moderator summary and conclusions [FS_NR_duplex_evo]

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

# Topic #1: Implementation Feasibility of SBFD: FR1 BS

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2308530 | Ericsson | A proposal for the section 10.1 on background for analysis. The proposed text, which is applicable for both FR1 and FR2, aims to outline in general terms the RSIC and inter-sector co-channel interference tables and their use as well as some of the issues that are considered for the feasibility in general terms. |
| R4-2307248 | Kumu Networks | In this TP, we provide some text for some sub-section within section 10.2 to cover feasibility of FR1 wide area BS aspects. |
| R4-2307388 | CATT | TP for TR 38.858:“Feasibility of FR1 Local Area BS aspects” |
| R4-2307389 | CATT | TP for TR 38.858:“Feasibility of FR1 Wide Area BS aspects” |
| R4-2307756 | Huawei, HiSilicon | TP to TR 38.858: Self-interference analysis for FR1 Wide Area BS |
| R4-2307757 | Huawei, HiSilicon | TP to TR 38.858: Co-site inter-sector interference analysis for FR1 Wide Area BS |
| R4-2308195 | CMCC | Proposal 1: following NF modelling assumption is reasonable. Besides, even larger point B is also OK.  For FR1 MR: (A, B, C, D) = (-38, 10, -20, 19)  For FR1 MR: (A, B, C, D) = (-35, 13, -17, 22). Proposal 2: for the ACSBS/ICSBS which refer to baseband suppression should be at least larger than 50dB. Observation 1: it seems certain assumed spatial isolation value may lead to larger input power than assumed -43dBm blocking level if without additional RF RSI or RF sub-band filter solution. Proposal 3: RF RSI or RF sub-band filter solution would help to reduce input power and avoid blocking in some cases when spatial isolation is limited. It’s encouraged to have detailed analysis of RF RSI feasibility or RF sub-band filter solution feasibility in final TR. |
| R4-2308457 | Qualcomm CDMA Technologies | TP to TR 38.858: Feasibility of FR1 wide area BS aspects |
| R4-2308531 | Ericsson | TP to TR 38.858 section 10.2 Feasibility of FR1 BS aspects |
| R4-2309041 | Nokia, Nokia Shanghai Bell | Observation 1: RF SIC is not feasible due to intolerable complexity and overwhelming processing burden Observation 2: Sub-band filtering is not feasible due to reasons mentioned above Proposal 1: Include the provided TP into the TR 38.858. |
| R4-2309178 | ZTE Corporation | Proposal 1: to confirm that MR and LA SBFD with 1dB sensitivity degradation is feasible. Observation 1: FR1 antenna isolation among different sectors separated in the vertical domain on the mast are expected to be around 60dBc which is much less than 100dBc. |
| R4-2309326 | Samsung | Observation 1: Samsung’s input for co-site inter-sector co-channel CLI for FR1 BS is provided in the following table, in which the interference from one co-site sector can be suppressed to the level lower than noise floor by 5dB. TP to TR38.858 on SBFD Implementation feasibility of SBFD on FR1 BS aspects |

The moderator can suggest a limited number of papers which could be presented.

## Open issues summary

Before f2f meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 1-1: Remaining Issues

#### Issue 1-1-1: Noise figure modelling

[Moderator] As discussed over the past two RAN4 meetings, the noise figure modelling is provided in two reply LSs to RAN1 (R4-2302885 and R4-2306004).

<Captured in R4-2302885> - The noise figure model is provided as below: - X-axis: Total received power is the linear sum of all received power, including wanted signal, self-interference, inter-gNB interference and inter-sector interference. - Y-axis: noise figure - The values of A, B, C and D: - A = -43dBm - B = -25dBm - C = 5dB - D = 14dB - If the total received power is larger than B, the receiver will be blocked.

<Captured in R4-2306004> Furthermore, RAN4 made the following agreement on noise figure model (i.e., noise figure over average total input power, captured in RAN4 LS R4-2302885): - For typical FR1 WA BS implementation, companies have different views on sub-band filter, and based on companies’ proposal on the implementation with sub-band filter, the blocking performance can be improved around 10 dB. The following values of A and B can apply to the noise figure model if sub-band filter adopted. - The values of A, B, C and D: - A = -35dBm - B = -17dBm - C = 5dB - D = 14dB - Note: RAN4 has not reach consensus on the implementation feasibility of sub-band filter. RAN4 will further evaluate and update to RAN1 if needed. - For FR1 MR BS class, the following noise figure model can be used as starting point: - (A, B, C, D) = (-38, -20, 10, 19) - Point A is suggested to follow the in-band blocking requirement as specified in TS 38.104, i.e. -38dBm. - Point C is suggested to follow the assumed noise figure, i.e. 10dB. - For FR1 LA BS class, the following noise figure model can be used as starting point: - (A, B, C, D) = (-35, -17, 13, 22) - Point A is suggested to follow the in-band blocking requirement as specified in TS 38.104, i.e. -35dBm. - Point C is suggested to follow the assumed noise figure, i.e. 13dB. - For FR2-1 BS, the following noise figure model can be used as starting point: - The values of A, B, C and D: - A = [-58] dBm - B = [-40] dBm - C = 10dB - D = [19] dB

- Proposals/Observations from CMCC:

  - Proposal 1: following NF modelling assumption is reasonable. Besides, even larger point B is also OK.

    - For FR1 MR: (A, B, C, D) = (-38, -20, 10, 19)

    - For FR1 MR: (A, B, C, D) = (-35, -17, 13, 22)

#### Issue 1-1-2: RF SIC feasibility

[Moderator] Over RAN4#106-Bis-e and the past several RAN4 meetings, RF SIC feasibility is discussed extensively. The following agreement is captured in the WF R4-2306006 to encourage companies further study on the implementation complexity.

Issue 2-2-1: RF SIC Agreement - For the implementation feasibility of RF SIC for SBFD operation: - RF cancellation can be used in SBFD to mitigate self-interference pre-Rx LNA in terms of reducing non-linearity effects and overall self-interference residue. - FFS on the implementation complexity.

- Proposals/Observations from Nokia:

  - Observation 1: RF SIC is not feasible due to intolerable complexity and overwhelming processing burden.

#### Issue 1-1-3: Subband filtering

[Moderator] Over RAN4#106-Bis-e and the past several RAN4 meetings, the feasibility on subband filtering is discussed extensively. The following agreements and WFs are captured in the WF R4-2306006 and LS R4-2306004.

<Captured in R4-2306006> Issue 2-3-2: Impact on RSI from analogue sub-band filter Agreement: - FFS the impact on RSI from analogue sub-band filter - FFS the implementation feasibility of sub-band filter in WA and MR BS classes - If no consensus reached, companies are encouraged to provide per-company analysis in their own feasibility section. <Captured in R4-2306004> Furthermore, RAN4 made the following agreement on noise figure model (i.e., noise figure over average total input power, captured in RAN4 LS R4-2302885): - For typical FR1 WA BS implementation, companies have different views on sub-band filter, and based on companies’ proposal on the implementation with sub-band filter, the blocking performance can be improved around 10 dB. The following values of A and B can apply to the noise figure model if sub-band filter adopted. - The values of A, B, C and D: - A = -35dBm - B = -17dBm - C = 5dB - D = 14dB - Note: RAN4 has not reach consensus on the implementation feasibility of sub-band filter. RAN4 will further evaluate and update to RAN1 if needed.

- Proposals/Observations:

  - Observation 1 (CMCC): it seems certain assumed spatial isolation value may lead to larger input power than assumed -43dBm blocking level if without additional RF RSI or RF sub-band filter solution.

  - Proposal 1 (CMCC): RF RSI or RF sub-band filter solution would help to reduce input power and avoid blocking in some cases when spatial isolation is limited. It’s encouraged to have detailed analysis of RF RSI feasibility or RF sub-band filter solution feasibility in final TR.

  - Observation 2 (Nokia): Sub-band filtering is not feasible due to reasons mentioned above

- Analog sub-band filtering in the RF domain. It has been demonstrated by simulations, that in order to achieve meaningful inter-subband filtering, high insertion loss must be accepted. - High insertion loss before the LNA will increase the receiver noise figure, and negate any system gains of SBFD. - High insertion loss placed after the LNA does not reduce LNA linearity requirements. Improved linearity LNAs are needed that add to cost and power consumption, which leads to thermal management issues. Other challenges include: - Significant insertion loss → NF increase that can hardly be accommodated in the 1 dB desensitization budget. - Considerable transition band and temperature dependency of the passband position → larger guard band between the sub-bands needed and less BW is usable for UL - Group delay distortion close to the cut-off frequencies - Incompatibility with a typical multi-carrier gNB design - Increased complexity as switches are needed for by-pass in UL slots for full BW - Additional space needed in RX chain that is not available in typical gNB design - Overall additional power consumption which leads to thermal management issues - Frequency drift over temperature that will impact filter insertion loss and rejection performance, hence impacts the RX lineup performance - The new sub-band specific filters would be operator’s spectrum specific and locking the spectrum configuration for any further changes or tuning. The existing bandpass filters for the operating band would anyway be required, suggesting that the new filters would double the filter size for the UL antenna panel. This all means higher cost and complexity.

#### Issue 1-1-4: ACS and ICS

[Moderator] In RAN4#106-Bis-e, the following agreements and WFs are captured in the WF R4-2306006 and LS R4-2306004 on ACS and ICS for gNB.

<Captured in R4-2306006> Regarding RAN1 Agreement-3 in R1-2302087, the following agreement is agreed to be captured in this WF: - ACSBS in the discussion refer to baseband suppression only. - ACSBS in the discussion is not directly related to gNB ACS minimum requirement, and has no impact on existing gNB ACS minimum requirement. <Captured in R4-2306004> - For the in-channel selectivity (or subband selectivity) ICSUE: - ICSUE = 33dB for FR1 and ICSUE = 23dB for FR2 - Above values are for simulation usage purpose and only based on the assumption of same bandwidth between interference and wanted signals - Above values derived based on single company’s measurement data. - This agreement has no impact on RAN4 specified minimum RF requirements

- Proposals/Observations from CMCC:

  - Proposal 1: for the ACSBS/ICSBS which refer to baseband suppression should be at least larger than 50dB.

#### Issue 1-1-5: MR and LA-BS SBFD with 1dB sensitivity degradation

[Moderator] For 1dB desense target, the following agreement achieved for co-existence study:

<Captured in R4-2306006> Issue 2-4-4: RSIC Assumption for co-existence study Agreement: - For the self-interference modeling used for coexistence study: - RAN4 to confirm the adoption of the impact of self-interference modelling that was used in the calibration phase to be further used in the coexistence study (N = noise floor -6 dB) to study the impact of SBFD operation on the RF requirements. - Note: The feasibility study on RSIC not concluded yet and will be further evaluated in RAN4.

- Proposals/Observations from ZTE:

  - Proposal 1: to confirm that MR and LA SBFD with 1dB sensitivity degradation is feasible.

#### Issue 1-1-6: Further contribution on FR1 antenna isolation

[Moderator] Based on companies’ input, the spatial isolation for FR1 inter-sector antenna has been studied, with the following agreement captured in R4-2302922:

Agreement Inter-sector isolation value range - Regarding spatial isolation values, the following values have been proposed for macro BS in RAN4: - FR1: 62-93dB with 75dB being typical values. - FR2: 75-98dB with 88dB being typical values. - Some companies have proposed that isolating materials could be added between sectors to increase the isolation. RAN4 has not yet discussed the details of what kind of materials and the building practice or whether such approaches can be applied to outdoor sites. Further improvement over the spatial isolation is FFS. - In forthcoming meetings values for macro and other BS classes should be proposed

- [Moderator] Further input from ZTE in this meeting.

- Observations from ZTE:

  - Observation 1: FR1 antenna isolation among different sectors separated in the vertical domain on the mast are expected to be around 60dBc which is much less than 100dBc.

### Sub-topic 1-2: Text Proposal to TR 38.858

The following TPs are provided in this meeting:

| T-doc | Company | Section/Sub-section |
| --- | --- | --- |
| R4-2308530 | Ericsson | TP for Section 10.1 Background for analysis |
| R4-2307248 | Kumu Networks | TP for Section 10.2 Feasibility of FR1 Wide Area BS aspects |
| R4-2307388 | CATT | TP for Section 10.4 Feasibility of FR1 Local Area BS aspects |
| R4-2307389 | CATT | TP for Section 10.2 Feasibility of FR1 Wide Area BS aspects |
| R4-2307756 | Huawei, HiSilicon | TP for Section 10.2.1 Self-interference analysis for FR1 Wide Area BS |
| R4-2307757 | Huawei, HiSilicon | TP for Section 10.2.2 Co-channel inter-sub-band co-site inter-sector interference analysis for FR1 Wide Area BS |
| R4-2308457 | Qualcomm CDMA Technologies | TP for Section 10.2 Feasibility of FR1 Wide Area BS aspects |
| R4-2308531 | Ericsson | TP for Section 10.2 Feasibility of FR1 Wide Area BS aspects TP for Section 10.3 Feasibility of FR1 Medium Range BS aspects |
| R4-2309041 | Nokia, Nokia Shanghai Bell | TP for Section 10.2 Feasibility of FR1 Wide Area BS aspects |
| R4-2309178 | ZTE Corporation | TP for Section 10.3 Feasibility of FR1 Medium Range BS aspects TP for Section 10.4 Feasibility of FR1 Local Area BS aspects |
| R4-2309326 | Samsung | TP for Section 10.2 Feasibility of FR1 Wide Area BS aspects |

# Topic #2: Implementation Feasibility of SBFD: FR2 BS

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2307758 | Huawei, HiSilicon | TP to TR 38.858: Self-interference analysis for FR2 BS |
| R4-2307759 | Huawei, HiSilicon | TP to TR 38.858: Co-site inter-sector interference analysis for FR2 BS |
| R4-2308532 | Ericsson | TP to TR 38.858 section 10.4 Feasibility of FR2 BS aspects |
| R4-2308630 (Moved to AI) | Nokia, Nokia Shanghai Bell | Related to UE requirement, shall be discussed/summarized in thread 307. |
| R4-2308957 | Qualcomm CDMA Technologies | TP to TR 38.858: Feasibility of FR2 wide area BS aspects |

The moderator can suggest a limited number of papers which could be presented.

## Open issues summary

Before f2f meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 2-2: Text Proposal to TR 38.858

The following TPs are provided in this meeting:

| T-doc | Company | Section/Sub-section |
| --- | --- | --- |
| R4-2307758 | Huawei, HiSilicon | TP for Section 10.5.1 Self-interference analysis for FR1 Wide Area BS |
| R4-2307759 | Huawei, HiSilicon | TP for Section 10.5 Co-channel inter-sub-band co-site inter-sector interference analysis for FR2-1 Wide Area BS |
| R4-2308532 | Ericsson | TP for Section 10.5 Feasibility of FR2-1 Wide Area BS aspects |
| R4-2308957 | Qualcomm CDMA Technologies | TP for Section 10.5 Feasibility of FR2-1 Wide Area BS aspects |

# Topic #3: Impacts on BS RF requirements

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| Rev R4-2307390 | CATT | Proposal 1: OTA sensitivity should be defined only for the self-interference scenario. Proposal 2: The exact DESENS value can be discuss and decided in WI phase. The REFSENS formula can be written as following, PREFSENSdBm=-174dBm+10×log10(BW)+NF+IM+SNR+ xdB desens x value can be different for each BS class pending on the discussion. Proposal 3: For OTA sensitivity test, following current FR2 FDD BS core requirement and test approach could be ok. No TRP or EIRP requirement is defined for the OTA sensitivity requirement. Proposal 4: In-channel adjacent subband leakage ratio and in-channel adjacent subband selectivity requirements are defined for SBFD capable BS. No separate sub-band blocking requirement is needed. Observation1: SBFD and non-SBFD slots are configured statically in the real deployment. Proposal 5: No transmit ON/OFF power requirement is defined for SBFD Observation 2: The legacy Tx intermodulation requirement may be able to guarantee the SBFD Tx IMD performance. |
| R4-2307761 | Huawei, HiSilicon | • Question-1: Whether the above values of A, B, C and D can be used for all the BS classes in FR1? If not, what are the values of A, B, C and D for each of BS classes in FR1? - Answer from RAN4: For FR1 different set of values are used for each of BS classes. The values for FR1 MR BS and LA BS can be found in the LS R4-2306004. • Question-2: Whether fixed noise figure can be used for FR2-1 in RAN1 evaluation? If not, what are the values of A, B, C and D for BS classes in FR2-1? - Answer from RAN4: The noise figure model is also applicable to FR2-1 BS. The values of A, B, C and D for FR2-1 can be found in the LS R4-2306004. RAN4 confirms that the [] can be removed. • Question-3: The feasibility and applicable scenarios of improved noise figure, e.g., by introducing additional interference reduction techniques like subband filtering. - Answer from RAN4: The previous LS R4-2306004 already provides the values of A and B applied to the noise figure model if sub-band filter is adopted |
| R4-2308528 | Ericsson | In the previous sections we made the following observations: Observation 1 The requirement on OTA sensitivity does not ensure that the BS has no more than XdB degradation compared to its real sensitivity (if it’s real sensitivity is better than the minimum requirement). Observation 2 Despite observation 1, the requirement can be based on minimum sensitivity for minimum requirements purposes. Observation 3 The desensitization in SBFD slots depends on all sources of interference. If 1dB degradation is allowed for self-interference only then the degradation during operation will be greater than 1dB. Observation 4 TX sub-band leakage from other sectors or gNBs is not captured in the OTA sensitivity requirement Observation 5 Means to mitigate TX sub-band leakage degradation to the RX sub-band cannot be applied for other nodes. Observation 6 The OTA sensitivity requirement does not capture RX degradation due to TX sub-bands from other nodes. Observation 7 Measures to mitigate the impact of TX sub-band leakage from the own transmitter may not work for power coming from other nodes. Observation 8 There can still be switching between UL and DL slots for SBFD Observation 9 When switching between TX and RX for the UL sub-band, a guard period and requirements on transient times are needed to prevent interference to/from other nodes, same as when all PRB are switched between TX/RX. Observation 10 When parts of the array are switched between TX and RX, or from transmitting in sub-bands to the whole carrier, a transient time may be needed until the TX (or RX) is stable in the new configuration. Observation 11 The transmitter should be built to be robust to reverse intermodulation Observation 12 Meeting RX intermodulation in regular DL slots may be sufficient. Observation 13 In case TX intermodulation is tested during SBFD slots, this relates to exceptional conditions and there should be no requirement on the RX sub-band in such conditions. Observation 14 Co-location requirements are standardized so that operators can deploy BS in different bands with confidence that proprietary solutions are not needed. Observation 15 Co-existence between bands requirements are standardized to ensure that deployments in different bands at different sites will not interfere to one another. Observation 16 Co-location and co-existence between bands requirements are optional and support is declared. Observation 17 Regardless of SBFD or non-SBFD, co-location requirements between bands do not assume synchronization between TDD patterns and are met by means of a band-wide analogue filter. Observation 18 It does not make sense to standardize relaxed co-location/co-existence requirements, since meeting them would not mean that proprietary solutions would not be needed. Based on the discussion in the previous sections we propose the following: Proposal 1 RAN4 discuss a reasonable degradation for self-interference only, considering an reasonable headroom for degradations from other interference sources. Proposal 2 Define the OTA sensitivity requirement over the EIRP accuracy directions range and OTA sensitivity RoAoA. Conformance testing should be in some EIRP testing directions and some OTA sensitivity RoAoA test directions. Proposal 3 Consider ACLR (scaled according to bandwidths) as a baseline for a TX sub-band leakage requirement. Proposal 4 Consider ACS as a baseline for an RX sub-band selectivity requirement. Proposal 5 For TX switching, consider transient times for RX sub-band switching and also for switching parts of the array. Proposal 6 Maintain co-location and co-existence between bands requirements that are optional for SBFD BS. Proposal 7 Adjust UL dynamic range requirements as needed. |
| R4-2308529 | Ericsson | Proposal 1 : No need to reply to the RAN1 LS |
| R4-2308628 | Nokia, Nokia Shanghai Bell | Observation 1: Question 1 in RAN1 LS was already answered by RAN4. Proposal 1: It is proposed to send LS to RAN1 and clarify on the feasibility and applicable scenarios of improved noise figure. |
| R4-2309042 | Nokia, Nokia Shanghai Bell | Observation 1: If all interferences are considered, then 1dB is accepted. If only self-interference is considered, then 0.5dB is accepted. Proposal 1: Use up to 0.5dB for self-interference only and maximum of 1dB for considering desensitization from all interferences Observation 2: Additional requirements are needed to be defined as they cannot be implicitly guaranteed by OTA sensitivity Proposal 2: There is a need to define additional requirements within SBFD time slot with in-channel adjacent subband leakage ratio and blocking being the most important ones |
| R4-2309179 | ZTE Corporation | Proposal 1: to consider the guard periods at the beginning of SBFD UL symbols/slots; Proposal 2a: to have different conducted declaration for normal DL symbols and SBFD DL symbols; Proposal 2b: to have different EIRP/TRP declaration for normal DL symbols and SBFD DL symbols; Proposal 3a; to reuse the existing RE power control dynamic range requirement for SBFD BS; Proposal 3b: to specify the total dynamic range requirement for SBFD based on the DL transmission bandwidth configuration in both normal DL symbols and SBFD DL symbols; Proposal 4: to further discuss the transition requirement for DL transmission and SBFD slot switching scenario and SBFD slots switching scenario and its exact requirement. Proposal 5: to reuse the existing freq error, EVM and TAE requirement for SBFD BS and further discuss the joint measurement for normal DL symbols/slots and SBFD DL symbols/slots and necessity of relaxation of measurement period. Proposal 6: to reuse existing OBW requirement for DL sub-band of SBFD BS; Proposal 7a: propose to consider the DL ACLR requirement outside of downlink carrier instead of sub-band carrier; Proposal 7b: for in-channel emission, to consider this emission in the gNB Refsens degradation via self interference and inter-sector interference as shown in Figure 2.1.4-1 implicitly. Proposal 8a: propose to consider the DL OBUE requirement outside of downlink carrier instead of sub-band carrier; Proposal 8b: for in-channel emission/OBUE, to consider this emission in the gNB Refsens degradation via self interference and inter-sector interference as shown in Figure 2.1.4-1 implicitly. Proposal 9a: for the general spurious emission requirement, to reuse the existing general spurious emission requirement of TS 38.104. Proposal 9b: for the protection of BS receiver of own or other different BS, the existing requirement in TS38.104 is not applicable. Proposal 9c: for additional spurious emission requirement, the existing additional spurious emission requirement in section 6.6.5.2.3 of TS 38.104 for SBFD BS. Proposal 9d: for co-location requirement with other BS, FFS for co-location requirement especially considering the ongoing coexistence study work in RAN4. Proposal 10a: further discuss whether to apply Tx requirement for SBFD slots and its coupling interfering power if necessary; Proposal 10b: if Tx requirement is considered for SBFD slots, then to add the Refsens degradation as one more performance metric in addition to transmitter OBUE/ACLR/spurious emission requirements. Proposal 11a: for the conducted refsens conformance testing, the antenna should be installed during the conformance testing otherwise there are no self interference injected by the OTA. Proposal 1b: for Refsens of SBFD symbols/slots, to define two set of requirement: 1) self interference; 2) self interference+ insector co-channel interference; Proposal 11c: further discuss the degradation levels for Set 1 requirement and Set 2 requirement; Proposal 11d: to further discuss the FRC for Refsens of SBFD UL symbols/slots in the WID phase. Proposal 12a: for dynamic range requirement of SBFD UL symbols/slots, to consider both DL transmission as interfer in addition to wide-band AGWN interfer and UL configuration as wanted signal; Proposal 12b: to further discuss the IoT level for dynamic range requirement of SBFD uplink symbols/slots; Proposal 12c: to further discuss the FRC for wanted signal and interference signal for dynmic range requirement of SBFD UL symbols/slots in the WID phase. Proposal 13a: for ACS, IBB, NBB requirement, propose to consider this requirement out of uplink carrier bandwidth instead of uplink sub-band. Proposal 13b: to consider two sets of requirement for ACS/IBB/NBB requirement: 1) with the self interference only; 2) with self interference and in-sector co-channel interference; Proposal 14a: for general OOBB requirement, the existing interfering power level for SBFD UL symbols/slot are applicable, wanted signal of general OOBB requirement should consider the self interference and in-sector co-channel interference in addition to OOBB interfer power; Proposal 14b: FFS for co-location OOBB requirement similar as co-location transmitter spurious emission and Tx intermodulation requirement; Proposal 15: for receiver spurious emission requirement in the SBFD uplink symbols/slot, the requirement is not supposed be applicable due to the installation of antenna in the conformance testing framework. Proposal 16: for receiver intermodulation requirement in the SBFD uplink symbols/slot, further consider IMD between CW/NBB/general intermodulation interfering signal intermodulate with SBFD DL transmission as shown in Figure 2.2.6-1. Proposal 17a: for ICS requirement of SBFD UL symbols/slots, to consider both DL transmission as interfer in addition to image interfer and UL configuration as wanted signal; Proposal 17b: to further discuss the IoT level for ICS requirement of SBFD uplink symbols/slots; Proposal 17c: to further discuss the FRC for wanted signal and interference signal for ICS requirement of SBFD UL symbols/slots in the WID phase. Proposal 18: to OTA transmitter conformance testing, propose not to add the any uplink signal transmitted within the chamber to aovid the interference. |
| R4-2309327 | Samsung | Observation 1: 1dB sensitivity degradation shall match with the scenario in which there is SBFD BS self-interference of DL transmission but without other interference sources like inter-sector interference and inter-site interference. Observation 2: To take inter-site interference and inter-sector interference into account in OTA (and also potentially conducted) reference sensitivity tests, we observed the following issues: (1) Both inter-sector and inter-site interference greatly depends on detailed deployment scenario, which is difficult to be modeled in both OTA and conducted sensitivity test. (2) In current conformance testing, macro gNB with one sectorized antenna can be regarded as DUT for testing, rather than a complex system including the co-sited three sectors. Proposal 1: For OTA sensitivity requirement within SBFD time slot for SBFD-capable gNB: - 1dB sensitivity degradation is introduced for self-interference; - OTA sensitivity shall be defined by considering self-interference, but without inter-sector and inter-site interference. Observation 3: It is difficult for RAN4 to agree on a single RF architecture to derive the potential new requirements for (1) in-channel adjacent subband leakage ratio, (2) in-channel adjacent subband blocking and (3) in-channel adjacent subband selectivity. Observation 4: With OTA sensitivity requirements if introduced for SBFD-capable gNB with the simultaneous TX in the SBFD time slot, in-channel adjacent subband leakage ratio, in-channel adjacent subband blocking and in-channel adjacent subband selectivity requirements can be guaranteed implicitly. Proposal 2: For SBFD-capable gNB, RAN4 shall not introduce new in-channel adjacent subband requirements, including: - in-channel adjacent subband leakage ratio, - in-channel adjacent subband blocking and - in-channel adjacent subband selectivity. Observation 5: Among three options in RAN1 for SBFD antenna configuration, the concerned switching from normal operation to SBFD operation (or backwards) for a certain array (or sub-array) only happens in SBFD antenna configuration Option-1. Proposal 3: For SBFD-capable gNB, RAN4 further discuss and identify the applicable scenario in which ON/OFF or potentially introduced transient period requirement for the concerned switching from normal operation to SBFD operation (or backwards) for a certain array (or sub-array). Observation 6: To consider the impact of Tx intermodulation for co-location scenario for SBFD-capable gNB, the conclusion for coupling loss between co-located gNB shall be followed instead. Proposal 4: RAN4 need further study the necessity of introducing additional Tx intermodulation requirement for SBFD-capable gNB in the SBFD slot. |

## Open issues summary

Before f2f meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 3-1: BS RF requirement impact

Sub-topic description:

[Moderator] In last meeting, BS RF requirement impact have been discussed, with the following agreements captured in WF [R4-2305918]:

## Issue 4-1-1: OTA sensitivity within SBFD time slot WF: - OTA sensitivity can be derived based on the following equation as a starting point: PREFSENSdBm=-174dBm+10×log10(BW)+NF+IM+SNR+[ 1dB desens target] - The followings should be discussed further - The exact value for [1dB desens target] - The declaration of maximum TRP for the requirement of OTA sensitivity within SBFD time slot - If OTA sensitivity should be defined considering all of the scenarios including self-interference, inter-site interference and inter-sector interference. ## Issue 4-1-2: In-channel adjacent subband leakage ratio, In-channel adjacent subband Blocking and adjacent subband selectivity within SBFD time slot WF: - FFS if these requirements need to be defined. - The following aspects are mentioned during the discussion in this meeting, - The potential request from the performance insurance when considering inter-site and inter-sector BS interference. - The possibility of adding inter-site and/or inter-sector BS interference into the OTA sensitivity test - The assumption of BS-BS isolation - The adopted interference suppression technology - Whether or not these requirements can be implicitly guaranteed by OTA sensitivity requirement ## Issue 4-1-4: Transition ON-OFF power and transition period WF: - Further discuss the necessity of transition requirement for the slot changes between SBFD slots and SBFD/non-SBFD slots. - The following aspect is mentioned during the discussion in this meeting, - The switching of parts of an array - Turn on the receiver side compared with DL transmission only - etc ## Issue 4-1-5: Tx intermodulation requirement and co-location out-of-band blocking WF: - Further discuss Tx intermodulation requirement for co-location scenario. - The following aspects are mentioned in this meeting, - Large Tx IM signal may block SBFD BS, no requirement or a reasonable requirement may be needed. - If new requirement is needed, the REFSENS DESENS should take self-interference DESENS into account. - If larger coupling loss between co-located gNBs should be considered for this requirement. - TX IM may be needed to ensure that TX emissions are maintained in the presence of an interferer (even if the interferer would de-sensitize the SBFD receiver, or during non-SBFD DL slots). ## Issue 4-1-6: Other new requirements (if needed) WF: - No agreements on this issue. - The following aspects are discussed in this meeting, - Self-interference impact on Rx IMD performance - The noise floor rise for the dynamic range requirement due to the in-channel interference from other gNBs. - Not precluded other new requirements.

#### Issue 3-1-1: Conducted/OTA sensitivity within SBFD time slot

- Proposals/Observations from Samsung:

  - Observation 1: 1dB sensitivity degradation shall match with the scenario in which there is SBFD BS self-interference of DL transmission but without other interference sources like inter-sector interference and inter-site interference.

  - Observation 2: To take inter-site interference and inter-sector interference into account in OTA (and also potentially conducted) reference sensitivity tests, we observed the following issues:

    - Both inter-sector and inter-site interference greatly depends on detailed deployment scenario, which is difficult to be modeled in both OTA and conducted sensitivity test.

    - In current conformance testing, macro gNB with one sectorized antenna can be regarded as DUT for testing, rather than a complex system including the co-sited three sectors.

  - Proposal 1: For OTA sensitivity requirement within SBFD time slot for SBFD-capable gNB:

    - 1dB sensitivity degradation is introduced for self-interference;

    - OTA sensitivity shall be defined by considering self-interference, but without inter-sector and inter-site interference.

- Proposals/Observations from CATT:

  - Proposal 1: OTA sensitivity should be defined only for the self-interference scenario.

  - Proposal 2: The exact DESENS value can be discuss and decided in WI phase. The REFSENS formula can be written as following,

PREFSENSdBm=-174dBm+10×log10(BW)+NF+IM+SNR+ xdB desens

x value can be different for each BS class pending on the discussion.

  - Proposal 3: For OTA sensitivity test, following current FR2 FDD BS core requirement and test approach could be ok. No TRP or EIRP requirement is defined for the OTA sensitivity requirement.

- Proposals/Observations from Ericsson:

  - Observation 1: The requirement on OTA sensitivity does not ensure that the BS has no more than XdB degradation compared to its real sensitivity (if it’s real sensitivity is better than the minimum requirement).

  - Observation 2: Despite observation 1, the requirement can be based on minimum sensitivity for minimum requirements purposes.

  - Observation 3: The desensitization in SBFD slots depends on all sources of interference. If 1dB degradation is allowed for self-interference only then the degradation during operation will be greater than 1dB.

  - Proposal 1: RAN4 discuss a reasonable degradation for self-interference only, considering an reasonable headroom for degradations from other interference sources.

  - Proposal 2: Define the OTA sensitivity requirement over the EIRP accuracy directions range and OTA sensitivity RoAoA. Conformance testing should be in some EIRP testing directions and some OTA sensitivity RoAoA test directions.

- Proposals/Observations from Nokia:

  - Observation 1:  If all interferences are considered, then 1dB is accepted. If only self-interference is considered, then 0.5dB is accepted.

  - Proposal 1: Use up to 0.5dB for self-interference only and 1dB for considering desensitization from all interferences

#### Issue 3-1-2: In-channel adjacent subband leakage ratio within SBFD time slot

- Proposals/Observations from Samsung:

  - Observation 1: It is difficult for RAN4 to agree on a single RF architecture to derive the potential new requirements for (1) in-channel adjacent subband leakage ratio, (2) in-channel adjacent subband blocking and (3) in-channel adjacent subband selectivity.

  - Observation 2: With OTA sensitivity requirements if introduced for SBFD-capable gNB with the simultaneous TX in the SBFD time slot, in-channel adjacent subband leakage ratio, in-channel adjacent subband blocking and in-channel adjacent subband selectivity requirements can be guaranteed implicitly.

  - Proposal 1: For SBFD-capable gNB, RAN4 shall not introduce new in-channel adjacent subband requirements, including:

    - in-channel adjacent subband leakage ratio.

- Proposals/Observations from CATT:

  - Proposal 1 (CATT): In-channel adjacent subband leakage ratio and in-channel adjacent subband selectivity requirements are defined for SBFD capable BS.

- Proposals/Observations from Ericsson:

  - Observation 1: TX sub-band leakage from other sectors or gNBs is not captured in the OTA sensitivity requirement

  - Observation 2: Means to mitigate TX sub-band leakage degradation to the RX sub-band cannot be applied for other nodes.

  - Proposal 1: Consider ACLR (scaled according to bandwidths) as a baseline for a TX sub-band leakage requirement.

- Proposals/Observations from Nokia:

Observation 1:  Additional requirements are needed to be defined as they cannot be implicitly guaranteed by OTA sensitivity

Proposal 1: There is a need to define additional requirements within SBFD time slot with in-channel adjacent subband leakage ratio and blocking being the most important ones

- Proposals/Observations from ZTE:

Proposal 1: for in-channel emission, to consider this emission in the gNB Refsens degradation via self interference and inter-sector interference as shown in Figure 2.1.4-1 implicitly.

#### Issue 3-1-3: In-channel adjacent subband Blocking and selectivity within SBFD time slot

- Proposals/Observations from Samsung:

  - Proposal 1 (Samsung): For SBFD-capable gNB, RAN4 shall not introduce new in-channel adjacent subband requirements, including:

    - in-channel adjacent subband blocking and

    - in-channel adjacent subband selectivity.

- Proposals/Observations from CATT:

  - Proposal 1: In-channel adjacent subband selectivity requirements is defined for SBFD capable BS. No separate sub-band blocking requirement is needed.

- Proposals/Observations from Ericsson:

  - Observation 1: The OTA sensitivity requirement does not capture RX degradation due to TX sub-bands from other nodes.

  - Observation 2: Measures to mitigate the impact of TX sub-band leakage from the own transmitter may not work for power coming from other nodes.

  - Proposal 1: Consider ACS as a baseline for an RX sub-band selectivity requirement.

- Proposals/Observations from Nokia:

  - Observation 1:  Additional requirements are needed to be defined as they cannot be implicitly guaranteed by OTA sensitivity

  - Proposal 1: There is a need to define additional requirements within SBFD time slot with in-channel adjacent subband leakage ratio and blocking being the most important ones

- Proposals/Observations from ZTE:

  - Proposal 17a: for ICS requirement of SBFD UL symbols/slots, to consider both DL transmission as interfer in addition to image interfer and UL configuration as wanted signal;

  - Proposal 17b: to further discuss the IoT level for ICS requirement of SBFD uplink symbols/slots;

  - Proposal 17c: to further discuss the FRC for wanted signal and interference signal for ICS requirement of SBFD UL symbols/slots in the WID phase.

#### Issue 3-1-4: Transition ON-OFF power and transition period

- Proposals/Observations from Samsung:

  - Observation 1: Among three options in RAN1 for SBFD antenna configuration, the concerned switching from normal operation to SBFD operation (or backwards) for a certain array (or sub-array) only happens in SBFD antenna configuration Option-1.

  - Proposal 1: For SBFD-capable gNB, RAN4 further discuss and identify the applicable scenario in which ON/OFF or potentially introduced transient period requirement for the concerned switching from normal operation to SBFD operation (or backwards) for a certain array (or sub-array).

- Proposals/Observations from CATT:

  - Observation 1: SBFD and non-SBFD slots are configured statically in the real deployment.

  - Proposal 1: No transmit ON/OFF power requirement is defined for SBFD.

- Proposals/Observations from Ericsson:

  - Observation 1: There can still be switching between UL and DL slots for SBFD

  - Observation 2: When switching between TX and RX for the UL sub-band, a guard period and requirements on transient times are needed to prevent interference to/from other nodes, same as when all PRB are switched between TX/RX.

  - Observation 3: When parts of the array are switched between TX and RX, or from transmitting in sub-bands to the whole carrier, a transient time may be needed until the TX (or RX) is stable in the new configuration.

  - Proposal 1: For TX switching, consider transient times for RX sub-band switching and also for switching parts of the array.

- Proposals/Observations from ZTE:

  - Proposal 1: to further discuss the transition requirement for DL transmission and SBFD slot switching scenario and SBFD slots switching scenario and its exact requirement.

#### Issue 3-1-5: Tx intermodulation requirement

- Proposals/Observations from Samsung:

  - Observation 1: To consider the impact of Tx intermodulation for co-location scenario for SBFD-capable gNB, the conclusion for coupling loss between co-located gNB shall be followed instead.

  - Proposal 1: RAN4 need further study the necessity of introducing additional Tx intermodulation requirement for SBFD-capable gNB in the SBFD slot.

- Proposals/Observations from CATT:

  - Observation 1: The legacy Tx intermodulation requirement may be able to guarantee the SBFD Tx IMD performance.

- Proposals/Observations from Ericsson:

  - Observation 1: The transmitter should be built to be robust to reverse intermodulation

  - Observation 2: Meeting RX intermodulation in regular DL slots may be sufficient.

  - Observation 3: In case TX intermodulation is tested during SBFD slots, this relates to exceptional conditions and there should be no requirement on the RX sub-band in such conditions.

- Proposals/Observations from ZTE:

  - Proposal 10a: further discuss whether to apply Tx requirement for SBFD slots and its coupling interfering power if necessary;

  - Proposal 10b: if Tx requirement is considered for SBFD slots, then to add the Refsens degradation as one more performance metric in addition to transmitter OBUE/ACLR/spurious emission requirements.

#### Issue 3-1-6: Co-location and co-existence

- Proposals/Observations from Ericsson:

  - Observation 1: Co-location requirements are standardized so that operators can deploy BS in different bands with confidence that proprietary solutions are not needed.

  - Observation 2: Co-existence between bands requirements are standardized to ensure that deployments in different bands at different sites will not interfere to one another.

  - Observation 3: Co-location and co-existence between bands requirements are optional and support is declared.

  - Observation 4: Regardless of SBFD or non-SBFD, co-location requirements between bands do not assume synchronization between TDD patterns and are met by means of a band-wide analogue filter.

  - Observation 5: It does not make sense to standardize relaxed co-location/co-existence requirements, since meeting them would not mean that proprietary solutions would not be needed.

  - Proposal 1: Maintain co-location and co-existence between bands requirements that are optional for SBFD BS.

- Proposals/Observations from ZTE:

  - Proposal 9b: for the protection of BS receiver of own or other different BS, the existing requirement in TS38.104 is not applicable.

  - Proposal 9d: for co-location requirement with other BS, FFS for co-location requirement especially considering the ongoing coexistence study work in RAN4.

#### Issue 3-1-7: Dynamic range

- Proposals/Observations from Ericsson:

  - Proposal 1: Adjust UL dynamic range requirements as needed.

- Proposals/Observations from ZTE:

  - Proposal 12a: for dynamic range requirement of SBFD UL symbols/slots, to consider both DL transmission as interfer in addition to wide-band AGWN interfer and UL configuration as wanted signal;

  - Proposal 12b: to further discuss the IoT level for dynamic range requirement of SBFD uplink symbols/slots;

  - Proposal 12c: to further discuss the FRC for wanted signal and interference signal for dynmic range requirement of SBFD UL symbols/slots in the WID phase.

#### Issue 3-1-8: Other requirements survey for Conducted TX

- Proposals/Observations for from ZTE:

| Requirement | Proposals/Observations |
| --- | --- |
| BS station output power | Proposal 2a: to have different conducted declaration for normal DL symbols and SBFD DL symbols; Proposal 2b: to have different EIRP/TRP declaration for normal DL symbols and SBFD DL symbols; |
| Output power dynamics | Proposal 3a; to reuse the existing RE power control dynamic range requirement for SBFD BS; Proposal 3b: to specify the total dynamic range requirement for SBFD based on the DL transmission bandwidth configuration in both normal DL symbols/slots and SBFD DL symbols/slots; |
| Transmitted signal quality | Proposal 5: to reuse the existing freq error, EVM and TAE requirement for SBFD BS and further discuss the joint measurement for normal DL symbols/slots and SBFD DL symbols/slots and necessity of relaxation of measurement period. |
| OBW | Proposal 6: to reuse existing OBW requirement for DL sub-band of SBFD BS; |
| ACLR | Proposal 7a: propose to consider the DL ACLR requirement outside of downlink carrier instead of sub-band carrier; |
| Unwanted emission mask | Proposal 8a: propose to consider the DL OBUE requirement outside of downlink carrier instead of sub-band carrier; Proposal 8b: for in-channel emission/OBUE, to consider this emission in the gNB Refsens degradation via self interference and inter-sector interference as shown in Figure 2.1.4-1 implicitly. |
| Transmitter spurious emission | General spurious emission requirement: Proposal 9a: for the general spurious emission requirement, to reuse the existing general spurious emission requirement of TS 38.104. Additional spurious emissions requirements Proposal 9d: for co-location requirement with other BS, FFS for co-location requirement especially considering the ongoing coexistence study work in RAN4. |

#### Issue 3-1-9: Other requirements survey for Conducted RX

- Proposals/Observations for from ZTE:

| Requirement | Proposals/Observations |
| --- | --- |
| Reference sensitivity level | Proposal 11a: for the conducted refsens conformance testing, the antenna should be installed during the conformance testing otherwise there are no self interference injected by the OTA. Proposal 1b: for Refsens of SBFD symbols/slots, to define two set of requirement: 1) self interference; 2) self interference+ insector co-channel interference; Proposal 11c: further discuss the degradation levels for Set 1 requirement and Set 2 requirement; Proposal 11d: to further discuss the FRC for Refsens of SBFD UL symbols/slots in the WID phase. |
| ACS, In-band blocking, Narrow band blocking requirement | Proposal 13a: for ACS, IBB, NBB requirement, propose to consider this requirement out of uplink carrier bandwidth instead of uplink sub-band. Proposal 13b: to consider two sets of requirement for ACS/IBB/NBB requirement: 1) with the self interference only; 2) with self interference and in-sector co-channel interference; |
| Out-of-band blocking | Proposal 14a: for general OOBB requirement, the existing interfering power level for SBFD UL symbols/slot are applicable, wanted signal of general OOBB requirement should consider the self interference and in-sector co-channel interference in addition to OOBB interfer power; Proposal 14b: FFS for co-location OOBB requirement similar as co-location spurious emission and Tx intermodulation requirement; |
| Receiver spurious emissions | Proposal 15: for receiver spurious emission requirement in the SBFD uplink symbols/slot, the requirement is not supposed be applicable due to the installation of antenna in the conformance testing framework. |
| Receiver intermodulation | Proposal 16: for receiver intermodulation requirement in the SBFD uplink symbols/slot, further consider IMD between CW/NBB/general intermodulation interfering signal intermodulate with SBFD DL transmission as shown in Figure 2.2.6-1. |

#### Issue 3-1-10: Other requirements survey for Radiated TX

- Proposals/Observations for from ZTE:

| Requirement | Proposals/Observations |
| --- | --- |
| Radiated transmit power | Proposal 18: to OTA transmitter conformance testing, propose not to add the any uplink signal transmitted within the chamber to aovid the interference. |
| OTA base station output power | the existing TRP accuracy requirement and the rated TRP output power limits in section 9.3 of TS 38.104 are still applicable, while different radiated TRP output powers for normal and SBFD slot. |
| OTA output power dynamics | the existing RE power control dynamic range and total power dynamic range requirement in section 6.3 of TS 38.104 are still applicable |
| OTA transmitted signal quality | The conclusion for conducted transmit transmitted signal quality could be also reused here. |
| OTA unwanted emissions (OBW, ACLR, OBUE, spurious emission) | The conclusion for conducted unwanted emission requirement could be also reused here. |
| OTA Tx intermodulation requirement | The conclusion for conducted Tx intermodulation requirements could be also reused here. |

#### Issue 3-1-11: Other requirements survey for Radiated RX

- Proposals/Observations for from ZTE:

| Requirement | Proposals/Observations |
| --- | --- |
| General | For FDD/SBFD operation the requirements shall be met with the transmitter unit(s) ON. |
| OTA dynamic range | The conclusion for conducted dynamic Reference sensitivity could be also reused here. |
| OTA In-band selectivity and blocking | The conclusion for conducted in-band selectivity and blocking could be also reused here. |
| OTA Out-of-band blocking | The conclusion for conducted OOBB requirements could be also reused here. |
| OTA Receiver spurious emissions | The conclusion for conducted receiver spurious emission requirement could be also reused here. It should be not applicable for SBFD slots/symbols. |
| OTA Receiver intermodulation | The conclusion for conducted Rx IMD requirements could be also reused here. |

### Sub-topic 3-2: Further reply LS on R1-2304182

#### Issue 3-2-1: Further reply LS on R1-2304182

[Moderator] It is common understanding that the questions in R1-2304182 have been addressed in RAN4’s reply LS R4-2306004, while only one issue proposed in R4-2308628 is to further clarify on the feasibility and applicable scenarios of improved noise figure by using subband filtering. Here the reply related to subband filtering is provided for information purpose:

Furthermore, RAN4 made the following agreement on noise figure model (i.e., noise figure over average total input power, captured in RAN4 LS R4-2302885): - For typical FR1 WA BS implementation, companies have different views on sub-band filter, and based on companies’ proposal on the implementation with sub-band filter, the blocking performance can be improved around 10 dB. The following values of A and B can apply to the noise figure model if sub-band filter adopted. - The values of A, B, C and D: - A = -35dBm - B = -17dBm - C = 5dB - D = 14dB - Note: RAN4 has not reach consensus on the implementation feasibility of sub-band filter. RAN4 will further evaluate and update to RAN1 if needed.

- Proposals:

  - Proposal 1 (Nokia): It is proposed to send LS to RAN1 and clarify on the feasibility and applicable scenarios of improved noise figure.

<Observations from Nokia copied from R4-2308628> Even though RAN4 agreed, sub-band filtering before or after the LNA is not seen feasible as it will have the effect of: - High insertion loss before the LNA will increase the receiver noise figure and negate any system gains of SBFD. - High insertion loss placed after the LNA does not reduce LNA linearity requirements. Improved linearity LNAs are needed that add to cost and power consumption, which leads to thermal management issues. Other challenges include: - Significant insertion loss → NF increase that can hardly be accommodated in the 1 dB desensitization budget. - Considerable transition band and temperature dependency of the passband position → larger guard band between the sub-bands needed and less BW is usable for UL - Group delay distortion close to the cut-off frequencies - Incompatibility with a typical multi-carrier gNB design - Increased complexity as switches are needed for by-pass in UL slots for full BW - Additional space needed in RX chain that is not available in typical gNB design - Overall additional power consumption which leads to thermal management issues - Frequency drift over temperature that will impact filter insertion loss and rejection performance, hence impacts the RX lineup performance

- Moderator: This issue can be discussed with Issue 1-1-3 for the same technical discussion.

# Topic #4: Regulatory survey

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2307064 | Spark NZ Ltd | Withdrawn |
| R4-2307181 | Spark NZ Ltd, Ericsson, Nokia, Nokia Shanghai Bell | TP provided. Proposal 1: it is proposed to approve the above mentioned TP towards Chapter 13 of TR 38.858 |
| R4-2307760 | Huawei, HiSilicon | TP on the conclusion subsection provided. |
| R4-2308631 | Nokia, Nokia Shanghai Bell | Observation 1: Details on US SAS coordinates TDD configurations can be captured in TR 38.858. |
| R4-2309459 | CHTTL, SGS Wireless | Proposal 1: Agree to include the text proposal of the regulation aspect for NR duplex evolution in Taiwan in section 5. |
| R4-2309662 | CableLabs, Charter Communications | Proposal 1: it is proposed to approve the above-mentioned TP towards Chapter 13 of TR 38.585. |

The moderator can suggest a limited number of papers which could be presented.

## Open issues summary

Before f2f meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 5-1: Text Proposal on regional regulation

Sub-topic description:

[Moderator] As one of the objectives in WID as below, RAN4 is tasked to summarize the regulatory aspects for deploying the duplex enhancement in TDD unpaired spectrum:

The detailed objectives are as follows: - ... - Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4).

Open issues and candidate options before e-meeting:

#### Issue 4-1-1: Text Proposal on Region 1

- Proposals

  - Option 1 (Ericsson, Spark, Nokia, Nokia Shanghai Bell, R4-2307181):

13.1	Region 1

13.1.1	Europe

CEPT made coexistence studies with adjacent services assuming a certain DL/UL ratio for IMT TDD bands, e.g. 3.4-3.8GHz band in Europe. The evolution of NR duplex operation would bring changes to the frame structures of legacy TDD operation and consequently may affect the outcomes of the coexistence studies and, consequently, the regulated license conditions.

To address the cross-border issue and facilitate coordination, ECC recommended the usage of two frame structures in the 3.4-3.8GHz frequency band (ECC Recommendation(20)03).

However, enabling operation with various TDD patterns and removing the need of synchronized networks, CEPT has specified additional baselines for unsynchronized or semi-unsynchronized networks. Nevertheless, those baselines are more stringent, making the BS design more challenging, impacting final cost and possibly product’s volume and weight. As an example, for the 3.4-3.8GHz band, inside the band, ECC specified below and above the block edge a restricted baseline of -34dBm/5 MHz EIRP for non AAS BS or -43dBm/MHz TRP for AAS BS (ECC Decision(11)06).operators, the situation may be reported to the competent authority for resolution.

#### Issue 4-1-2: Text Proposal on Region 2

Based on WF approved in R4-2305919: 3.2.2 Issue 2-3: In USA, the spectrum access system (SAS) coordinates TDD configurations among operators in band 48/n48 under FCC guidance, which is missing. • Recommended WF: CableLabs and Charter Communications will prepare a TP for USA market. Companies are encouraged to bring contributions to next meeting on this.

- Proposals for US SAS coordination for Band 48/n48

  - Observation 1 (Nokia, R4-2308613): Details on US SAS coordinates TDD configurations can be captured in TR 38.858.

  - Option 1 (CableLabs and Charter, R4-2309662):

In the US, TDD network operators operating in proximal geographic areas in adjacent bands are encouraged and sometimes required to synchronize their networks and coordinate their TDD configurations to avoid mutual interference. The US Federal Communications Commission (FCC) requires 3450 – 3550 MHz service (AMBIT band) licensees to negotiate with 3550 – 3700 MHz (CBRS band) licensees to enable TDD synchronization across these services. Notice that the term TDD synchronization refers to aligning TDD uplink and downlink. FCC recognizes the potential for harmful interference from a high-power AMBIT band downlink transmission to a CBRS band uplink. Licensees in the 3700 – 3980 MHz band (C-Band) are encouraged to explore synchronization of TDD operations to minimize interference between adjacent band services.

The shared band 48/n48 (3550 – 3700 MHz), also known as the CBRS band, requires spectrum sharing among three tiers of users controlled by one or multiple spectrum access systems (SASs). Coexistence, including TDD synchronization, among cellular users within the band is supported by OnGo Alliance coexistence requirements set forth in OnGo-TS-2001.

The Innovation, Science and Economic Development Canada (ISED) is reallocating portions of the 3500 to 4200 MHz band as TDD bands for cellular use. The ISED is considering TDD synchronization as a means of facilitating sharing and co-existence with adjacent band services.

Currently there are no specific regulatory requirements for SBFD operation in North America. Some SBFD operations result in the same interference scenarios as found in unsynchronized TDD systems. The potential coexistence risk introduced by SBFD may break the regulatory considerations on TDD synchronization.

- Proposals for general Region 2:

  - Option 1 (Ericsson, Spark, Nokia, Nokia Shanghai Bell, R4-2307181):

No TDD pattern has been mandated in US, nor in Canada, but operators are encouraged to coordinate their network deployment and make sure they don’t interfere with each other.

Unsynchronized operation is allowed, more stringent regulation parameters have not been specified for such case but, again, operators would have to work their differences to avoid any claim to FCC/ISED.

When SBFD will be introduced, Regulators might issue some consultations to understand whether there would be some oversight of the feature.

Open issues and candidate options before e-meeting:

#### Issue 4-1-3: Text Proposal on Region 3

- Proposals

  - Option 1 (CHTTL, SGS Wireless, R4-2309459):

13.3.x	Taiwan

No TDD pattern has been mandated in Taiwan. The mutual interference between the released TDD spectrum segments shall be solved by using technical means or retaining guard bands based on the coordination between the operators. If such agreement cannot be reached among operators, the situation may be reported to the competent authority for resolution.

  - Option 2 (Ericsson, Spark, Nokia, Nokia Shanghai Bell, R4-2307181):

13.3.1	China

In China, spectrum is allocated with clearly stating it for TDD or FDD operation. MIIT has specified a TDD pattern that should be used by the operators when operating adjacent TDD networks, assuming then synchronization between those operators.

There is no SBFD regulatory requirements in China until now. MIIT mainly cares interference between different operators. Necessary interference coordination mechanism and solutions may be proposed by MIIT to avoid interference before any SBFD deployment.

13.3.2	Japan

No TDD pattern has been mandated in Japan, but operators are required to coordinate their network deployment to avoid interference. Operators are allowed to use unsynchronized operation as far as there is no interference with the adjacent network(s), e.g. for indoor usage.

13.3.3		New Zealand

In NZ a TDD pattern has been mandated and in addition the networks must be time synchronised. Operator deployments that do not conform to the synchronisation requirement must not interfere with deployments that are conforming with the described synchronisation requirements , and therefore cannot claim protection from interference. Therefore it will be extremely difficult to introduce SBFD.

13.3.4		Australia

In Australia there are frame structure requirements which only apply when interference occurs between licences and there is no agreement between licensees on how to resolve it. Operators can use different frame structures if there are no issues.

13.3.5		India

In India no frame structure is mandated. In case operators have incompatible frame structures resulting in interference then the onus of mitigating interference falls amongst the operators.

Open issues and candidate options before e-meeting:

#### Issue 4-1-4: Text Proposal on Summary

- Proposals

  - Option 1 (Huawei, R4-2307760):

In present the spectrum is issued with clearly defined duplex mode, i.e. FDD or TDD, and probably SDL or SUL. In 3GPP specifications, it assumes the TDD base stations deployed in the same geographical area and use the same or adjacent operating band, are synchronized. No additional co-existence requirements are covered for unsynchronized operation. To avoid cross-link interference situations, regulatory conditions at national level define the common TDD frame structures for multiple operators’ operation in the same band or administrations ask MNOs to agree on common frame structure for Macro cellular deployments.

For single operator’s TDD network, there may be no limitation on the frame structure and it is up to operator’s choice.

It already possible today to use different TDD frame structure for isolated deployment, e.g. isolated indoor factory, as long as obligation to avoid interference is guaranteed.

SBFD operation would allow simultaneous transmission and reception in different sub-bands within the same carrier. New regulatory requirements might be needed to allow SBFD operation for multiple operators’ deployment.

  - Option 2 (Ericsson, Spark, Nokia, Nokia Shanghai Bell, R4-2307181):

Regulators always pay high attention to any new technology that might create interference to incumbent services operating in or adjacent to the considered spectrum, specifying new conditions to prevent any such interference.

When allocating spectrum to IMT TDD operation, Regulators made coexistence studies with incumbent services assuming a certain TDD pattern. Based on the conclusions of those studies, Regulators have then specified the corresponding specific parameters to enable such deployment.

In most of the countries, operators are expected to synchronize their adjacent TDD networks. Some Regulators have even recommended specific TDD frame structure usage to facilitate this, addressing then cross-border issues between countries (e.g. in Europe).

To enable unsynchronized TDD deployments without creating interference in the adjacent network(s), some Regulators have specified more stringent parameters (e.g. CEPT specified below and above the block edge a restricted baseline of -34dBm/5 MHz EIRP for non AAS BS or -43dBm/MHz TRP for AAS BS), increasing BS design’s complexity significantly.

Regulators might revise existing regulatory rules to allow SBFD operations and/or mandate more stringent requirements.

Nevertheless, when deployed in environments which guarantee and prevent any interference in the adjacent spectrum (like isolated indoor deployment), no specific condition nor recommendation have been specified by the Regulators, allowing any TDD deployment in such environments as long as no interference disturbs adjacent services. For such type of deployments, existing regulation rules should not be impacting when operating SBFD.