3GPP TSG-RAN WG4 Meeting # 107	R4-2309742

# Incheon, South Korea, 22 – 26 May 2023

Title:	LS on EVM for shorter transient period capability in FR1

Response to:

Release:	Rel-16

Work Item:	NR_RF_FR1-Core

Source:	RAN WG 4

To:	RAN WG 5

Cc:

Contact person:

Name:	Laurent Noel

E-mail Address:	laurent.noel@skyworksinc.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# Overall description

RAN4 has made the following agreement on transient assumptions for verification of a UE that signals a shorter transient period capability within FR1.

For testing of EVM with shorter transient period capability, RAN4 assumes that:

- Transients are verified using the TS 38.101-1 Figure 6.3.3.7-3 ON to ON time mask,

- Transients are not triggered by TPC commands, they are triggered by an RB allocation change.

In addition, RAN4 has also agreed to add the following new uplink Reference Measurement Channel (RMC) with 5ms periodicity in TS 38.101-1 to verify the transient at SCS 15kHz according to the Figure 6.3.3.7-3 time-mask.

Additional TDD configuration Parameter; Value. SCS 15 kHz (µ0). TDD Slot Configuration pattern (Note 1); 2DS2U. Special Slot Configuration (Note 2); 10D+2G+2U. referenceSubcarrierSpacing; 15 kHz. UL-DL configuration; dl-UL-TransmissionPeriodicity; 5 ms. nrofDownlinkSlots; 2. nrofDownlinkSymbols; 10. nrofUplinkSlot; 2. nrofUplinkSymbols; 2. NOTE 1: D denotes a slot with all DL symbols; S denotes a slot with a mix of DL, UL and guard symbols; U denotes a slot with all UL symbols. The field is for information. NOTE 2: D, G, U denote DL, guard and UL symbols, respectively. The field is for information..

# Actions

To RAN5:

ACTION: RAN4 respectfully asks RAN5 to take the above into consideration.

# Date of Next TSG-RAN WG4 Meetings

TSG-RAN WG4 Meeting #108	21 Aug – 25 Aug 2023	Toulouse, FR

TSG-RAN WG4 Meeting #108-bis	9 Oct – 13 Oct 2023		Xiamen, CN