3GPP TSG-RAN WG4 Meeting #108	R4-2311159

Toulouse, France, 21st August – 25th August 2023

Agenda item:	8.6.4.1

Source: 		NTT DOCOMO, INC.

Title: 	Discussion on PUSCH demodulation requirements for FR2 UL256QAM.

Document for:	Discussion

- Introduction

The WI for FR2 UL256QAM has been implemented for several RAN4 Main and RRM meetings. RAN4 should specify the demodulation part in accordance with WID [1] and it’s time to kick-off demodulation discussion for the WI. In this contribution. We provide the view on SCS/CBW combination for PUSCH demodulation requirements for FR2 UL256QAM.

- Discussion

Waveform

To test for the improved throughput in FR2 UL256QAM, we adopt the appropriate waveform. NR supports both of CP-OFDM and DFT-s-OFDM for UL. CP-OFDM is used for high throughput and DFT-s-OFDM is used for coverage enhancement, respectively. Thus, we propose CP-OFDM for FR2 UL256QAM demodulation test.

Proposal 1: Adopt CP-OFDM for PUSCH test on FR2 UL256QAM demodulation.

SCS/CBW combination

According to proposal 1 as above noted, for SCS/CBW combinations, FR2 UL256QAM demodulation is not different from other modulation. The other modulations (up to 64QAM) testing covers the SCS/CBW combinations:

- 60 kHz SCS: 50MHz and 100MHz

- 120 kHz SCS: 50MHz, 100MHz and 200MHz

The same SCS/CBW combinations can be reused.

Proposal2: To adopt 60kHz SCS:50MHz and 100MHz.

Proposal 3: To adopt 120kHz SCS:50MHz, 100MHz and 200MHz.

- Conclusion

In this contribution, we provide views on PUSCH demodulation requirements for FR2 UL256QAM. The following an observation and a proposal are obtained.

Proposal 1: Adopt CP-OFDM for PUSCH test on FR2 UL256QAM demodulation.

Proposal 2: To adopt 60kHz SCS:50MHz and 100MHz.

Proposal 3: To adopt 120kHz SCS:50MHz, 100MHz and 200MHz.

References

- RP-222909, revised WID: NR RF requirements enhancement for frequency range 2 (FR2), Phase 3, Nokia, Xiaomi, RAN#98-e, December 2022