3GPP TSG RAN WG4 Meeting #108		R4-2311245

Toulouse, France, August 21st – 25th 2023

Agenda item:	7.33.3

Source:	Apple

Title:	A-MPR results for NR US 900 and 3MHz CBW

WI/SI:	NR_900MHz_US-Core

Release:	Rel-18

Document for:	Approval

# 1	Introduction

LTE US 900 MHz band has been introduced with [1] in RAN#107. The RF requirements are not specified for NR spec and are left open for further discussion. This contribution further explores the need of A-MPR for NR DFT-s-OFDM and CP-OFDM and makes observations accordingly.

# 2  Discussion

###### 2.1 Simulation assumptions

The following assumptions and requirements for the simulations are used:

- Power Class 3

- NR Waveform

- Calibration: 1dB MPR: DFT-s-OFDM QPSK 20MHz, 100RB0

- Carrier Leakage: 28dBc

- Image: 28dBc

- CIM3: 60dBc

- CIM5: 70dBc

- CBW: 3MHz

- 3GPP SEM & ACLR

- Protection of frequency range 869 to 894 MHz with -30dBm/MHz.

###### 2.2 Simulation results

For wideband flat out-of-band emissions the requirement -30dBm/MHz is slightly sharper than the general spurious emission limit of -36dBm/100kHz while for narrow spurious emissions the latter is more limiting. To check whether A-MPR would be required for compliance with band 5 protection, simulations have been conducted for 3MHz CBW at the lower edge of the US 900MHz band. Below the results for QPSK are provided.

Table 1: Power Back-off results for US 900MHz band with protecting band 5

In case of QPSK the available MPR allowance seems sufficient for DFT-s-OFDM and CP-OFDM. The simulations do not show any limitation from the band 5 protection requirement. Other modulations such as Pi/2 BPSK were also checked and MPR seems to be sufficient.

Observation: For the NR waveforms DFT-s-OFDM and CP-OFDM there is no need for additional power back-off in case the emission limit for the frequency range 869 to 894 MHz is set to -30dBm/MHz.

Proposal: Set emission limit for the frequency range 869 to 894 MHz to -30dBm/MHz.

# 4	Conclusions

This contribution provides A-MPR simulation results. The following observation is made:

Observation: For the NR waveforms DFT-s-OFDM and CP-OFDM there is no need for additional power back-off in case the emission limit for the frequency range 869 to 894 MHz is set to -30dBm/MHz.

Proposal: Set emission limit for the frequency range 869 to 894 MHz to -30dBm/MHz.

# 4	References

- R4-2310488, “CR to TS36.101: the introduction of 900 MHz LTE new band“, ZTE Corporation, 3GPP TSG-RAN WG4 Meeting #107

- R4-2307132, “Simulation results for LTE US 900”, Nokia, Nokia Shanghai Bell, 3GPP TSG-RAN WG4 Meeting #107