3GPP RAN TSG-RAN4 Meeting 108	R4-xxxxxx

Toulouse, France, August 21 – August 25, 2023

Agenda item:       7.30.1

Source:                 Apple Inc.

Title:                     MSD analysis for CA_n5-n105, CA_n28-n105, and CA_n26-n28

WI/SI:                   NR_700800900_combo_enh

Release:                 Rel-18

Document for:      Discussion

# Introduction and scope

The study item (SI) about the CA_n5-n105, CA_n28-n105 and CA_n26-n28 band combinations was recently concluded as can be seen in the big CR R4-2307546 “study on enhancement for sub-1GHz NR band combinations”, which was agreed in the previous RAN4 meeting ( #107 in Incheon, May 22-26, 2023 ) [1]. During the recent RAN plenary meeting (#110 in Taipei, June 12-14, 2023), the enhancement for 700/800/900MHz band combinations WID [2] was revised to include the 3 newly approved inter-band CA combinations CA_n5-n105, CA_n28-n105, and CA_n26-n28 as highlighted in bold in the Table 1-1 below.

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n5A-n8A | CA_n5A-n8A | n5 | 5, 10 | 0 |
|  |  | n8 | 5, 10 |  |
| CA_n5A-n28A | CA_n5A-n28A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
| CA_n8A-n20A-n28A | CA_n8A-n20A CA_n8A-n28A CA_n20A-n28A | n8 | 5, 10, 15, 20 | 0 |
|  |  | n20 | 5, 10, 15, 20 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
| CA_n5A-n105A | CA_n5A-n105A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |
| CA_n28A-n105A | - | n28 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |
| CA_n26A-n28A | CA_n26A-n28A | n26 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |

Table 1-1: Updated table for the 700/800/900 WID with the new bands highlighted in bold

In this discussion paper, we will present the RF requirements for these 3 new band combinations in an attempt to address the following key objectives highlighted in the WID:

- Define the Tib/Rib for the band combinations

- Define the REFSENS degradation  for some specific band combinations if it’s needed

- Define other necessary requirements for some specific band combinations if they are identified to be

needed.

During the study phase for these band combinations, three RF front-end architectures were commonly considered [3], [4], [5], [6]:

- Two-antenna architecture with a quadplexer on the main path and a duplexer on the diversity path

- Two-antenna architecture with a triplexers on main and diversity paths

- Three-antenna architecture with duplexers on the two main paths and the diversity path

For UE applications, two-antenna architecture with triplexers on main and diversity paths seems to provide the best trade-off for antenna size, performance for, and ease of integration. This architecture will be used as the baseline architecture for our analysis.

# Discussion

  - CA_n5-n105 Combination

The aggregated spectrum allocated for the CA_n5-n105 is shown below in Figure 2.1-1.

Figure 2.1-1: Aggregated spectrum allocation for the CA_n5-n105 combination

As stated at the introduction section above, we will use as a baseline the topology for 2-antenna architecture illustrated in Figure 2,1-2 below:

-	1) n5UL+n5DL+n105DL triplexer is routed through the  main antenna

-	2) n105UL+n105DL+n5DL triplexer is routed through the  diversity antenna

Figure 2.1-2: CA_n5-n105 RF front-end architecture based on 2 antennas and 2 triplexers

    - CA_n5-n105 MSD Analysis

For single-UL, as can be seen in the table below, there is no significant cross-band interference due to ACLR because of the wide frequency separations between the UL and DL bands (166MHz and 172MHz). With 20MHz of typical UL CBW, there could be cross-band interference due to ACLR8, but this contribution will be negligeable.

For dual-UL there is no significant IMD levels that fall in either of the RX bands. So the cross-band IMD contribution on the MSD will be ignored.

There is still, however, a potential MSD contribution from the TX noise leaking to the RX bands. This will be the focus of the analysis below, where we will consider both the n5 TX noise leaking into the n105 RX and the n05 TX noise leaking into the n5 RX.

| Interference Source | 1 Uplink (n5UL or n105UL) | 2 Uplinks (n5UL+n105UL) |
| --- | --- | --- |
| Harmonic/harmonic mixing | No issue | No issue |
| Intermodulation | No issue | No significant issue |
| Cross-band isolation | - No significant ACLR issue - Potential TX noise leakage issue |  |

Table 2.1.1-1: CA_n5-n105 MSD cross-band interference  sources

For the MSD analysis, we will use the following worst-case assumptions for the RF front-end performance parameters:

- n5 Tx filter rejection at n105 RX (612MHz-652MHz): 30dB

- n105 Tx filter rejection at n5 RX (869MHz-894MHz): 30dB

- Cross-band TX to RX isolation: 40dB

- Front-end insertion loss: 4.5dB

- Antenna coupling: 10dB

- RxBW: 4.5MHz

Given the assumptions above, the MSD analysis is summarized in the table below

| MSD Source | Parameter | Unit | n5 Tx n105 RX |  | n105 Tx n5 RX |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | n105 main path | n105 div. path | n5 main path | n5 div. path |
| PA Noise + ACLR | TX noise at PA output (PC3) | dBm/RxBW | -53.5 |  | -52.8 |  |
|  | Total PA noise @ ANT | dBm/RxBW | -88.0 | -98.0 | -87.3 | -97.3 |
| Thermal Noise | 3GPP REFSENS | dBm | -97.2 | -97.2 | -98 | -98 |
|  | SNR | dB | -1 | -1 | -1 | -1 |
|  | Thermal noise | dBm | -96.2 | -96.2 | -97 | -97 |
| Composite Noise | Composite noise | dBm | -87.4 | -94.0 | -86.9 | -94.2 |
|  | Composite noise after MRC | dBm | -94.9 |  | -95.0 |  |
| MSD |  | dB | 1.3 |  | 2.0 |  |

Table 2.1.1-2: CA_n5-n105 MSD analysis

    - CA_n5-n105 ∆TIB,c and ∆RIB,c Values

It seems that two-antenna solutions with triplexers will not be too challenging to implement for CA_n5-n105 as the frequency separations between UL and DL are more than 100MHz. We can use the same ∆TIB,c as the one used for CA_n5-n71 and for ∆RIB,c the numbers for CA_n5-n12 can be reused, as captured in the two tables below.

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n5-n105 | n5 | 0.5 |
|  | n105 | 0.5 |

Table 2.1.2-1: ∆TIB,c  for CA_n5-n105

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n5-n105 | n5 | 0.3 |
|  | n105 | 0.3 |

Table 2.1.2-2: ∆RIB,c  for CA_n5-n105

  - CA_n28-n105 Combinations

The aggregated spectrum allocated for the CA_n5-n105 is shown below in Figure 2.2-1 and the architecture for dual-antenna with triplexers is captured in Figure 2.2-2.

Figure 2.2-1: Aggregated spectrum allocation for the CA_n5-n105 combination

Figure 2.2-2: CA_n28-n105 RF front-end architecture based on 2 antennas and 2 triplexers

    - CA_n28-n105 MSD Analysis

For dual-UL, earlier study done during the previous stage showed that there is an IMD3/IMD5 issue that may create a severe receiver desensitization issue either in n105 DL, n28 DL, or both [7]. As a result, in the WF in RAN4 meeting #106e, it was recommended that for cross-band isolation interference, only single-UL should be considered [8]. We will therefore do the MSD analysis only for a single-UL, either n28 UL or n105 UL, using the CBWs shown in the table below.

| UL Band | UL Low | UL High | DL Band | DL Low | DL High | UL-DL Gap | Max UL BW | Source of interference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n28 | 703MHz | 748MHz | n105 | 612MHz | 652MHz | 51MHz | 30MHz | ACLR2 (IMD5) |
| n105 | 663MHz | 703MHz | n28 | 758MHz | 803MHz | 55MHz | 20MHz | ACLR3 (IMD7) |

Figure 2.2.1-1: CA_n28-n105 spectrums and CBWs

| Interference Source | 1 Uplink (n28UL or n105UL) | 2 Uplinks (n28UL+n105UL) |
| --- | --- | --- |
| Harmonic/harmonic mixing | No issue | No issue |
| Intermodulation | No issue | IMD3/IMD5 issue |
| Cross-band isolation | - ACLR2 (for n28UL n105 DL case) - ACLR3 issues (for n105 UL n28 DL case) - TX noise leakage issue |  |

Table 2.2.1-2: CA_n28-n105 MSD cross-band interference sources

For the MSD analysis, we will use the following worst-case assumptions for the RF front-end performance parameters:

- n28 Tx filter rejection at n105 RX (612MHz-652MHz): 30dB

- n105 Tx filter rejection at n28 RX (758MHz-803MHz): 30dB

- Cross-band TX to RX isolation: 40dB

- ACLR2 sideband level: 30dBc

- ACLR3 sideband level: 30dBc

- Front-end insertion loss: 4.5dB

- Antenna coupling: 10dB

- RxBW: 4.5MHz

- Output Power at antenna: 23dBm

Given the assumptions above, the MSD analysis is summarized in the table below:

| MSD Source | Parameter | Unit | n28 Tx n105 RX |  | n105 Tx n28 RX |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | n105 main path | n105 div. path | n5 main path | n5 div. path |
| PA noise + ACLR | TX noise leakage at PA output due to ACLR2/3 (PC3) | dBm/RxBW | -45.7 |  | -51.1 |  |
|  | Total PA noise @ ANT | dBm/RXBW | -79.7 | -89.7 | -80.6 | -90.6 |
| IIP2 effects | Typical Rx IIP2 | dBm | 54 | 54 | 54 | 54 |
|  | TX IM2 noise level @ ANT | dBm | -98 | -108 | -98 | -108 |
| Thermal Noise | 3GPP REFSENS | dBm | -97.2 | -97.2 | -98.5 | -98.5 |
|  | SNR | dB | -1 | -1 | -1 | -1 |
|  | Thermal noise | dBm | -96.2 | -96.2 | -97.5 | -97.5 |
| Composite | Composite noise | dBm | -79.5 | -88.7 | -80.4 | -89.7 |
|  | Composite noise after MRC | dBm | -89.2 |  | -90.1 |  |
| MSD |  | dB | 7.0 |  | 7.3 |  |

Table 2.1.1-3: CA_n28-n105 MSD analysis

    - CA_n28-n105 ∆TIB,c and ∆RIB,c Values

The spectrum of CA_n28-105 with two-antenna architecture is similar to the CA_n71-n85 spectrum so we could use the same ∆TIB,c and ∆RIB,c values as proposed in [11].

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n5-n105 | n5 | 0.5 |
|  | n105 | 0.5 |

Table 2.1.2-1: ∆TIB,c  for CA_n5-n105

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n5-n105 | n5 | 0.3 |
|  | n105 | 0.3 |

Table 2.1.2-2: ∆RIB,c  for CA_n5-n105

  - CA_n26-n28 Combination

During the study phase for the CA_n26-n28 band combination, there was no agreement on whether to consider only the lower 30MHz section of n28 or the entire band, as captured in the following WF[9]:

Case 1: lower 30MHz of n28 (UL: 703~733MHz, DL: 758~788MHz) is supported.

Case 2: entire n28 frequency range (UL: 703~748MHz, DL: 758~803MHz) is supported.

As a consequence of this, we will do the MSD analysis and access the ∆TIB,c and ∆RIB,c for both cases. The corresponding aggregated spectrums are shown in Figures 2.3-1 and 2.3-2 below:

Figure 2.3-1: Aggregated spectrum allocation for the CA_n26-n28 combination (full n28 band)

Figure 2.3-2: Aggregated spectrum allocation for the CA_n26-n28 combination (lower 30MHz of n28 band)

The two-antenna architecture is captured in the following figure.

Figure 2.3-3: CA_n26-n28 RF front-end architecture based on 2 antennas and 2 triplexers

    - CA_n26-n28 MSD Analysis

As can be seen from the MSD contributor table below, for n28 UL n26 DL, there is more than 110MHz of spectrum gap. Therefore, the cross-band interference for that case will be minimum and so the MSD for n26 DL will be small. However, for the n26 UL  n28 DL, there is only 11MHz of separation when the n28 full 45MHz band is considered and 26MMHz when the lower 30MHz of n28 (n28A) is assumed. Due to that small frequency separation, there is a strong cross-band interference that will affect the n28 DL. Next, we will examine the n28 DL MSD for both cases.

| Interference Source | 1 Uplink (n26UL or n28UL) | 2 Uplinks (n26UL+n28UL) |
| --- | --- | --- |
| Harmonic/harmonic mixing | No issue | No issue |
| Intermodulation | No issue | No significant issue |
| Cross-band isolation | Option # 1: Entire n28 band - Significant ACLR1 and cross band noise issue for n26ULn28DL case - Potential TX noise leakage issue Option # 2: Lower 30MHz of 28 band - Significant ACLR2 and cross band noise issue for n26ULn28DL case - Potential TX noise leakage issue |  |

Table 2.3.1-1: CA_n26-n28 MSD cross-band interference sources

For the n28 DL MSD analysis, we will use the following worst-case assumptions for the RF front-end performance parameters:

- n26 Tx filter rejection at n28 RX (758MHz-803MHz): 30dB

- Cross-band TX to RX isolation: 40dB

- ACLR2 sideband level: 30dBc

- ACLR3 sideband level: 30dBc

- Front-end insertion loss: 4.5dB

- Antenna coupling: 10dB

- RxBW: 4.5MHz

- Output Power at antenna: 23dBm

Given the assumptions above, the MSD analysis is summarized in the table below

| MSD Source | Parameter | Unit | n26 Tx n28 RX Case 1 (Full 45MHz BW) |  | n26 Tx n28 RX Case 2 (Lower 30MHz BW only) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | n28 main path | n28 div. path | n28 main path | n28 div. path |
| PA noise + ACLR | TX noise leakage at PA output due to ACLR2 (PC3) | dBm/RxBW | -14 |  | -39.5 |  |
|  | Total PA noise @ ANT | dBm/RXBW | -48.5 | -58.5 | -79 | -89 |
| IIP2 effects | Typical Rx IIP2 | dBm | 54 | 54 | 54 | 54 |
|  | TX IM2 noise level @ ANT | dBm | -98 | -108 | -98 | -108 |
| Thermal Noise | 3GPP REFSENS | dBm | -98.5 | -98.5 | -98.5 | -98.5 |
|  | SNR | dB | -1 | -1 | -1 | -1 |
|  | Thermal noise | dBm | -97.5 | -97.5 | -97.5 | -97.5 |
| Composite | Composite noise | dBm | -48.5 | -58.5 | -78.9 | -88.3 |
|  | Composite noise after MRC | dBm | -65.7 |  | -88.7 |  |
| MSD |  | dB | >31.0 |  | 8.7 |  |

Table 2.3.1-2: CA_n26-n28 MSD analysis

    - CA_n26-n28 ∆TIB,c and ∆RIB,c Values

For CA_n26-n28, we can reuse the values already derived for the CA_n5-n28 as proposed in [10].

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n26-n28 | n26 | 0.7 |
|  | n28 | 0.7 |

Table 2.3.2-1: ∆TIB,c  for CA_n26-n28

| Inter-band CA Configuration | NR Band | RIB,c [dB] |
| --- | --- | --- |
| CA_n26-n28 | n26 | 0.2 |
|  | n28 | 0.2 |

Table 2.3.2-1: ∆RIB,c  for CA_n26-n28

# Conclusion

In this discussion paper, we shared the results of our analysis of the cross-band MSD for CA_n5-n105, CA_n28-n105, and CA_n26-n28 using the two-antenna architectures. Below is a summary of our proposals:

- Proposal 1: CA_n5-n105 MSD due to cross-band isolation.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n105 | n5 | 693 | 20 | 15 | 20 (RBstart=86) | 871.5 | 5 | 1.3 | >ACLR2 |
| n5 | n105 | 834 | 20 | 15 | 20 (RBstart=0) | 649.5 | 5 | 2.0 | >ACLR2 |

Table 3-1: MSD due to cross band isolation for the CA_n5-n105 combination

- Proposal 2: ∆RIB,c and ∆TIB,c for CA_n5-n105

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n5-n105 | n5 | 0.5 |
|  | n105 | 0.5 |
| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| CA_n5-n105 | n5 | 0.3 |
|  | n105 | 0.3 |

Table 3-2: ∆RIB,c and ∆TIB,c for CA_n5-n105 due to cross band isolation

- Proposal 3: CA_n28-n105 MSD due to cross-band isolation.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n105 | n28 | 693 | 20 | 15 | 20 (RBstart=86) | 760.5 | 5 | 7.3 | >ACLR2 |
| n28 | n105 | 718 | 30 | 15 | 25 (RBstart=0) | 649.5 | 5 | 7.0 | ACLR2 |

Table 3-3: MSD due to cross band isolation for the CA_n28-n105 combination

- Proposal 4: ∆RIB,c and ∆TIB,c for CA_n28-n105

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n28-n105 | n28 | 1.0 |
|  | n105 | 1.0 |
| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| CA_n28-n105 | n28 | 0.8 |
|  | n105 | 0.8 |

Table 3-4: ∆RIB,c and ∆TIB,c for CA_n28-n105 due to cross band isolation

- Proposal 5: CA_n26-n28 MSD due to cross-band isolation.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n26 | n28 | 824 | 20 | 15 | 25 (RBstart=0) | 785.5 | 5 | 8.7 | ACLR2 |
| n26 | n28 | 824 | 20 | 15 | 25 (RBstart=0) | 800.5 | 5 | >31 | ACLR |

Table 3-5: MSD due to cross band isolation for the CA_n26-n28 combination

- Proposal 6: ∆RIB,c and ∆TIB,c for CA_n26-n28

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n26-n28 | n26 | 0.7 |
|  | n28 | 0.7 |
| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| CA_n26-n28 | n26 | 0.2 |
|  | n28 | 0.2 |

Table 3-6: ∆RIB,c and ∆TIB,c for CA_n26-n28 due to cross band isolation

# 5	References

- R4-2307546 Big CR for TR 38.872 to introduce outcomes of CA_n5-n105 CA_n28-n105 CA_n26-n28 in sub-1GHz SI

- RP-231450 Revised WID on enhancement for 700/800/900MHz band combinations

- R4-2306470 WF on RF architecture and critical issues for CA_n28-n105, Apple

- R4-2306469 WF on RF architecture and critical issues for CA_n5-n105, Xiaomi

- R4-2306471 WF on RF architecture and critical issues for CA_n26-n28, Huawei

- R4-2307546 Big CR for TR 38.872 to introduce outcomes of CA_n5-n105 CA_n28-n105 CA_n26-n28 in sub-1GHz SI, Huawei, Hisilicon

- R4-2309093 Draft CR for TR 38.872: TP for CA_n28A-n105A UE RF architectures, Apple

- R4-2306470 WF on RF architecture and critical issues for CA_n28-n105, Apple, RAN4#106b-e

- R4-2306471 WF on RF architecture and critical issues for CA_n26-n28, Huawei, HiSilicon, RAN4#106b-e

- R4-2300657 2UL cross band MSDs for CA_n5-n28, Skyworks, RAN4#106

- R4-2300414 TP to TR 38.717-02-01: Addition of CA_n71-n85, Nokia, T-Mobile, RAN4#106