3GPP TSG-RAN4 Meeting #108 		R4-2311860

Toulouse, FR, 21th Aug. – 25th Aug., 2023

Title:	Discussion on RRM requirements for NTN deployment in above 10 GHz bands

Source:	Xiaomi

Agenda Item:	8.26.5.1

Document for:	Discussion

- Introduction

In RAN4#107 meeting, RAN4 had some discussion on RRM requirements for Rel-18 NTN deployment in above 10GHz bands, and the related WF was approved in [1]. In this contribution, we discuss the potential RRM requirements for Rel-18 NTN deployment in above 10GHz bands and provide our proposals.

- Discussion

2.1	UE UL timing requirement

### Issue 2-1: UE UL Timing Accuracy Requirements for UL SCSs of 60kHz and 120kHz Agreement: - The assumption of the maximum total positioning error due to UE location and Satellite position estimation error shall be tightened as below: - For UL SCS of 60kHz, [X] meters. - For UL SCS of 120kHz, [Y] meters - The above is applicable only when SSB SCS is equal to or higher than 120kHz - FFS on whether applicable to UE in mobile platform. - FFS on whether and how to connect the tightened UE positioning error to the advanced GNSS capability or UE type. - FFS whether to use different requirements for different types of devices defined in the RF session and/or different satellite types - FFS on different UE UL Timing Accuracy Requirements for different physical signals/channels, e.g. relaxed requirements for PRACH for certain PRACH formats, compared to other signal/channels.

In Rel-18 NTN, the NTN deployment in above 10GHz bands, e.g. Ka band is supported for VSAT UE, and according to the agreements in RF session in [2], the numerology of NTN Ka band is agreed as follows:

- For R18 NTN Ka band, 60 kHz and 120 kHz Sub-Carrier Spacing (SCS) are considered. 120 kHz and 240 kHz SCS are considered for SSB. - The possibility of operating with 30 kHz SCS for both data and SSB to support smaller channel bandwidth than 50 MHz is not precluded in RAN4 at this moment, pending on feasibility study conclusion.

From RRM perspective, RAN4 need to evaluate the UE initial transmission error requirement for Ka band. In order to guarantee the performance of UE UL transmission and gNB reception, the error budget for UE UL transmission should be smaller than half CP, which can be expressed as:

(CP -Tdelay_spread)/2 - (Te_NTN + ∆TA_command + ∆TA_adjust) > 0

Where,

    - Tdelay_spread is delay spread which does not need to be considered for Ka band (TR38.811)

    - Te_NTN is UE UL initial timing error, Te_NTN = Te + Te_GNSS + Te_SAT

      - Te is the legacy timing error

      - Te_GNSS is the GNSS accuracy, Te_GNSS = 2* (GNSS positioning accuracy/c);

      - Te_SAT is the serving-satellite position estimation error, Te_SAT = 2* (serving-satellite positioning estimation accuracy /c)

    - ∆TA_command is TA command quantization error (82μ) Ts units (TS38.213)

    - ∆TA_adjust is UE TA adjustment error (TS 38.133, 7.3.2)

According to delay spread assumption for Ka band in [3], the flat fading can be assumed for Ka band signals, thus, Tdelay_spread does not need to be considered for Ka band.

7.3.5.2.2 Assessment of conditions for NR operation in Non-Terrestrial networks Delay spread in satellite propagation channels Signal echoes are associated to the presence of indirect rays that reach the receiver antenna and carry a significant energy with respect to the energy of the direct ray. ITU-R recommendation [34] defines for the 2 GHz band three parameter sets of wideband models, including LOS and NLOS cases, applicable for an elevation range from 15 to 55° and for urban, suburban and rural environments. The delay spread of these three parameter sets ranges between 180 ns to 250 ns, whereas the 250 ns are stated to cover 90% of the cases. For higher elevations than 55°, we assume that the delay spread of the satellite channel will be in the same range or even lower due to the traveling distances of the echoes arriving at a receiver. Few papers are available on delay spread measurements in Ka-Band. Reference [35] is stating the coherence bandwidth to be 30 MHz at 40 GHz with omnidirectional antennas. According to [36], the coherence bandwidth (Δ f)c of a channel with maximum delay spread Tm is (Δ f)c ≈ 1 / (5T) For the stated coherence bandwidth in [35], this results in a maximum delay spread of Tm = 25 ns for omni-directional antennas. For directional antennas, echoes with significant delay are normally filtered out by the antenna radiation pattern, so flat fading can be assumed for Ka-band signals.

Based on above assumption and analysis, the maximum error for GNSS position and satellite position can be calculated in following table.

| SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | CP length | ∆TA_command | ∆TA_adjust | Te | Remaining budget for (Te_GNSS+Te_SAT) | GNSS position error + satellite position error |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 120 | 60 | 36*64*Tc | 2*64*Tc | 2*64*Tc | 3.5*64*Tc | 10.5*64*Tc | 56m |
|  | 120 | 18*64*Tc | 1*64*Tc | 0.5*64*Tc | 3.5*64*Tc | 4*64*Tc | 21m |
| 240 | 60 | 36*64*Tc | 2*64*Tc | 2*64*Tc | 3*64*Tc | 11*64*Tc | 58m |
|  | 120 | 18*64*Tc | 1*64*Tc | 0.5*64*Tc | 3*64*Tc | 4*64*Tc | 21m |

Observation 1: The maximum position error used for GNSS position and serving-satellite position is around 56m for 60 KHz UL SCS case and around 20m for 120 KHz UL SCS case.

Proposal 1: For 60 KHz UL SCS case, the GNSS position error is assumed as 15m and the serving-satellite position error is assumed as 30m, respectively.

Proposal 2: FFS to define the GNSS position error and the serving-satellite position error for 120 KHz UL SCS case.

2.2	Measurement and mobility requirement

### Issue 2-2: Measurement and Mobility Requirements in NR-NTN above 10 GHz bands - No agreement.

According to RF session agreement, the radiated requirement will be defined for NTN bands above 10GHz. From RRM perspective, the UE Rx beamforming related requirement should be considered in RRM requirement. In legacy requirement, the UE Rx beam sweeping factor is assumed as 8, which will cause a long measurement delay requirement. Considering NGSO scenario, the satellite service time would be not long, the UE may not be able to complete the mobility measurement on target cell. Thus, it is necessary to reduce the measurement delay requirement for Ka bands, e.g. reduce the Rx beam sweeping factor.

Proposal 3: RAN4 to consider the enhanced measurement requirement for Ka bands, e.g. reduce the UE Rx beam sweeping factor.

- Conclusion

In this contribution, we discussed the RRM impact for NTN bans above 10GHz and proposals are provided as follows:

Observation 1: The maximum position error used for GNSS position and serving-satellite position is around 56m for 60 KHz UL SCS case and around 20m for 120 KHz UL SCS case.

Proposal 1: For 60 KHz UL SCS case, the GNSS position error is assumed as 15m and the serving-satellite position error is assumed as 30m, respectively.

Proposal 2: FFS to define the GNSS position error and the serving-satellite position error for 120 KHz UL SCS case.

Proposal 3: RAN4 to consider the enhanced measurement requirement for Ka bands, e.g. reduce the UE Rx beam sweeping factor.

- Reference

[1]	R4-2310168	WF on NR NTN enhancement RRM requirements, Qualcomm

[2]	R4-2305926	LS on the system parameters for NTN above 10 GHz, CATT

[3]	TR38.811