3GPP TSG-RAN WG4 Meeting #108	R4-2312010

Toulouse, France, August 21 – August 25, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DraftCR | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on Pre-MG and ConMGs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MG_enh2-Core |  |  |  |  |  | Date: |  |  | 2023-7-31 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The WI for MG further enh. was discussed in RAN4, but no related measurement requirement was captured in spec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduce the measurement requirement for concurrent pre-configured gaps into spec. as follows. - Applicability - The condition where the concurrent pre-configured gaps requirements applied, e.g., UE capability, network configuration, CA/DC scenarios - Association - The association for each frequency layer to Pre-MG - UE Capability - Max # of supported gaps for both with and without concurrent Pre-MG capability - Introduce the dynamic collision rule for concurrent Pre-MG. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements for concurrent pre-configured gaps will be missing and their performance cannot be guaranteed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.x, 9.1.x.1, 9.1.x.2, 9.1.x.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------NEXT CHANGE---------------------------

### 9.1.x	Concurrent measurement gaps with pre-configured measurement gap

#### 9.1.x.1	Introduction

When UE supports [ConMGs with Pre-MG] capability, network can provide multiple measurement gap patterms with at least one pre-configured measurement gap (Pre-MG) pattern configured by RRC message(s) as specified in TS 38.331 [2].

Requirements in this section apply when the UE is in SA operation mode and requires measurement gaps to identify and measure intra-frequency cells and/or inter-frequency cells and/or inter-RAT E-UTRAN cells.

#### 9.1.x.2	Requirements

If the UE supports [ConMGs with Pre-MG] but does not support independent measurement gap patterns for different frequency ranges as specified in Table 5.1-1 in [18, 19, 20], then the requirements in the following clauses apply provided that the network configures the UE with not more than two per-UE Pre-MG patterns for monitoring all the frequency layers.

If the UE supports both [ConMGs with Pre-MG] and independent measurement gap patterns for different frequency ranges as specified in Table 5.1-1 in [18, 19, 20], then the requirements defined for [ConMGs with Pre-MG] apply provided that the network configures the UE with the measurement gap patterns’combinations specified in Table 9.1.x.1 for monitoring of all frequency layers. The supported measurement gap combination configurations for UE supporting both [ConMGs with Pre-MG] and independent measurement gap patterns for different frequency ranges are specified in Table 9.1.x-1.

Table 9.1.x-1: The number of Gap Combination Configurations by UE supporting both [ConMGs with Pre-MG] and independent measurement gap patterns

| Gap Combination Configuration Id | The number of simultaneous configured measurement gap patterns |  |  |
| --- | --- | --- | --- |
|  | Per-FR1 measurement gap | Per-FR2 measurement gap |  | Per-UE measurement gap |  |
| 0 | 2 | 1 |  | 0 |  |
| 1 | 1 | 2 |  | 0 |  |
| 2 | 0 | 0 |  | 2 |  |
| 3Note 1 | 1 | 0 |  | 1 |  |
| 4Note 1 | 0 | 1 |  | 1 |  |
| 5Note 1 | 1 | 1 |  | 1 |  |
| 6 | 2 | 0 | 0 |  |  |
| 7 | 0 | 2 | 0 |  |  |
| Note 1: Gap Combination Configuration Id #3, #4, #5 are only applicable when the per-UE measurement gap is associated to measure PRS for any RSTD, PRS-RSRP, and UE Rx-Tx time difference measurement defined in TS 38.215 [4]. [Note 2]: For UE capable of [Concurrent Pre-MG], up to 2 measurement gap patterns can be configured as Pre-MG in one FR, regardless of whether they are per-UE or per-FR configuration and the configuration in the other FR. Otherwise, the gaps can only be configured as Type-1/2 MG. |  |  |  |

When UE supports [Concurrent Pre-MG], the gap association for a frequency layer is configured by the network via [gapAssociation] in [ToBeMeasureConfig]. In this case the gap association rules in clause 9.1.8.2 shall also apply.

When UE supports [Concurrent Pre-MG], each measurement gap pattern supported by the UE is listed in Table 9.1.2-1 based on the applicability specified in table 9.1.2-3.

When autonomous mechanism [1] is used for activation/deactivation of Pre-MG pattern, the UE shall autonomously determine the Pre-MG status only based on the measurement objects associated with the concerned Pre-MG. The related Pre-MG autonomous activation/deactivation mechanism is specified in clause 9.1.7.3.1.

When network-controlled mechanism [1] is used for activation/deactivation, the UE determines the Pre-MG status only based on the bits corresponding to the concerned Pre-MG in preConfGapStatus. The related Pre-MG network-controlled activation/deactivation mechanism is speficified in clause 9.1.7.3.2.

The requirements in clause 9.1.2 are applicable for the UE capable of [Concurrent Pre-MG] and configured with multiple concurrent measurement gap patterns within each activated Pre-MG pattern.

#### 9.1.x.3	Collision for concurrent pre-configured measurement gaps

Dynamic collision between two measurement gap occasions may occur as specified in this clause for the following scenarios.

- two concurrent pre-configured measurement gap (Pre-MG) patterns if both Pre-MGs are activated,

- one Pre-MG pattern and one measurement gap pattern if the Pre-MG is activated,

and these two measurement gaps are

-	two per-UE measurement gaps, or

-	two per-FR measurement gaps in the same FR, or

-	one per-UE measurement gap and one per-FR measurement gap

and if the gap collision condition specified in clause 9.1.8.3 is met then the gap collison rule applies.

When any Pre-MG is deactivated then gap collison does not occur, and no gap collision rule applies.

The priority for a Pre-MG is configured by networks via [gapPriority] in [GapConfig]. The configured priority of a Pre-MG shall remain unchanged until it is reconfigured by RRC signalling. The requirements for [concurrent measurement gaps with Pre-MG] apply provided that the two measurement gaps colliding with each other are configured with different priorities.

When one measurement gap pattern and one Pre-MG pattern are configured by network, the measurement gap occasion is considered as colliding with the Pre-MG activation procedure if the distance in time between the measurement gap occasion and the Pre-MG activation procedure is equal to or smaller than 4ms.

The distance between the measurement gap occasion and the Pre-MG activation procedure is defined as the time difference between the ending point of the gap occasion and the starting point of the Pre-MG activation procedure, or the starting point of the gap occasion and the ending point of the Pre-MG activation procedure. The starting and ending point of the Pre-MG activation procedure are defined in clause [8.x.y].

In case of the collision, the UE shall perform measurements in the occasion of the measurement gap during the Pre-MG activation procedure and the earliest occasion of the Pre-MG gap to be activated after the Pre-MG activation procedure shall be dropped if the distance in time between the occasion of Pre-MG to be activated and the measurement gap occasion is equal to or smaller than 5ms. The UE is expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI in the corresponding NR serving cells in the slots of the configured Pre-MG that are dropped according to the requirements in clause 9.1.8.4.

--------------------END OF CHANGES--------------------------