3GPP TSG RAN WG4 Meeting #108	R4-2312129

Toulouse, France, Aug 21st – Aug 25th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 3460 | rev |  | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on Conditional handover including target MCG and candidate SCG for CPC for FR1-FR1 in NR-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2023-08-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In Rel-18 FeMob enh, the mechanism of Conditional handover including target MCG and candidate SCG for CPC in NR-DC is supported, the corresponding RRM requirement needs to be specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce and specify the requirements on Conditional handover including target MCG and candidate SCG for CPC for FR1-FR1 NR-DC |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No requirement on Conditional handover including target MCG and candidate SCG for CPC for FR1-FR1 NR-DC |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# <Start of change 1>

6.1.7.1	Conditional handover including target MCG and candidate SCG for CPC in NR-DC

The purpose of NR Conditional handover including target MCG and candidate SCG for CPC is to change the NR PCell to another NR cell and change the PSCell along with PCell handover. The requirements in this clause are applicable to:

-	Conditional handover including target MCG in FR1 and candidate SCG for CPC in FR1 in NR-DC

-	Conditional handover including target MCG in FR1 and candidate SCG for CPC in FR2 in NR-DC

6.1.7.1.1	Conditional handover including target MCG in FR1 and candidate SCG for CPC in FR1 in NR-DC

The requirements in this clause are applicable to both intra-frequency and inter-frequency conditional handover including target MCG in FR1 and candidate SCG for CPC in FR1 in NR-DC.

Procedure delays for all procedures that can command a conditional handover including target MCG and candidate SCG for CPC are specified in TS 38.331 [2].

When the UE receives a RRC message implying conditional handover including target MCG and candidate SCG for CPC,

-	The UE shall be ready to start the transmission of the new uplink PRACH channel of the target PCell within DCHOwithCPC_PCell ms from the end of the last TTI containing the RRC command, and

-	The UE shall be capable of transmitting PRACH preamble towards the target PSCell no later than DCHOwithCPC_PSCell ms from the end of the last TTI containing the RRC command.

Where:

-	DCHOwithCPC_PCell is the PCell conditional handover delay stated in clause 6.1.7.1.1.1.

-	DCHOwithCPC_PSCell is the PSCell conditional change delay stated in clause 6.1.7.1.1.2.

6.1.7.1.1.1	 PCell conditional handover delay

-	DCHOwithCPC_PCell = TRRC_delay + max (TEvent_DU + Tmeasure_PCell, TEvent_DU + Tmeasure_PSCell) + TUE_preparation + Tprocessing + T∆_PCell + TPCell_ DU + 2 ms

-	TRRC_delay is the RRC procedure delay defined in clause 12 in TS 38.331 [2] for processing the RRC command.

-	TEvent_DU in PCell and PSCell handover delay requirement is the delay uncertainty which is the time from when the UE successfully decodes the RRC command for CHO including target MCG and candidate SCG for CPC until conditions exist at the measurement reference point which will trigger the CHO and CPC.

-	Tmeasure_PCell is the measurements time stated in clause 6.1.7.1.1.1.1.

-	Tmeasure_PSCell is the measurements time stated in clause 6.1.7.1.1.2.1.

-	TUE_preparation is the UE preparation time for conditional handover and conditional PSCell change, and starts at simultaneous execution of PCell handover and PSCell change to a target PCell and a target PSCell respectively. TUE_preparation is up to 10ms.

-	Tprocessing is the SW processing time needed by UE, including RF warm up period.

When

For FR1-FR1 NR-DC to FR1-FR1 NR-DC, Tprocessing = 30 ms if SMTC of the target unknown PSCell is configured in targetcellSMTC-SCG-r16 but not configured in reconfigurationWithSync. Otherwise, Tprocessing = 25 ms.

For FR1-FR2 NR-DC to FR1-FR1 NR-DC, Tprocessing = 50 ms if SMTC of the target unknown PSCell is configured in targetcellSMTC-SCG-r16 but not configured in reconfigurationWithSync. Otherwise, Tprocessing = 45 ms.

-	T∆_PCell is time for fine time tracking and acquiring full timing information of the target PCell. T∆ = 1*Trs ms.

-	Trs is the SMTC periodicity of the target cell if the UE has been provided with an SMTC configuration for the target cell in handover message, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this clause is applied with Trs = 5 ms assuming the SSB transmission periodicity is 5 ms. There is no requirement if the SSB transmission periodicity is not 5 ms.

-	TPCell_ DU is the delay uncertainty in acquiring the first available PRACH occasion in the PCell. TPCell_ DU is up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in Table 8.1-1 of TS 38.213 [3].

6.1.7.1.1.1.1	Measurement time

The measurement time delay is defined from the end of TEvent_DU until UE realizes the condition of PCell handover is met and identity of new PCell is determined.

For intra-frequency PCell handover, the measurement time delay measured without Time To Trigger (TTT) and L3 filtering shall be less than Tidentify intra with index or Tidentify_intra_without_index defined in clause 9.2.5.1 or clause 9.2.6.2.

For inter-frequency PCell handover, the measurement time delay measured without Time To Trigger (TTT) and L3 filtering shall be less than Tidentify_inter_without_index or Tidentify_inter_with_index defined in clause 9.3.4. When TTT or L3 filtering is used an additional delay can be expected.

A cell is detectable only if at least one SSB measured from the cell being configured remains detectable during the time period Tidentify_intra_without_index or Tidentify_intra_with_index for intra-frequency PCell handover or the time period Tidentify_inter_without_index or Tidentify_inter_with_index for inter-frequency PCell handover. If a cell, which has been detectable at least for the time period Tidentify_intra_without_index or Tidentify_intra_with_index for intra-frequency PCell handover or the time period Tidentify_inter_without_index or Tidentify_inter_with_index for inter-frequency PCell handover, becomes undetectable for a period and then the cell becomes detectable again and triggers a PCell handover, the measurement time delay shall be less than TSSB_measurement_period_intra or TSSB_measurement_period_inter provided the timing to that cell has not changed more than ± 3200/2µ Tc while the measurement gap has not been available and the L3 filter has not been used, where µ is the SCS configuration as defined in clause 4.2 of TS 38.211 [3]. When L3 filtering is used, an additional delay can be expected.

##### 6.1.7.1.1.2	 PSCell conditional change delay

-	DCHOwithCPC_PSCell = TRRC_delay + max (TEvent_DU + Tmeasure_PCell, TEvent_DU + Tmeasure_PSCell) + TUE_preparation + Tprocessing + T∆_PSCell + TPSCell_ DU + 2 ms

-	TRRC_delay is the RRC procedure delay defined in clause 12 in TS 38.331 [2] for processing the RRC command.

-	TEvent_DU in PCell and PSCell handover delay requirement is the delay uncertainty which is the time from when the UE successfully decodes the RRC command for CHO including target MCG and candidate SCG for CPC until conditions exist at the measurement reference point which will trigger the CHO and CPC.

-	Tmeasure_PCell is the measurements time stated in clause 6.1.7.1.1.1.1.

-	Tmeasure_PSCell is the measurements time stated in clause 6.1.7.1.1.2.1.

-	TUE_preparation is the UE preparation time for conditional handover and conditional PSCell change, and starts at simultaneous execution of PCell handover and PSCell change to a target PCell and a target PSCell respectively. TUE_preparation is up to 10ms.

-	Tprocessing is as specified in clause 6.1.7.1.1.1.

-	T∆_PCell is time for fine time tracking and acquiring full timing information of the target PSCell. T∆_PCell = 1*Trs ms.

-	Trs is the SMTC periodicity of the target cell if the UE has been provided with an SMTC configuration for the target cell in PSCell addition message, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this clause is applied with Trs = 5 ms assuming the SSB transmission periodicity is 5 ms. There is no requirement if the SSB transmission periodicity is not 5 ms.

-	TPSCell_ DU is the delay uncertainty in acquiring the first available PRACH occasion in the PSCell. TPSCell_ DU is up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in Table 8.1-1 of TS 38.213 [3].

6.1.7.1.1.2.1	Measurement time

The measurement time delay is defined from the end of TEvent_DU until UE realizes the condition of PSCell change is met and identity of new PSCell is determined.

For intra-frequency PSCell change, the measurement time delay measured without Time To Trigger (TTT) and L3 filtering shall be less than Tidentify intra with index or Tidentify_intra_without_index defined in clause 9.2.5.1 or clause 9.2.6.2.

For inter-frequency PSCell change, the measurement time delay measured without Time To Trigger (TTT) and L3 filtering shall be less than Tidentify_inter_without_index or Tidentify_inter_with_index defined in clause 9.3.4. When TTT or L3 filtering is used an additional delay can be expected.

A cell is detectable only if at least one SSB measured from the cell being configured remains detectable during the time period Tidentify_intra_without_index or Tidentify_intra_with_index for intra-frequency PCell handover or the time period Tidentify_inter_without_index or Tidentify_inter_with_index for inter-frequency PCell handover. If a cell, which has been detectable at least for the time period Tidentify_intra_without_index or Tidentify_intra_with_index for intra-frequency PCell handover or the time period Tidentify_inter_without_index or Tidentify_inter_with_index for inter-frequency PCell handover, becomes undetectable for a period and then the cell becomes detectable again and triggers a PCell handover, the measurement time delay shall be less than TSSB_measurement_period_intra or TSSB_measurement_period_inter provided the timing to that cell has not changed more than ± 3200/2µ Tc while the measurement gap has not been available and the L3 filter has not been used, where µ is the SCS configuration as defined in clause 4.2 of TS 38.211 [3]. When L3 filtering is used, an additional delay can be expected.

# <End of change 1>