3GPP TSG-RAN WG4 Meeting #108              	R4-2308831

Toulouse, France, 21th Aug-25th Aug, 2023

Agenda item:	8.12.5.2

Source:	Samsung

Title:	Discussion and simulation results for PDSCH with CA

Document for:	Discussion

- Introduction

In the last RAN4 meeting, the deployment, test scenario, channel model and test scope for demodulation requirements were under discussion. The related agreements were captured in the WF [1].  Regarding the test scope and test setup for PDSCH requirement with CA, the related agreement was made as following

- Carrier combination for requirements - Define Rel-18 FR2 CA PDSCH demodulation requirement with carrier combination as only TDD 120KHz SCS+TDD 120KHz SCS - Rel-18 FR2 CA PDSCH demodulation requirements for HST are only applicable for intra-band contiguous CA and intra-band non-contiguous CA combinations - Whether existing SNR requirement or 200MHz CBW defined in single carrier can be reused for CA case with 200MHz - For FR2 HST CA Demod requirements, RAN4 will define requirements for the agreed BW carrier configuration based on collected impairment results provided by companies. - RRH and Channel model assumption for CA - For FR2 HST CA Demod tests, the same RRH is assumed to transmit all CCs, Channel and Doppler trajectories should be the same for all CCs; - Additional noise margin for CA requirement based on single carrier - No additional noise and imperfection model modelling based on single carrier link level simulation for PDSCH CA alignment result is considered. - The impact of additional noise in CA can be considered into the impairment results up to companies’ implementation - Simulation results alignment and requirement derivation - Companies are encouraged to provide the simulation result in the next meeting for further alignment according to the simulation assumption R4-2305893 - The requirement derivation and whether to add additional margin will be discussed based on the collected simulation results. - Test applicability rule - Apply the existing applicability and test rules for FR2 CA PDSCH demodulation (TS38.101-4 clause 7.1.1.5) to Rel-18 CA PDSCH demodulation requirements for HST. - For HST FR2 with CA, TDD 120 KHz + TDD 120 KHz shall be considered, with the largest aggregated CA bandwidth combination shall be the highest priority - Release independent - FFS on Rel-18 FR2 HST PDSCH CA requirements are release independent from Rel-17.

In this contribution, the view on remaining issue for PDSCH requirement with CA is provided. Meanwhile, the initial simulation results are provided for alignment purpose.

# 2	Discussion

Release independent

Rel-17 FR2 HST is mainly focusing on single carrier scenario with new PC6 UE introduced to support FR2 HST scenario. Considering carrier aggregation is necessary to improve the throughput, it is proposed to introduce Rel-18 FR2 HST PDSCH requirement with CA from Rel-17. Compared with Rel-17, there is no additional UE capability introduced or signaling to support CA scenario, meanwhile, less commercial UE supporting Rel-17 FR2 HST feature is released into the market, therefore, it is feasible that Rel-18 FR2 HST PDSCH CA requirements are release independent from Rel-17.

Proposal 1: It is proposed that Rel-18 FR2 HST PDSCH CA requirements are release independent from Rel-17.

FRC

In Rel-17, only single carrier requirement with 200MHz requirement is introduced. It was agreed that to specify CA PDSCH requirements for FR2 HST with component carrier configuration as {50, 100, 200, 400} MHz for 120 KHz SCS. Therefore, new FRC tables should be introduced for both HST-DPS-FR2-UNI-A and HST-DPS-FR2-BI-B test cases.

As analyzed in our paper [2], we think the current FRC table should be updated to reflect the 3 DMRS symbol configuration in special slot, since the PDSCH was scheduled in the special slot for TDD pattern with DDDSU (S= 10D: 2GP:2U)

Proposal 2: Update the FRC for PDSCH requirement with CA as following table

Table 1	FRC for test 1 (HST-DPS-FR2-BI-B, single TCI states)

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.5-12.2 TDD (For information) | FRC 1-1 | FRC 1-2 | FRC 1-3 |
| Channel bandwidth | MHz | 200 | 50 | 100 | 400 |
| Subcarrier spacing | kHz | 120 | 120 | 120 | 120 |
| Allocated resource blocks | PRBs | 132 | 32 | 66 | 264 |
| Number of consecutive PDSCH symbols |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} |  | 9 | 9 | 9 | 9 |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {2,…,159} |  | 13 | 13 | 13 | 13 |
| For Slot I =1 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |
| Allocated slots per 2 frames |  | 126 | 126 | 126 | 126 |
| MCS table |  | 64QAM | 64QAM | 64QAM | 64QAM |
| MCS index |  | 17 | 17 | 17 | 17 |
| Modulation |  | 64QAM | 64QAM | 64QAM | 64QAM |
| Target Coding Rate |  | 0.43 | 0.43 | 0.43 | 0.43 |
| Number of MIMO layers |  | 2 | 2 | 2 | 2 |
| Number of DMRS REs |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} |  | 18 | 18 | 18 | 18 |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {2,…,159} |  | 18 | 18 | 18 | 18 |
| For Slot i = 1 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |
| Overhead for TBS determination |  | 6 | 6 | 6 | 6 |
| Information Bit Payload per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | Bits | 57376 | [13832] | [28680] | [114776] |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {2,…,159} | Bits | 90176 | [21504] | [45096] | [180376] |
| For Slot i = 1 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |
| Transport block CRC per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | Bits | 24 | 24 | 24 | 24 |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {2,…,159} | Bits | 24 | 24 | 24 | 24 |
| For Slot i = 1 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |
| Number of Code Blocks per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | CBs | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | CBs | 7 | [2] | [4] | [14] |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {2,…,159} | CBs | 11 | [3] | [6] | [22] |
| For Slot i = 1 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |
| Binary Channel Bits Per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A | N/A |
| For Slots i = 2 and 82 (Note 3) | Bits | 181968 | [33408] | [76332] | [392640] |
| For Slots i = 3 and 83 (Note 3) | Bits | 109104 | [15744] | [40200] | [246912] |
| For Slot i, if mod(i, 5) = 3 for i from {4,…,79,84,…, 159} | Bits | 137808 | [33408] | [68904] | [275616] |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {4,…,79,84,…,159} | Bits | 210672 | [51072] | [105336] | [421344] |
| For Slot i = 1 | Bits | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |
| Max. Throughput averaged over 2 frames | Mbps | 515.629 | [123.2] | [257.839] | [1031.409] |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: Binary Channel Bits are calculated under assumption of 52 PRBs TRS allocation when the number of allocated resource blocks are more than 52. Note 4: SS/PBCH block is transmitted in slot #1 with periodicity 20ms |  |  |  |  |  |

Table 2	FRC for test 2 (HST-DPS-FR2-UNI-A, two TCI state)

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.5-12.1 TDD (For information) | FRC 2-1 | FRC 2-2 | FRC 2-3 |
| Channel bandwidth | MHz | 200 | 50 | 100 | 400 |
| Subcarrier spacing | kHz | 120 | 120 | 120 | 120 |
| Allocated resource blocks | PRBs | 132 | 32 | 66 | 264 |
| Number of consecutive PDSCH symbols |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} |  | 9 | 9 | 9 | 9 |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} |  | 13 | 13 | 13 | 13 |
| For Slot i=1 |  | 13 | 13 | 13 | 13 |
| Allocated slots per 2 frames |  | 127 | 127 | 127 | 127 |
| MCS table |  | 64QAM | 64QAM | 64QAM | 64QAM |
| MCS index |  | 17 | 17 | 17 | 17 |
| Modulation |  | 64QAM | 64QAM | 64QAM | 64QAM |
| Target Coding Rate |  | 0.43 | 0.43 | 0.43 | 0.43 |
| Number of MIMO layers |  | 2 | 2 | 2 | 2 |
| Number of DMRS REs |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} |  | 18 | 18 | 18 | 18 |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {2,…,159} |  | 18 | 18 | 18 | 18 |
| For Slot i = 1 |  | 18 | 18 | 18 | 18 |
| Overhead for TBS determination |  | 6 | 6 | 6 | 6 |
| Information Bit Payload per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | Bits | 57376 | [13832] | [28680] | [114776] |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {2,…,159} | Bits | 90176 | [21504] | [45096] | [180376] |
| For Slot i = 1 |  | 90176 | [21504] | [45096] | [180376] |
| Transport block CRC per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | Bits | 24 | 24 | 24 | 24 |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {2,…,159} | Bits | 24 | 24 | 24 | 24 |
| For Slot i = 1 |  | 24 | 24 | 24 | 24 |
| Number of Code Blocks per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | CBs | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | CBs | 7 | [2] | [4] | [14] |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {2,…,159} | CBs | 11 | [3] | [6] | [22] |
| For Slot i = 1 |  | 11 | [3] | [6] | [22] |
| Binary Channel Bits Per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A | N/A |
| For Slots i = 2 and 82 (Note 3) | Bits | 195696 | [41856] | [90360] | [406368] |
| For Slots i = 3 and 83 (Note 3) | Bits | 122832 | [24192] | [53928] | [260640] |
| For Slot i, if mod(i, 5) = 3 for i from {4,…,79,84,…, 159} | Bits | 137808 | [33408] | [68904] | [293040] |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {4,…,79,84,…,159} | Bits | 210672 | [51072] | [105336] | [421344] |
| For Slot i = 1 | Bits | 210672 | [51072] | [105336] | [421344] |
| Max. Throughput averaged over 2 frames | Mbps | 520.138 | [124.275] | [260.094] | [1040.428] |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: Binary Channel Bits are calculated under assumption of 52 PRBs TRS allocation when the number of allocated resource blocks are more than 52. Note 4: SS/PBCH block is transmitted in slot #1 with periodicity 20ms |  |  |  |  |  |

# 3	Simulation results

In this section, the initial simulation results are provided based on agreed test cases listed in the R4-2305893.

Table 3-1 ideal simulation results for PDSCH requirement with CA

| Test number | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Number of active PDSCH TCI states | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | FRC 1-1 | 50 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-BI-B | 1 | 2x2 | 70 | 10.7 |
| 1-2 | FRC 1-2 | 100 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-BI-B | 1 | 2x2 | 70 | 11 |
| 1-3 | FRC 1-3 | 400 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-BI-B | 1 | 2x2 | 70 | 11.4 |
| 1-4 | R.PDSCH.5-12.2 TDD | 200 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-BI-B | 1 | 2x2 | 70 | 11.5 |
| 2-1 | FRC 2-1 | 50 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-UNI-A | 2 | 2x2 | 70 | 10.9 |
| 2-2 | FRC 2-2 | 100 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-UNI-A | 2 | 2x2 | 70 | 11.2 |
| 2-3 | FRC 2-3 | 400 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-UNI-A | 2 | 2x2 | 70 | 11.5 |
| 2-4 | R.PDSCH.5-12.1 TDD | 200 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-UNI-A | 2 | 2x2 | 70 | 11.6 |

# 4	Conclusion

In this contribution, the view on the remaining issue of test setup for PDSCH requirement with CA is provided. Meanwhile, the initial simulation results are provided for alignment purpose

Proposal 1: It is proposed that Rel-18 FR2 HST PDSCH CA requirements are release independent from Rel-17.

Proposal 2: Update the FRC for PDSCH requirement with CA as following table

Table 1	FRC for test 1 (HST-DPS-FR2-BI-B, single TCI states)

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.5-12.2 TDD (For information) | FRC 1-1 | FRC 1-2 | FRC 1-3 |
| Channel bandwidth | MHz | 200 | 50 | 100 | 400 |
| Subcarrier spacing | kHz | 120 | 120 | 120 | 120 |
| Allocated resource blocks | PRBs | 132 | 32 | 66 | 264 |
| Number of consecutive PDSCH symbols |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} |  | 9 | 9 | 9 | 9 |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {2,…,159} |  | 13 | 13 | 13 | 13 |
| For Slot I =1 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |
| Allocated slots per 2 frames |  | 126 | 126 | 126 | 126 |
| MCS table |  | 64QAM | 64QAM | 64QAM | 64QAM |
| MCS index |  | 17 | 17 | 17 | 17 |
| Modulation |  | 64QAM | 64QAM | 64QAM | 64QAM |
| Target Coding Rate |  | 0.43 | 0.43 | 0.43 | 0.43 |
| Number of MIMO layers |  | 2 | 2 | 2 | 2 |
| Number of DMRS REs |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} |  | 18 | 18 | 18 | 18 |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {2,…,159} |  | 18 | 18 | 18 | 18 |
| For Slot i = 1 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |
| Overhead for TBS determination |  | 6 | 6 | 6 | 6 |
| Information Bit Payload per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | Bits | 57376 | [13832] | [28680] | [114776] |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {2,…,159} | Bits | 90176 | [21504] | [45096] | [180376] |
| For Slot i = 1 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |
| Transport block CRC per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | Bits | 24 | 24 | 24 | 24 |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {2,…,159} | Bits | 24 | 24 | 24 | 24 |
| For Slot i = 1 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |
| Number of Code Blocks per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | CBs | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | CBs | 7 | [2] | [4] | [14] |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {2,…,159} | CBs | 11 | [3] | [6] | [22] |
| For Slot i = 1 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |
| Binary Channel Bits Per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A | N/A |
| For Slots i = 2 and 82 (Note 3) | Bits | 181968 | [33408] | [76332] | [392640] |
| For Slots i = 3 and 83 (Note 3) | Bits | 109104 | [15744] | [40200] | [246912] |
| For Slot i, if mod(i, 5) = 3 for i from {4,…,79,84,…, 159} | Bits | 137808 | [33408] | [68904] | [275616] |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {4,…,79,84,…,159} | Bits | 210672 | [51072] | [105336] | [421344] |
| For Slot i = 1 | Bits | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |
| Max. Throughput averaged over 2 frames | Mbps | 515.629 | [123.2] | [257.839] | [1031.409] |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: Binary Channel Bits are calculated under assumption of 52 PRBs TRS allocation when the number of allocated resource blocks are more than 52. Note 4: SS/PBCH block is transmitted in slot #1 with periodicity 20ms |  |  |  |  |  |

Table 2	FRC for test 2 (HST-DPS-FR2-UNI-A, two TCI state)

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.5-12.1 TDD (For information) | FRC 2-1 | FRC 2-2 | FRC 2-3 |
| Channel bandwidth | MHz | 200 | 50 | 100 | 400 |
| Subcarrier spacing | kHz | 120 | 120 | 120 | 120 |
| Allocated resource blocks | PRBs | 132 | 32 | 66 | 264 |
| Number of consecutive PDSCH symbols |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} |  | 9 | 9 | 9 | 9 |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} |  | 13 | 13 | 13 | 13 |
| For Slot i=1 |  | 13 | 13 | 13 | 13 |
| Allocated slots per 2 frames |  | 127 | 127 | 127 | 127 |
| MCS table |  | 64QAM | 64QAM | 64QAM | 64QAM |
| MCS index |  | 17 | 17 | 17 | 17 |
| Modulation |  | 64QAM | 64QAM | 64QAM | 64QAM |
| Target Coding Rate |  | 0.43 | 0.43 | 0.43 | 0.43 |
| Number of MIMO layers |  | 2 | 2 | 2 | 2 |
| Number of DMRS REs |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} |  | 18 | 18 | 18 | 18 |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {2,…,159} |  | 18 | 18 | 18 | 18 |
| For Slot i = 1 |  | 18 | 18 | 18 | 18 |
| Overhead for TBS determination |  | 6 | 6 | 6 | 6 |
| Information Bit Payload per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | Bits | 57376 | [13832] | [28680] | [114776] |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {2,…,159} | Bits | 90176 | [21504] | [45096] | [180376] |
| For Slot i = 1 |  | 90176 | [21504] | [45096] | [180376] |
| Transport block CRC per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | Bits | 24 | 24 | 24 | 24 |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {2,…,159} | Bits | 24 | 24 | 24 | 24 |
| For Slot i = 1 |  | 24 | 24 | 24 | 24 |
| Number of Code Blocks per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | CBs | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | CBs | 7 | [2] | [4] | [14] |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {2,…,159} | CBs | 11 | [3] | [6] | [22] |
| For Slot i = 1 |  | 11 | [3] | [6] | [22] |
| Binary Channel Bits Per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A | N/A |
| For Slots i = 2 and 82 (Note 3) | Bits | 195696 | [41856] | [90360] | [406368] |
| For Slots i = 3 and 83 (Note 3) | Bits | 122832 | [24192] | [53928] | [260640] |
| For Slot i, if mod(i, 5) = 3 for i from {4,…,79,84,…, 159} | Bits | 137808 | [33408] | [68904] | [293040] |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {4,…,79,84,…,159} | Bits | 210672 | [51072] | [105336] | [421344] |
| For Slot i = 1 | Bits | 210672 | [51072] | [105336] | [421344] |
| Max. Throughput averaged over 2 frames | Mbps | 520.138 | [124.275] | [260.094] | [1040.428] |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: Binary Channel Bits are calculated under assumption of 52 PRBs TRS allocation when the number of allocated resource blocks are more than 52. Note 4: SS/PBCH block is transmitted in slot #1 with periodicity 20ms |  |  |  |  |  |

# References

R4-2309825    WF on NR FR2 HST Enhancements RRM requirements		Samsung

R4-2312217    FRC and simulation assumption correction for Rel-17 FR2 HST       Samsung