3GPP TSG-RAN WG4 Meeting # 108	R4-2312286

# Toulouse, France, August 21 – August 25, 2023

Source:	LG Electronics

Title:	Discussion on RRM core requirement for SL positioning enhancement

Agenda item:	8.22.3.2

Document for:	Discussion

Introduction

In this contribution, we provide our views on SL positioning measurements and procedures related to RRM issues based on the approved WF [1] in RAN4#107 and simulation results based on simulation assumption [2] in RAN4#107.

Discussion

Issue 1-1-3: Parameters in the measurement period requirements:

- The measurements period requirements equation is FFS - Number of samples for positioning measurements is FFS - Option 1: 1 - Option 2: 2 - Option 3: 3 - Option 4: 4 - The number of Rx beams is equal to 1 in FR1.

Regarding the number of samples for positioning measurement it would be better to consider after evaluation.

Proposal 1: The number of samples can be consider after SL-PRS detection performance evaluation.

Issue 1-3-3: Simulation assumption:

- SL PRS pattern: - Case 1: Fully staggered SL PRS patterns - Case 2: Partially staggered SL PRS patterns - Channel model for RSTD, Rx-Tx and RSRP: - AWGN - TDL-A (30 ns delay spread, 5Hz Doppler spread) - TDL-B (100ns delay spread, 200Hz Doppler spread) - Antenna: - 1T/2R - SL PRS BW: - 10 MHz, 20 MHz, 40 MHz - SCS: - 15kHz, 30kHz, 60kHz - CBW: - equal to the SL PRS BW - Sampling rate: - Reuse the assumption for NR PRS based measurements - Other details can be found in R4-2310077 and its revision.

According to the simulation assumption [2] in RAN4#107 the detail parameters are captured below.

Table 1: General parameters

| Parameter | Value |
| --- | --- |
| Frequency Range | FR1 |
| Anchor UEs configuration | 3 anchor UEs at distinct locations: <UE 1, UE 2, UE 3>, where UE 1 is the reference anchor UE |
| Synchronization of anchor UEs | • Synchronous with time shifts <0, 0, 3 us> |
| Data and CCH load in PRS symbols | no other cell transmissions in its positioning symbols, except PRS |
| Data and CCH load in non-PRS symbols | - 50% utilization in time - 100% RE utilization |
| DRX | OFF |
| Carrier frequency / BW / SCS | - 2 GHz - 15 kHz - 4 GHz - 30 kHz, 60 kHz |
| Propagation conditions [TS 38.101-4] | For SL-PRS based RSTD, RSRP and Rx-Tx time difference: - AWGN, - TDL-A (30 ns delay spread, 5Hz Doppler shift), - TDL-B (100 ns delay spread, 200Hz Doppler shift). |
|  | For SL-PRS based RSRPP: Two-tap channel defined in 38.101-4 Annex B.2.4, a = 1, τd=0.45 µs and fD=5 Hz |
| Es/Iot for three anchor UEs (UE 1, UE 2, UE 3), [dB] | For SL-PRS based RSTD/Rx-Tx/RSRP/RSRPP: (-3, -6, -6), (0, -6, -6) |
| Number of UE receive antennas | 2 |
| UE measurement bandwidth | SL PRS BW |
| Number of transmit SL-PRS antennas | 1 |
| Power boosting | No power boosting |
| NOTE: Companies are encouraged to check suitability of the parameters and the values. |  |

Table 2: SL-PRS transmission configuration parameters

| Parameter | Value |  |  |
| --- | --- | --- | --- |
| SCS, RB num | SCS (kHz) | RB num | Sample rate (Tc) |
|  | 15 | 52 | 128 |
|  |  | 104 | 64 |
|  | 30 | 24 | 128 |
|  |  | 48 | 64 |
|  |  | 96 | 32 |
|  | 60 | 24 | 64 |
| SL-PRS comb size (N) | 4 |  |  |
| SL-PRS number of symbols (M) | 4 (for full staggering),2 (for partial staggering) |  |  |
| Number of samples | 1, 2,3,4 |  |  |
| SL-PRS periodicity | 40ms |  |  |
| TOA estimation | Realistic |  |  |
| Path # | First path |  |  |
| NOTE 1: Companies are encouraged to check suitability of the parameters and the values. NOTE 2: For number of samples equal to 1, only configurations with RB num ≥ 48 are simulated. |  |  |  |

And the performance metric also captured below.

At least the following performance characteristics are to be provided for SL-PRS RSTD: - SL-PRS RSTD error CDFs for the 2 anchor UEs (UE 2 and UE 3) - 5%-ile and 95%-ile of the SL-PRS RSTD errors At least the following performance characteristics are to be provided for SL-PRS RSRP: - SL-PRS RSRP error CDFs for 3 anchor UEs - 5%-ile and 95%-ile of the SL-PRS RSRP errors for each anchor UE At least the following performance characteristics are to be provided for SL-PRS RSRPP: - SL-PRS RSRPP error CDFs for 3 anchor UEs - 5%-ile and 95%-ile of the SL-PRS RSRPP errors for each anchor UE At least the following performance characteristics are to be provided for SL-PRS Rx-Tx: - TSL-UE-RX error CDFs for the 3 anchor UEs - 90%-ile of the TSL-UE-RX errors for each anchor UE In the above SL PRS based measurement, - RSTD error = estimated RSTD – ideal RSTD (based on perfect channel knowledge). - RSRP error = estimated RSRP – ideal RSRP (based on perfect channel knowledge). - RSRPP error = estimated RSRPP – ideal RSRPP (refer to 10.1.38.2.1 of TS 38.133). Rx-Tx error = abs(estimated TSL-UE-RX – ideal TSL-UE-RX ) (based on perfect channel and UE location knowledge).

We simulated only 100% RE utilization and 1 sample case.

SL-PRS RSTD error CDF [unit: Tc]

| SCS |  |  | 15 kHz |  |  |  | 30 kHz |  |  |  |  |  | 60 kHz |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RB num |  |  | 52 |  | 104 |  | 24 |  | 48 |  | 96 |  | 24 |  |
|  |  |  | 5%-ile | 95%-ile | 5%-ile | 95%-ile | 5%-ile | 95%-ile | 5%-ile | 95%-ile | 5%-ile | 95%-ile | 5%-ile | 95%-ile |
| AWGN | Es/Iot -3,-6,-6 | M: 2 | 5888 | 5888 | 5888 | 5888 | 5888 | 5888 | 5888 | 5888 | 5888 | 5888 | 5888 | 5888 |
|  |  | M: 4 | 5888 | 5888 | 5888 | 5888 | 5888 | 5888 | 5888 | 5888 | 5888 | 5888 | 5888 | 5888 |
|  | Es/Iot 0,-6,-6 | M: 2 | 5888 | 5888 | 5888 | 5888 | 5888 | 5888 | 5888 | 5888 | 5888 | 5888 | 5888 | 5888 |
|  |  | M: 4 | 5888 | 5888 | 5888 | 5888 | 5888 | 5888 | 5888 | 5888 | 5888 | 5888 | 5888 | 5888 |
| TDL-A (30 ns delay spread, 5Hz Doppler shilt) | Es/Iot -3,-6,-6 | M: 2 | 5888 | 6016 | 5888 | 6016 | 5888 | 6144 | 5888 | 6016 | 5888 | 6016 | 5888 | 6016 |
|  |  | M: 4 | 5888 | 6016 | 5888 | 6016 | 5888 | 6144 | 5888 | 6016 | 5888 | 6016 | 5888 | 6016 |
|  | Es/Iot 0,-6,-6 | M: 2 | 5888 | 6016 | 5888 | 6016 | 5888 | 6144 | 5888 | 6016 | 5888 | 6016 | 5888 | 6016 |
|  |  | M: 4 | 5888 | 6016 | 5888 | 6016 | 5888 | 6144 | 5888 | 6016 | 5888 | 6016 | 5888 | 6016 |
| TDL-B (100 ns delay spread, 200Hz Doppler shilt) | Es/Iot -3,-6,-6 | M: 2 | 5632 | 6272 | 5760 | 6144 | 5632 | 6272 | 5760 | 6144 | 5760 | 6016 | 5760 | 6144 |
|  |  | M: 4 | 5632 | 6272 | 5760 | 6144 | 5632 | 6272 | 5760 | 6144 | 5760 | 6016 | 5760 | 6144 |
|  | Es/Iot 0,-6,-6 | M: 2 | 5632 | 6272 | 5760 | 6144 | 5632 | 6272 | 5760 | 6144 | 5760 | 6016 | 5632 | 6144 |
|  |  | M: 4 | 5632 | 6272 | 5760 | 6144 | 5632 | 6272 | 5760 | 6144 | 5760 | 6016 | 5632 | 6144 |

SL-PRS RSRP error CDF - UE1 [unit: dB]

| SCS |  |  | 15 kHz |  |  |  | 30 kHz |  |  |  |  |  | 60 kHz |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RB num |  |  | 52 |  | 104 |  | 24 |  | 48 |  | 96 |  | 24 |  |
|  |  |  | 5%-ile | 95%-ile | 5%-ile | 95%-ile | 5%-ile | 95%-ile | 5%-ile | 95%-ile | 5%-ile | 95%-ile | 5%-ile | 95%-ile |
| AWGN | Es/Iot -3,-6,-6 | M: 2 | -0.6 | 0.6 | -0.4 | 0.4 | -1.05 | 0.95 | -0.65 | 0.65 | -0.45 | 0.45 | -1 | 1 |
|  |  | M: 4 | -0.45 | 0.4 | -0.3 | 0.3 | -0.7 | 0.65 | -0.45 | 0.45 | -0.3 | 0.3 | -0.65 | 0.65 |
|  | Es/Iot 0,-6,-6 | M: 2 | -0.4 | 0.4 | -0.3 | 0.3 | -0.6 | 0.6 | -0.45 | 0.45 | -0.3 | 0.3 | -0.6 | 0.6 |
|  |  | M: 4 | -0.3 | 0.3 | -0.25 | 0.25 | -.045 | 0.4 | -0.3 | 0.3 | -0.2 | 0.2 | -0.4 | 0.4 |
| TDL-A (30 ns delay spread, 5Hz Doppler shilt) | Es/Iot -3,-6,-6 | M: 2 | -4.4 | 3.6 | -3.6 | 2.8 | -4 | 3.4 | -4 | 2.8 | -3.6 | 2.8 | -3.2 | 2.4 |
|  |  | M: 4 | -4.4 | 3.6 | -3.6 | 2.8 | -4.4 | 3.4 | -3.6 | 2.8 | -4 | 3 | -3.4 | 2.8 |
|  | Es/Iot 0,-6,-6 | M: 2 | -4 | 3.6 | -3.6 | 2.8 | -4.8 | 3.6 | -3.6 | 2.8 | -4 | 2.8 | -3.6 | 2.8 |
|  |  | M: 4 | -4 | 3.6 | -3.6 | 2.8 | -4.4 | 3.6 | -4 | 2.8 | -3.6 | 2.8 | -3.6 | 2.8 |
| TDL-B (100 ns delay spread, 200Hz Doppler shilt) | Es/Iot -3,-6,-6 | M: 2 | -3.6 | 3.2 | -3.2 | 2.8 | -2.8 | 2.8 | -3.2 | 2.8 | -2.4 | 2.4 | -2.6 | 2.4 |
|  |  | M: 4 | -3.6 | 3 | -3.2 | 2.8 | -3.4 | 2.8 | -3.2 | 3 | -2.4 | 2.2 | -3.2 | 2.6 |
|  | Es/Iot 0,-6,-6 | M: 2 | -3.6 | 3.2 | -3.6 | 2.8 | -3.6 | 3.2 | -3.6 | 2.8 | -2.8 | 2.4 | -3.6 | 2.8 |
|  |  | M: 4 | -3.6 | 3.2 | -3.6 | 2.8 | -3.6 | 3.2 | -3.6 | 2.8 | -2.8 | 2.2 | -3.2 | 2.8 |

SL-PRS RSRP error CDF – UE2 [unit: dB]

| SCS |  |  | 15 kHz |  |  |  | 30 kHz |  |  |  |  |  | 60 kHz |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RB num |  |  | 52 |  | 104 |  | 24 |  | 48 |  | 96 |  | 24 |  |
|  |  |  | 5%-ile | 95%-ile | 5%-ile | 95%-ile | 5%-ile | 95%-ile | 5%-ile | 95%-ile | 5%-ile | 95%-ile | 5%-ile | 95%-ile |
| AWGN | Es/Iot -3,-6,-6 | M: 2 | -0.8 | 0.8 | -0.6 | 0.6 | -1.35 | 1.25 | -0.9 | 0.9 | -0.6 | 0.6 | -1.5 | 1.5 |
|  |  | M: 4 | -0.65 | 0.55 | -0.4 | 0.4 | -0.9 | 0.9 | -0.6 | 0.6 | -0.45 | 0.45 | -0.9 | 0.9 |
|  | Es/Iot 0,-6,-6 | M: 2 | -1.2 | 1.3 | -0.8 | 1.2 | -1.9 | 1.4 | -1.7 | 1.1 | -1.4 | 0.8 | -2 | 1.5 |
|  |  | M: 4 | -1 | 1.2 | -0.6 | 1.15 | -1.6 | 0.9 | -1.5 | 0.8 | -1.4 | 0.6 | -1.7 | 1.2 |
| TDL-A (30 ns delay spread, 5Hz Doppler shilt) | Es/Iot -3,-6,-6 | M: 2 | -4.4 | 3.6 | -3.8 | 3.2 | -4 | 3.4 | -4 | 3 | -4.4 | 3.2 | -3.4 | 2.8 |
|  |  | M: 4 | -4.4 | 3.6 | -3.8 | 3.0 | -4.4 | 3.4 | -3.6 | 2.8 | -4.4 | 3.2 | -3.4 | 2.8 |
|  | Es/Iot 0,-6,-6 | M: 2 | -5.6 | 4 | -4.4 | 3.6 | -5.6 | 4 | -4.4 | 3.6 | -4.8 | 3.6 | -4.8 | 3.6 |
|  |  | M: 4 | -5.6 | 4 | -4.4 | 3.6 | -5.6 | 4 | -4.8 | 3.6 | -5.2 | 4 | -4.8 | 3.6 |
| TDL-B (100 ns delay spread, 200Hz Doppler shilt) | Es/Iot -3,-6,-6 | M: 2 | -4 | 3.4 | -4 | 3.2 | -3.2 | 3 | -4 | 3.2 | -3.6 | 2.8 | -3.2 | 2.6 |
|  |  | M: 4 | -4 | 3.4 | -4 | 3.2 | -3.6 | 3 | -4 | 3.2 | -4 | 2.8 | -3.6 | 2.8 |
|  | Es/Iot 0,-6,-6 | M: 2 | -4.8 | 4 | -4.4 | 4 | -5.2 | 4 | -4.4 | 4.4 | -4 | 4 | -4.4 | 4 |
|  |  | M: 4 | -4.8 | 4 | -4.4 | 4 | -5.2 | 4 | -4.4 | 4.4 | -4 | 4 | -4.4 | 4 |

SL-PRS RSRP error CDF – UE3 [unit: dB]

| SCS |  |  | 15 kHz |  |  |  | 30 kHz |  |  |  |  |  | 60 kHz |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RB num |  |  | 52 |  | 104 |  | 24 |  | 48 |  | 96 |  | 24 |  |
|  |  |  | 5%-ile | 95%-ile | 5%-ile | 95%-ile | 5%-ile | 95%-ile | 5%-ile | 95%-ile | 5%-ile | 95%-ile | 5%-ile | 95%-ile |
| AWGN | Es/Iot -3,-6,-6 | M: 2 | -0.8 | 0.8 | -0.6 | 0.6 | -1.35 | 1.25 | -0.9 | 0.9 | -0.6 | 0.6 | -1.5 | 1.5 |
|  |  | M: 4 | -0.65 | 0.55 | -0.4 | 0.4 | -0.9 | 0.9 | -0.6 | 0.6 | -0.45 | 0.45 | -0.9 | 0.9 |
|  | Es/Iot 0,-6,-6 | M: 2 | -1.2 | 1.3 | -0.8 | 1.2 | -1.9 | 1.4 | -1.7 | 1.1 | -1.4 | 0.8 | -2 | 1.5 |
|  |  | M: 4 | -1 | 1.2 | -0.6 | 1.15 | -1.6 | 0.9 | -1.5 | 0.8 | -1.4 | 0.6 | -1.7 | 1.2 |
| TDL-A (30 ns delay spread, 5Hz Doppler shilt) | Es/Iot -3,-6,-6 | M: 2 | -4.4 | 3.6 | -3.8 | 3.2 | -4 | 3.4 | -4 | 3 | -4.4 | 3.2 | -3.4 | 2.8 |
|  |  | M: 4 | -4.4 | 3.6 | -3.8 | 3.0 | -4.4 | 3.4 | -3.6 | 2.8 | -4.4 | 3.2 | -3.4 | 2.8 |
|  | Es/Iot 0,-6,-6 | M: 2 | -5.6 | 4 | -4.4 | 3.6 | -5.6 | 4 | -4.4 | 3.6 | -4.8 | 3.6 | -4.8 | 3.6 |
|  |  | M: 4 | -5.6 | 4 | -4.4 | 3.6 | -5.6 | 4 | -4.8 | 3.6 | -5.2 | 4 | -4.8 | 3.6 |
| TDL-B (100 ns delay spread, 200Hz Doppler shilt) | Es/Iot -3,-6,-6 | M: 2 | -4 | 3.4 | -4 | 3.2 | -3.2 | 3 | -4 | 3.2 | -3.6 | 2.8 | -3.2 | 2.6 |
|  |  | M: 4 | -4 | 3.4 | -4 | 3.2 | -3.6 | 3 | -4 | 3.2 | -4 | 2.8 | -3.6 | 2.8 |
|  | Es/Iot 0,-6,-6 | M: 2 | -4.8 | 4 | -4.4 | 4 | -5.2 | 4 | -4.4 | 4.4 | -4 | 4 | -4.4 | 4 |
|  |  | M: 4 | -4.8 | 4 | -4.4 | 4 | -5.2 | 4 | -4.4 | 4.4 | -4 | 4 | -4.4 | 4 |

SL-PRS Rx-Tx error CDF - UE1 [unit: Tc]

| SCS |  |  | 15 kHz |  |  |  | 30 kHz |  |  |  |  |  | 60 kHz |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RB num |  |  | 52 |  | 104 |  | 24 |  | 48 |  | 96 |  | 24 |  |
|  |  |  | 5%-ile | 95%-ile | 5%-ile | 95%-ile | 5%-ile | 95%-ile | 5%-ile | 95%-ile | 5%-ile | 95%-ile | 5%-ile | 95%-ile |
| AWGN | Es/Iot -3,-6,-6 | M: 2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  |  | M: 4 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | Es/Iot 0,-6,-6 | M: 2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  |  | M: 4 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| TDL-A (30 ns delay spread, 5Hz Doppler shilt) | Es/Iot -3,-6,-6 | M: 2 | 0 | 128 | 0 | 128 | 0 | 256 | 0 | 128 | 0 | 128 | 0 | 128 |
|  |  | M: 4 | 0 | 128 | 0 | 128 | 0 | 256 | 0 | 128 | 0 | 128 | 0 | 128 |
|  | Es/Iot 0,-6,-6 | M: 2 | 0 | 128 | 0 | 128 | 0 | 256 | 0 | 128 | 0 | 128 | 0 | 128 |
|  |  | M: 4 | 0 | 128 | 0 | 128 | 0 | 256 | 0 | 128 | 0 | 128 | 0 | 128 |
| TDL-B (100 ns delay spread, 200Hz Doppler shilt) | Es/Iot -3,-6,-6 | M: 2 | 0 | 384 | 0 | 256 | 0 | 384 | 0 | 256 | 0 | 256 | 0 | 256 |
|  |  | M: 4 | 0 | 384 | 0 | 256 | 0 | 384 | 0 | 256 | 0 | 256 | 0 | 256 |
|  | Es/Iot 0,-6,-6 | M: 2 | 0 | 384 | 0 | 256 | 0 | 384 | 0 | 256 | 0 | 256 | 0 | 256 |
|  |  | M: 4 | 0 | 384 | 0 | 256 | 0 | 384 | 0 | 256 | 0 | 256 | 0 | 256 |

SL-PRS Rx-Tx error CDF – UE2 [unit: Tc]

| SCS |  |  | 15 kHz |  |  |  | 30 kHz |  |  |  |  |  | 60 kHz |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RB num |  |  | 52 |  | 104 |  | 24 |  | 48 |  | 96 |  | 24 |  |
|  |  |  | 5%-ile | 95%-ile | 5%-ile | 95%-ile | 5%-ile | 95%-ile | 5%-ile | 95%-ile | 5%-ile | 95%-ile | 5%-ile | 95%-ile |
| AWGN | Es/Iot -3,-6,-6 | M: 2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  |  | M: 4 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | Es/Iot 0,-6,-6 | M: 2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  |  | M: 4 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| TDL-A (30 ns delay spread, 5Hz Doppler shilt) | Es/Iot -3,-6,-6 | M: 2 | 0 | 128 | 0 | 128 | 0 | 256 | 0 | 128 | 0 | 128 | 0 | 128 |
|  |  | M: 4 | 0 | 128 | 0 | 128 | 0 | 256 | 0 | 128 | 0 | 128 | 0 | 128 |
|  | Es/Iot 0,-6,-6 | M: 2 | 0 | 128 | 0 | 128 | 0 | 256 | 0 | 128 | 0 | 128 | 0 | 128 |
|  |  | M: 4 | 0 | 128 | 0 | 128 | 0 | 256 | 0 | 128 | 0 | 128 | 0 | 128 |
| TDL-B (100 ns delay spread, 200Hz Doppler shilt) | Es/Iot -3,-6,-6 | M: 2 | 0 | 384 | 0 | 256 | 0 | 384 | 0 | 256 | 0 | 256 | 0 | 256 |
|  |  | M: 4 | 0 | 384 | 0 | 256 | 0 | 384 | 0 | 256 | 0 | 256 | 0 | 256 |
|  | Es/Iot 0,-6,-6 | M: 2 | 0 | 384 | 0 | 256 | 0 | 384 | 0 | 256 | 0 | 256 | 0 | 256 |
|  |  | M: 4 | 0 | 384 | 0 | 256 | 0 | 384 | 0 | 256 | 0 | 256 | 0 | 256 |

SL-PRS Rx-Tx error CDF – UE3 [unit: Tc]

| SCS |  |  | 15 kHz |  |  |  | 30 kHz |  |  |  |  |  | 60 kHz |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RB num |  |  | 52 |  | 104 |  | 24 |  | 48 |  | 96 |  | 24 |  |
|  |  |  | 5%-ile | 95%-ile | 5%-ile | 95%-ile | 5%-ile | 95%-ile | 5%-ile | 95%-ile | 5%-ile | 95%-ile | 5%-ile | 95%-ile |
| AWGN | Es/Iot -3,-6,-6 | M: 2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  |  | M: 4 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | Es/Iot 0,-6,-6 | M: 2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  |  | M: 4 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| TDL-A (30 ns delay spread, 5Hz Doppler shilt) | Es/Iot -3,-6,-6 | M: 2 | 0 | 128 | 0 | 128 | 0 | 256 | 0 | 128 | 0 | 128 | 0 | 128 |
|  |  | M: 4 | 0 | 128 | 0 | 128 | 0 | 256 | 0 | 128 | 0 | 128 | 0 | 128 |
|  | Es/Iot 0,-6,-6 | M: 2 | 0 | 128 | 0 | 128 | 0 | 256 | 0 | 128 | 0 | 128 | 0 | 128 |
|  |  | M: 4 | 0 | 128 | 0 | 128 | 0 | 256 | 0 | 128 | 0 | 128 | 0 | 128 |
| TDL-B (100 ns delay spread, 200Hz Doppler shilt) | Es/Iot -3,-6,-6 | M: 2 | 0 | 384 | 0 | 256 | 0 | 384 | 0 | 256 | 0 | 256 | 0 | 256 |
|  |  | M: 4 | 0 | 384 | 0 | 256 | 0 | 384 | 0 | 256 | 0 | 256 | 0 | 256 |
|  | Es/Iot 0,-6,-6 | M: 2 | 0 | 384 | 0 | 256 | 0 | 384 | 0 | 256 | 0 | 256 | 0 | 256 |
|  |  | M: 4 | 0 | 384 | 0 | 256 | 0 | 384 | 0 | 256 | 0 | 256 | 0 | 256 |

SL-PRS RSRPP error CDF - UE1: RSRPP of path1/RSRPP of path2 [unit: dB]

| SCS |  |  | 15 kHz |  |  |  | 30 kHz |  |  |  |  |  | 60 kHz |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RB num |  |  | 52 |  | 104 |  | 24 |  | 48 |  | 96 |  | 24 |  |
|  |  |  | 5%-ile | 95%-ile | 5%-ile | 95%-ile | 5%-ile | 95%-ile | 5%-ile | 95%-ile | 5%-ile | 95%-ile | 5%-ile | 95%-ile |
| Two-tap channel | Es/Iot -3,-6,-6 | M: 2 | -0.7/-0.7 | 0.7/0.7 | -0.5/-0.5 | 0.5/0.5 | -1.2/-1.2 | 1.2/1.2 | -0.8/-0.8 | 0.8/0.8 | -0.6/-0.6 | 0.6/0.6 | -1.2/-1.2 | 1.2/1.2 |
|  |  | M: 4 | -0.5/-0.5 | 0.5/0.5 | -0.4/-0.4 | 0.4/0.4 | -0.8/-0.8 | 0.8/0.8 | -0.6/-0.6 | 0.6/0.6 | -0.5/-0.5 | 0.5/0.5 | -0.8/-0.8 | 0.8/0.8 |
|  | Es/Iot 0,-6,-6 | M: 2 | -0.5/-0.5 | 0.5/0.5 | -0.4/-0.4 | 0.4/0.4 | -0.8/-0.8 | 0.8/0.8 | -0.6/-0.6 | 0.6/0.6 | -0.4/-0.4 | 0.4/0.4 | -0.8/-0.8 | 0.8/0.8 |
|  |  | M: 4 | -0.4/-0.4 | 0.4/0.4 | -0.3/-0.3 | 0.3/0.3 | -0.6/-0.6 | 0.6/0.6 | -0.4/-0.4 | 0.4/0.4 | -0.3/-0.3 | 0.3/0.3 | -0.6/-0.6 | 0.6/0.6 |

SL-PRS RSRPP error CDF – UE2: RSRPP of path1/RSRPP of path2

| SCS |  |  | 15 kHz |  |  |  | 30 kHz |  |  |  |  |  | 60 kHz |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RB num |  |  | 52 |  | 104 |  | 24 |  | 48 |  | 96 |  | 24 |  |
|  |  |  | 5%-ile | 95%-ile | 5%-ile | 95%-ile | 5%-ile | 95%-ile | 5%-ile | 95%-ile | 5%-ile | 95%-ile | 5%-ile | 95%-ile |
| Two-tap channel | Es/Iot -3,-6,-6 | M: 2 | -1/-1 | 1/1 | -0.7/-0.7 | 0.7/0.7 | -1.6/-1.6 | 1.6/1.6 | -1.2/-1.2 | 1.2/1.2 | -0.8/-0.8 | 0.8/0.8 | -1.6/-1.6 | 1.6/1.6 |
|  |  | M: 4 | -0.7/-0.7 | 0.7/0.7 | -0.5/-0.5 | 0.5/0.5 | -1.2/-1.2 | 1.2/1.2 | -0.8/-0.8 | 0.8/0.8 | -0.6/-0.6 | 0.6/0.6 | -1.2/-1.2 | 1.2/1.2 |
|  | Es/Iot 0,-6,-6 | M: 2 | -1/-1 | 1/1 | -0.7/-0.7 | 0.7/0.7 | -1.6/-1.6 | 1.6/1.6 | -1.2/-1.2 | 1.2/1.2 | -0.8/-0.8 | 0.8/0.8 | -1.8/-1.8 | 1.8/1.8 |
|  |  | M: 4 | -0.7/-0.7 | 0.7/0.7 | -0.5/-0.5 | 0.5/0.5 | -1.2/-1.2 | 1.2/1.2 | -0.8/-0.8 | 0.8/0.8 | -0.6/-0.6 | 0.6/0.6 | -1.2/-1.2 | 1.2/1.2 |

SL-PRS RSRPP error CDF – UE3: RSRPP of path1/RSRPP of path2

| SCS |  |  | 15 kHz |  |  |  | 30 kHz |  |  |  |  |  | 60 kHz |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RB num |  |  | 52 |  | 104 |  | 24 |  | 48 |  | 96 |  | 24 |  |
|  |  |  | 5%-ile | 95%-ile | 5%-ile | 95%-ile | 5%-ile | 95%-ile | 5%-ile | 95%-ile | 5%-ile | 95%-ile | 5%-ile | 95%-ile |
| Two-tap channel | Es/Iot -3,-6,-6 | M: 2 | -1/-1 | 1/1 | -0.7/-0.7 | 0.7/0.7 | -1.6/-1.6 | 1.6/1.6 | -1.2/-1.2 | 1.2/1.2 | -0.8/-0.8 | 0.8/0.8 | -1.6/-1.6 | 1.6/1.6 |
|  |  | M: 4 | -0.7/-0.7 | 0.7/0.7 | -0.5/-0.5 | 0.5/0.5 | -1.2/-1.2 | 1.2/1.2 | -0.8/-0.8 | 0.8/0.8 | -0.6/-0.6 | 0.6/0.6 | -1.2/-1.2 | 1.2/1.2 |
|  | Es/Iot 0,-6,-6 | M: 2 | -1/-1 | 1/1 | -0.7/-0.7 | 0.7/0.7 | -1.6/-1.6 | 1.6/1.6 | -1.2/-1.2 | 1.2/1.2 | -0.8/-0.8 | 0.8/0.8 | -1.8/-1.8 | 1.8/1.8 |
|  |  | M: 4 | -0.7/-0.7 | 0.7/0.7 | -0.5/-0.5 | 0.5/0.5 | -1.2/-1.2 | 1.2/1.2 | -0.8/-0.8 | 0.8/0.8 | -0.6/-0.6 | 0.6/0.6 | -1.2/-1.2 | 1.2/1.2 |

From the simulation assumption the anchor UE3 has a time shift by 3us which can make SL-PRS RSTD performance degradation. The mean of SL-PRS RSTD error is 5888 Tc which is equivalent to about 3us, which is a natural results. And a time shift of 3us can result in a positioning error equivalent to a distance of about 300 meters. It means that SL-PRS RSTD accuracy is sensitive to synchronization among anchor UEs. Therefore, for ease of evaluation, simulation assumption with a time shift of 0us should be considered. Additionally SL positioning should detect the SL-PRS in poorer channel environments, so at least AWGN channels with Es/Iot below -10dB should be considered for further simulation environments to verify SL-PRS detection performance.

Proposal 2: Revise the parameter of synchronization of anchor UEs value from <0,0,3 us> to <0,0,0 us> in the simulation assumption.

Proposal 3: Introduce an additional channel environment of lower than -10 dB of Es/Iot at AWGN for SL-PRS detection performance checking.

Conclusion

In this contribution, we provide our views on SL positioning measurements and procedures related to RRM issues based on the approved WF [1] in RAN4#107 and simulation results based on simulation assumption [2] in RAN4#107.

Proposal 1: The number of samples can be consider after SL-PRS detection performance evaluation.

Proposal 2: Revise the parameter of synchronization of anchor UEs value from <0,0,3 us> to <0,0,0 us> in the simulation assumption.

Proposal 3: Introduce an additional channel environment of lower than -10 dB of Es/Iot at AWGN for SL-PRS detection performance checking.

Reference

[1] R4-2310163, “WF on NR Positioning – SL positioning and CPP,” CATT.

[2] R4-2310077, “Simulation assumptions for sidelink positioning,” CATT.