3GPP TSG-RAN WG4 Meeting # 108	R4-2313045

Toulouse Meeting, August 21st – August 25th, 2023

Title:	TP to 38.718-02-01 to add CA_n71-n85

Source:	Nokia, T-Mobile US

Agenda item:	7.1.1.2

Document for:	Approval

# 1	Introduction

This is a continuation of the effort to add CA_n71A-n85A via TP to TR 38.718-02-01 as first submitted at RAN4#105 in [1] and subsequently at RAN4#106 and RAN4#106bis in [2] and [3].

During the discussion at previous RAN4 meeting several WFs have been agreed, [4] [5] [6].

In the latest WF from RAN4#107 [6] the remaining open issue is the cross band MSD for n85 due to n71 UL which were left TBD. In [8] we made an analysis for the cross-band MSD for n85 due to n71 UL at 20 MHz. In the levelplan shown in Figure 1, we analyse the MSD for n85 due to n71 UL at 35 MHz. The analysis is based on the simulation of PA noise for n71 UL with the simulation input listed in Table 1.

Figure 1: Frequency offset of the UAW channel from the lower edge of the band B3.

Table 1: Levelplan for cross-band MSD

| n85 DL MSD due to n71 UL (35 MHz BW, 20 RB, RBstart=168) |  | n85 Main lower 5MHz | n85 Div. lower 5 MHz |
| --- | --- | --- | --- |
| n71 TX noise in n85 DL, 680.5 MHz, 35 MHz BW, 20 RB, RBstart=168 | [dBm] | -25.98 |  |
| Tx-Antenna duplexer attenuation | [dB] | 45 |  |
| Diplexer attenuation towards victim DL | [dB] | 15 |  |
| Diplexer to antenna IL | [dB] | 2 |  |
| Antenna isolation | [dB] |  | 10 |
| In-channel noise in n85 DL, BW=5MHz, level antenna | [dBm] | -87.98 | -97.98 |
| n85 REFSENS | [dBm] | -94 | -94 |
| n85 de-sense | [dB] | 6.99 | 1.46 |
| In-channel noise level at antenna, MRC | [dBm] | -87.6 |  |
| n85 REFSENS, MRC | [dBm] | -97 |  |
| MSD for n85, MRC | [dB] | 9.9 |  |

According to levelplan calculation, there is an increase of MSD for n85 by approximately 10dB due to ACLR1 impact.

There is an increase of MSD for n85 by approximately 10dB due to ACLR1 impact.

# 2	TP to 38.718-02-01

Based on the previous proposed values for MSD for cross-band isolation issues the TP as first presented in [2] have been revised as the following.

************************************* Start of TP*****************************************

## 6.x		CA_n71-n85

### 6.x.1	Common for 1 band UL and 2 bands UL CA

#### 6.x.1.1 Operating bands for CA

Table 6.X.1.1-1: CA band combination of band n71+n85

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
| n85 | 698 MHz | – | 716 MHz | 728 MHz | – | 746 MHz | FDD |

#### 6.x.1.2	Channel bandwidths per operating band for CA

Table 6.x.1.2-1: Supported bandwidths per CA band combination of band n5+n77

| CA operating/channel bandwidth [MHz] |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n71A-n85A | - | n71 | See n71 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n85 | See n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n71(2A)-n85A | - | n71 | CA_n71(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n85 | See n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n71B-n85A | - | n71 | CA_n71B BCS 4 and 5 | 4 and 5 |
|  |  | n85 | See n85 channel bandwidths in Table 5.3.5-1 |  |

#### 6.x.1.3	Co-existence studies

Table 6.x.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n71-n85.

Table 6.x.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n71 | 663 | 698 | 617 | 652 | 1326 | 1396 | 1989 | 2094 | 2652 | 2792 | 3315 | 3490 |
| n85 | 698 | 716 | 728 | 746 | 1396 | 1432 | 2094 | 2148 | 2792 | 2864 | 3490 | 3580 |

Based on above table, there is no harmonic interference.

Table 6.x.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n71 | 663 | 698 | 617 | 652 | 1234 | 1304 | 1851 | 1956 | 2468 | 2608 | 3085 | 3260 |
| n85 | 698 | 716 | 728 | 746 | 1456 | 1492 | 2184 | 2238 | 2912 | 2984 | 3640 | 3730 |

Based on above table, there is no harmonic mixing issue for CA_n71-n85.

Based on the analysis of cross-band interference provided by multiple companies the cross-band isolation issue of CA_n71A-n85A is address via MSD as defined in Table 6.x.1.3-3.

Table 6.x.1.3-3: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC3 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n85 | 688 | 20 | 15 | 20 (RBstart=86) | 730.5 | 5 | 8.2X | ACLR2 |
|  |  | 680.5 | 35 | 15 | 20 (Rbstart=168) | 730.5 | 5 | [10]Y | ACLR1 |
| n85 | n71 | 705.5 | 15 | 15 | 20 (Rbstart=59) | 649.5 | 5 | 3.8 | >ACLR2 |
| NOTE X: applicable to UE not supporting n71 optional maximum symmetrical UL/DL channel bandwidth NOTE y: applicable to UE supporting n71 optional maximum symmetrical UL/DL channel bandwidth |  |  |  |  |  |  |  |  |  |

#### 6.x.1.4	∆TIB and ∆RIB values

For CA_n71-n85, the TIB,c and RIB values are as agreed in WF.

Table 6.x.1.4-1: ΔTIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n71-n85 | 1 | 1 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 6.x.1.4-2: ΔRIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n71-n85 | 0.8 | 0.8 |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

#### 6.x.1.5	REFSENs requirements

#### There are no specific REFSENS requirements for 1 band UL

#### 6.x.1.6	OOB blocking exception requirements

There is no OOB exception for the CA combination. This since the need of additional OBB exception requirement for the band combinations is only valid for a wideband (e.g. n77/n78/n79) when paired with a low frequency band (< 1 GHz).

Table 6.x.1.6-1: CA band combination with exceptions allowed

CA band combination

No exceptions

************************************* End of TP*****************************************

# 4	References

- R4-2219113, “TP to TR 38.717-02-01: Addition of CA_n71-n85”, Nokia / T-Mobile, R4#105-e

- R4-2300414, “TP to TR 38.717-02-01: Addition of CA_n71-n85”, Nokia / T-Mobile, R4#106, Athens, Greece

- R4-2300414, “TP to TR 38.717-02-01: Addition of CA_n71-n85”, Nokia / T-Mobile, R4#106bis-e

- R4-2220560, WF on 1UL CA_n71-n85 architecture and requirement, T-Mobile USA, R4#105-e

- R4-2302798, “Topic summary for [106][105] NR_Baskets_Part_1”, Skyworks Solutions Inc, R4#106, Athens, Greece

- R4-2306479, WF on CA_n71-n85 and related band 12 and 71 combinations, T-Mobile USA, R4#106bis-e

- R4-2310318, “WF on MSD test point for n71-n85 depending on max BW and legacy/new BW support”, T-Mobile USA, Skyworks Solutions, Inc., R4#107, Incheon, South Korea

- R4-2304957, “Discussion on addition of CA_n71-n85”, Nokia,T-Mobile, R4#106bis-e