3GPP TSG-RAN WG4 Meeting #108                                                               R4-2314297

Toulouse, France, August 21st – 25th, 2023

Agenda item:			8.12.6

Source:	Samsung

Title:	WF on FR2 HST RRM requirements (part 1)

Document for:	Approval

# Introduction

For Rel-18 work item on enhanced NR support for high speed train scenario in frequency range 2, there are two threads  [215] and [216] organized for RRM discussion on RAN4#108, and this document is used to capture the WF on topics in thread [215].

# Topic #1: Simultaneous multi-panel operation for train roof-mounted FR2 high power devices

## Agreements in Monday RRM session

The following agreement was achieved in Monday session:

Sub-topic 1-1: Possible scenarios analysis in Rel-18 FR2 HST - Agreement - Define the RRM requirements for Rel-18 FR2 HST Multi-Rx based on the following scenarios described in R4-2314160 clause 1.2.1: - Scenario A & D: The FR2 PC6 UE uses two active panels pointed forward and backward to receive DL signal transmitted from two RRHs with different QCL TypeD RSs at the same time.

## Sub-topic 1-2: Whether need to define a new or reuse the existing (simultaneousReceptionDiffTypeD-r16) capability for multi-panel simultaneous reception

Issue 1-2-1: Whether need to define a new or reuse the existing (simultaneousReceptionDiffTypeD-r16) capability for multi-panel simultaneous reception

- Agreement:

  - Define a new UE capability [simultaneousReceptionFR2HST-r18] to indicate support of simultaneous multi-panel reception for Rel-18 FR2 PC6 UE

    - Details can be discussed in Rel-18 feature list discussion.

      - The conclusion from Rel-18 Multi-RX WI could be considered.

## Sub-topic 1-3: Whether to define the new scheduling restriction requirements for data+L1 measurement

Issue 1-3-1: Whether to define the new scheduling restriction requirements for data+L1 measurement

- Way Forward:

  - In the maintenance of WI, RAN4 will discuss the feasibility and try to define enhanced scheduling/measurement restriction requirements for data+L1 measurement for Rel-18 FR2 HST multi-panel simultaneous reception

    - FFS, whether/how to capture in the TS that no scheduling/measurement restrictions apply for the case of simultaneous data reception on one panel and SSB reception on the other panel in case of data and SSB transmission from different RRHs

## Sub-topic 1-4: Whether to/How to define the new measurement restriction requirements for simultaneous L1+L1 measurement

Issue 1-4-1: Restriction on SSB configuration for neighboring RRHs

- Agreement:

  - There is no need to consider measurement restriction enhancement on SSB configuration for neighboring RRHs for Rel-18 FR2 HST multi-panel simultaneous reception

## Sub-topic 1-5: Whether to/How to reduce the sweeping factor N1

Issue 1-5-1: Whether to reduce the sweeping factor N1

- Agreement:

  - Introduce enhanced scaling factors in L1 measurement requirements for PC6 UE supporting Rel-18 multi-panel reception and when highSpeedDeploymentTypeFR2-r17 is configured as bidirectional:

    - Baseline:

      - N1 = [1.5] when highSpeedMeasFlagFR2-r17 = set1;

        - One company have the concern on the impact of N1=[1.5], and may further study the feasibility in the maintenance of Rel-18.

      - N1 = [4] when highSpeedMeasFlagFR2-r17 = set2

  - Use the following text proposal for the enhancement for RLM, BFD and L1-RSRP measurement:

    - Note: Scaling factor N=2 when highSpeedMeasFlagFR2-r17 is configured to set1 or scaling factor N=6 when highSpeedMeasFlagFR2-r17 is configured to set2, if UE is not supporting [simultaneousReceptionFR2HST-r18] or when highSpeedDeploymentTypeFR2-r17 is not configured as bidirectional; Scaling factor N=[TBD] when highSpeedMeasFlagFR2-r17 is configured to set1 or scaling factor N=[4] when highSpeedMeasFlagFR2-r17 is configured to set2, if UE is supporting [simultaneousReceptionFR2HST-r18] and when highSpeedDeploymentTypeFR2-r17 is configured as bidirectional.

## Draft CRs for simultaneous multi-panel operation (for Information)

| R4-2312341 revised to R4-2314286 | Intel | CR on requirements for MRTD |  |
| --- | --- | --- | --- |
| R4-2313798 (late contribution） | Qualcomm | DraftCR on RLM for FR2 HST Enh |  |
| R4-2311850 revised to R4-2314285 | Xiaomi | CR on requirements for SSB based BFD |  |
| R4-2312357 revised to R4-2314289 | Ericsson, Nokia, Nokia Shanghai Bell | CR on scheduling availability requirement of UE during RLM for FR2 HST |  |
| R4-2313543¦ revised to R4-2314288 | Samsung | CR on scheduling availability requirement of UE during L1-RSRP measurement for FR2 HST |  |
| R4-2312872 revised to R4-2314287 | Huawei, HiSilicon | CR on measurement requirements for SSB based L1-RSRP |  |

# Topic #2: Intra-band carrier aggregation (CA) scenario

## Agreements in Monday RRM session

The following agreement was achieved in Monday session:

Topic #2: Intra-band carrier aggregation (CA) scenario Issue 2-1-1: Whether need to support separate CA and inter-frequency enhancements for Rel-18 FR2 HST - Agreements - Introduce per-UE capability to indicate whether the UE is capable of supporting the enhanced RRM requirements for Connected and Idle mode inter-frequency measurements for FR2 HST - FFS whether to introduce a cell specific network signalling to inform UE whether to apply the enhanced RRM requirements for Connected and Idle inter-frequency measurement for FR2 HST. - FFS: Do not introduce new network indication or UE capabilities to support CA enhancements for R18 FR2 HST

## Sub-topic 2-1: Inter-frequency measurement requirements

Issue 2-1-1: Whether need to support separate CA and inter-frequency enhancements for Rel-18 FR2 HST (UE capability)

- Agreement:

  - A single per-UE capability is introduced for FR2 HST CA and inter-frequency measurement enhancement:

    - [measurementEnhancementCAInterFreqFR2-r18] indicates whether the FR2 PC6 UE supports the enhanced RRM requirements for carrier aggregation as specified in TS 38.133 and the UE supports the enhanced RRM requirements for inter-frequency measurements in connected and idle mode as specified in TS 38.133.

Issue 2-1-1: Whether need to support separate CA and inter-frequency enhancements for Rel-18 FR2 HST (network flag signaling)

The following agreement was achieved in Tuesday ad-hoc session:

- Agreement:

  - Reuse Rel-17 signaling highSpeedMeasFlagFR2 to inform UE whether to apply the enhanced RRM requirements for inter-frequency measurement for FR2 HST in Connected mode.

  - Reuse Rel-17 signaling highSpeedMeasFlagFR2 to inform UE whether to apply the enhanced RRM requirements for intra-frequency measurement on SCC for FR2 HST in Connected mode.

Issue 2-1-2: Whether need to introduce signalling for idle mode inter-frequency measurement requirements

- Agreement:

  - Reuse Rel-17 IE highSpeedMeasFlagFR2-r17 in SIB to inform UE whether to apply the enhanced RRM requirements for inter-frequency measurement for FR2 HST in idle mode.

## Sub-topic 2-2: Intra-frequency measurement requirements on SCC

- Agreement:

  - For UE supporting power class 6 with [highSpeedMeasFlagFR2] configured, the TSSB_measurement_period_intra specified for UE supporting [measurementEnhancementCAInterFreqFR2-r18] shall apply for SCC.

    - Specification change is required, because new capability [measurementEnhancementCAInterFreqFR2-r18] is introduced.

## Sub-topic 2-3: The RRM impact for SCell activation delay requirement for Rel-18 FR2 HST UE

- Agreement:

  - SCell activation delay with 3ms Tactivation_time can be applied without having to fulfil legacy conditions in HST FR2 scenario, and the below text proposal is used in CR:

If the SCell being activated belongs to FR2 and if there is at least one active serving cell on that FR2 band, if the FR2 power class 6 UE is configured with highSpeedMeasFlagFR2-r17, Tactivation_time is 3 ms, provided - the RS (s) of SCell being activated is (are) QCL-TypeD with RS (s) of one active serving cell on that FR2 band. Editor Notes: FFS additional condition/capability is needed.

## Draft CRs for Intra-band CA (for Information)

| R4-2312637 revised to R4-2314290 | Huawei, HiSilicon | CR on intra-frequency requirements enhancement in FR2 HST |  |
| --- | --- | --- | --- |
| R4-2312191 revised to R4-2314289 | Nokia, Nokia Shanghai Bell | CR on inter-frequency requirements enhancement in FR2 HST |  |
| R4-2313544 revised to R4-2314291 | Samsung | CR on inter-frequency measurement requirements in Idle mode for FR2 HST |  |
| R4-2312358 revised to R4-2314292 | Ericsson | CR on SCell activation delay for FR2 HST |  |