3GPP TSG-RAN WG4 Meeting # 108	R4-2313268

# Toulouse, France, August 21 – August 25, 2023

Title: 	Test parameters and simulation results for advanced receiver for MU-MIMO

Source: 	Huawei, HiSilicon

Agenda item:	8.18.1.2

Document for:	Discussion

Background

As per [1], WF for advanced receiver for MU-MIMO was agreed and all simulation cases for phase I study are determined. In this contribution, we provide our simulation results, corresponding observations and suggestions on test setup.

Discussions

The performance for blind DMRS port detection and FDRA detection can be guaranteed based on the study experience of CRS-IM topic with acceptable implementation complexity, furthermore, it seems impossible to introduce related signalling, the necessity to perform such study is low from our side.

Simulation assumptions are listed in Table 2-1:

Table 2-1: Simulation assumptions

| Parameter |  | Unit | Value |  |
| --- | --- | --- | --- | --- |
|  |  |  | Target UE | Co-scheduled UE |
| Channel Bandwidth/SCS |  | MHz/KHz | 10/15 |  |
| Duplex mode |  |  | FDD |  |
| Allocation for interference UE and target UE | Rank allocation |  | 1 | 1 |
|  |  |  | 2 | 2 |
|  | Scrambling ID |  | Same scrambling ID for both UEs |  |
| MIMO and antenna configuration |  |  | For Rank 1+1: 2T2R Medium For Rank 2+2: 4T4R Low |  |
| Number of CDM groups without data |  |  | 1 for paired UE allocated in same CDM groups and 2 for paired UE allocated in different CDM groups |  |
| HARQ process number |  |  | 4 |  |
| PDSCH configuration | Mapping type |  | Type A |  |
|  | Starting symbol (S) |  | 2 |  |
|  | Length (L) |  | 12 |  |
|  | PRB bundling size |  | 2 |  |
|  | PRB bundling type |  | Static |  |
| PDSCH DMRS configuration | DMRS Type |  | DMRS Type 1 |  |
|  | Number of additional DMRS |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |  |
| Test metric |  |  | SNR @ %70 of maximum Throughput | N/A |

Cases with genie knowledge of required information

Figure 2-1 shows simulation results for genie aided knowledge of required information

Figure 2-1: Simulation results for genie aided knowledge of required information

Table 2-1 shows simulation summary for genie aided knowledge of required information

Table 2-1: Summary of simulation results for genie aided knowledge of required information

| Rank allocation | Propagation Conditions | PMI selection | MCS for Serving UE | Interference UE | MMSE-IRC | E-MMSE-IRC | R-ML | E-MMSE-IRC(Gain) | R-ML(Gain) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rank 1+1 | TDLC300-100 Medium | Random | 4 | QPSK | 5.6 | 5.3 | 4.2 | 0.3 | 1.1 |
|  |  |  | 13 | QPSK | 21.9 | 20.8 | 13.7 | 1.1 | 8.2 |
| Rank 2+2 | TDLA30-10 Low | Orthogonal | 13 | QPSK | 12.1 | 12.1 | 9.3 | 0.0 | 2.8 |
|  |  |  |  | 16QAM | 12.1 | 12.0 | 11.3 | 0.1 | 0.8 |
|  |  |  |  | 64QAM | 12.1 | 12.1 | 11.7 | 0.0 | 0.4 |
|  |  |  | 17 | QPSK | 16.4 | 16.4 | 13.1 | 0.0 | 3.3 |
|  |  |  |  | 16QAM | 16.5 | 16.4 | 15.1 | 0.1 | 1.4 |
|  |  |  |  | 64QAM | 16.5 | 16.4 | 16.2 | 0.1 | 0.2 |
|  | TDLC300-100 Low | Random | 13 | QPSK | 22.3 | 18.3 | 13.7 | 4.0 | 8.6 |
|  |  |  |  | 64QAM | 22.3 | 18.3 | 17.4 | 4.0 | 4.9 |
|  |  | Orthogonal |  | QPSK | 14.5 | 13.3 | 10.7 | 1.2 | 3.8 |
|  |  |  |  | 64QAM | 14.5 | 13.3 | 12.9 | 1.2 | 1.6 |

The common observations should be captured in the TR:

Observation 1: High fading selective propagation conditions contributes most to the performance gain of E-MMSE-IRC

Observation 2:

- The modulation of co-scheduled UE is lower, the performance gain of R-ML is larger.

- Modulation order has no impact on MMSE-IRC and E-MMSE-IRC receiver.

- Under the condition of genie aided knowledge of required information, R-ML has larger performance gain than E-MMSE-IRC in all above cases.

Observation 3: Better PMI implementation reduces the performance gain of advanced receiver

Cases with modulation order estimation

Figure 2-2 shows the performance of R-ML receiver with modulation order detection. For case of Rank2+2, serving UE assumes two layers have same modulation order in each PRB.

Figure 2-2: Simulation results for cases of modulation order estimation

Table 2-2 shows simulation summary for modulation order detection.

Table 2-2: Summary of simulation results for case with modulation order estimation

| Rank allocation | Propagation Conditions | PMI selection | MCS for Serving UE | Interference UE | E-MMSE-IRC | R-ML (With Modulation order detection) | R-ML (Ideal) | Performance loss for modulation order estimation |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rank 1+1 | TDLC300-100 Medium | Random | 13 | QPSK | 20.8 | 13.3 | 13.3 | 0.0 |
|  |  |  |  | 16QAM | TBD | 17.1 | 16.9 | 0.2 |
|  |  |  |  | 64QAM | TBD | 19.6 | 18.9 | 0.7 |
| Rank 2+2 | TDLA30-10 Low | Orthogonal | 13 | QPSK | 12.1 | 10.7 | 9.3 | 0.6 |
|  |  |  |  | 16QAM | 12.0 | 11.4 | 11.4 | 0.0 |
|  |  |  |  | 64QAM | 12.1 | 12.0 | 11.7 | 0.3 |
|  |  |  | 17 | QPSK | 16.4 | 14.0 | 13.1 | 0.9 |
|  |  |  |  | 16QAM | 16.4 | 16.0 | 15.1 | 0.9 |
|  |  |  |  | 64QAM | 16.4 | 16.2 | 16.2 | 0.0 |

We have following observation for performance of R-ML receiver with modulation order estimation.

Observation 4: The performance loss due to the modulation order detection is about 0~0.9dB.

Observation 5: Performance gain of R-ML with modulation order estimation compared to E-MMSE-IRC for Rank 2+2 is quite low for most cases. For case of Rank1+1, the performance gain is quite high (7.5dB for MCS13 (Target UE)+QPSK (Co-scheduled UE)).

It is proposed to introduce two optional UE features based on UE’s declaration, one is supporting R-ML receiver on MU-MIMO scenario with signalling on modulation order of co-scheduled UEs, the other is supporting R-ML receiver on MU-MIMO scenario without signalling on modulation order of co-scheduled UEs (modulation order detection), same time, RAN4 can introduce two type of cases to verify the capabilities of supporting such two cases. Based on observation 2, we suggest to configure QPSK for co-scheduled UE to maximize the performance gain of R-ML and consider both Rank 1+1 and Rank 2+2 for case without modulation order detection. For case with modulation order detection, our concern is that it’s very difficult to align the results from companies for Rank 2+2 due to diverse modulation order detection methods and based on observation 5, case of Rank 1+1 has more performance gain, hence it is more feasible to only consider Rank 1+1.

Proposal 1: RAN4 to consider following test setup in phase II if R-ML receiver is agreed as baseline in phase I

- Define two optional features based on UE’s declaration: supporting R-ML receiver on MU-MIMO scenario with signalling on modulation order of co-scheduled UEs and supporting R-ML receiver on MU-MIMO scenario without signalling on modulation order of co-scheduled UEs (modulation order detection)

- For case without modulation order detection:

  - Rank 1+1, TDLC300-100 medium

  - Rank 2+2, TDLA30-10 Low

  - QPSK configured for co-scheduled UE

- For case with modulation order detection:

  - Rank 1+1, TDLC300-100 medium

  - QPSK configured for co-scheduled UE

Conclusion

This contribution provides the simulation results, corresponding observations and suggestions on test setup for advanced receiver on MU-MIMO scenario. The observations are:

Observation 1: High fading selective propagation conditions contributes most to the performance gain of E-MMSE-IRC

Observation 2:

- The modulation of co-scheduled UE is lower, the performance gain of R-ML is larger.

- Modulation order has no impact on MMSE-IRC and E-MMSE-IRC receiver.

- Under the condition of genie aided knowledge of required information, R-ML has larger performance gain than E-MMSE-IRC in all above cases.

Observation 3: Better PMI implementation reduces the performance gain of advanced receiver

Observation 4: The performance loss due to the modulation order detection is about 0~0.9dB.

Observation 5: Performance gain of R-ML with modulation order estimation compared to E-MMSE-IRC for Rank 2+2 is quite low for most cases. For case of Rank1+1, the performance gain is quite high (7.5dB for MCS13 (Target UE)+QPSK (Co-scheduled UE)).

Proposal 1: RAN4 to consider following test setup in phase II if R-ML receiver is agreed as baseline in phase I

- Define two optional features based on UE’s declaration: supporting R-ML receiver on MU-MIMO scenario with signalling on modulation order of co-scheduled UEs and supporting R-ML receiver on MU-MIMO scenario without signalling on modulation order of co-scheduled UEs (modulation order detection)

- For case without modulation order detection:

  - Rank 1+1, TDLC300-100 medium

  - Rank 2+2, TDLA30-10 Low

  - QPSK configured for co-scheduled UE

- For case with modulation order detection:

  - Rank 1+1, TDLC300-100 medium

  - QPSK configured for co-scheduled UE

Reference

- R4-2309892 WF on advanced receiver for MU-MIMO scenario