3GPP TSG-RAN WG4 Meeting #108-bis	R4-2315012

Xiamen, China, 09 – 13 October 2023

3GPP TSG RAN WG1 #114	R1-2308625

Toulouse, France, Aug 21st – 25th, 2023

# Title:	LS on L1 measurement and TA management for LTM

# Response to:

# Release:	Rel-18

# Work Item:	NR_mob_enh2-Core

Source:	RAN1

To:	RAN2, RAN3, RAN4

Cc:

Contact Person:

Name:	Xin Su

E-mail Address:	suxin@catt.cn

Name:	Yosuke Akimoto

E-mail Address:	akimoto.yosuke@fujitsu.com

Name:	Parisa Cheraghi

E-mail Address:	Parisa.Cheraghi@mediatek.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

# 1. Overall Description:

The following agreements and conclusions have been made in RAN1 #114 meeting.

## 1.1 L1 enhancements for inter-cell beam management

Agreement

Confirm the following working assumption achieved in RAN-112bis-e

Working Assumption

- On the presence of beam indication within cell switch command, at least for scenario 2, following is supported:

- A field to indicate 1 joint or 1 pair of UL and DL unified TCI State index for the target cell field is always present in the cell switch command.

- Note: If scenarios 1 and 3 are agreed to be supported in R18 LTM other solutions may be considered.

Agreement

On top the confirmed working assumption, on the presence of beam indication within cell switch command, at least for scenario 2 following is supported:

- A field to indicate 1 joint or 1 pair of UL and DL unified TCI State index for the target cell field is always present in the cell switch command.

- FFS UE behaviour for the beam indication field for the RACH-based handover scenario after cell switch command

Conclusion

In R18 LTM, there is no consensus to support triggering of aperiodic SRS transmission to the target cell in the cell switch command.

Agreement

In R18 LTM, on the QCL source of the TCI state before/during the cell switch command,

- SSB or TRS can be configured in a TCI state for the candidate cell(s) before/during cell switch command

  - Whether the TRS can be used for the candidate cell(s) before/during cell switch command is up to UE capability

Agreement

- In Rel-18 LTM, only CD-SSB is supported for L1 intra- and inter-frequency measurement

Agreement

- For the beam selection for SSB based L1-RSRP measurement report,

  - For the value of M, L

    - the RRC configured candidate values are:

      - M = 1, 2, 3, 4

      - L = 1, 2, 3, 4

    - Note: the maximum value of M*L and combination of M and L is up to UE capability

- Note: the common understanding is that L=1 with configuration of inclusion of serving cell is not a typical case.

- No need to confirm the corresponding working assumption (made in RAN1#113).

Agreement

- TCI state activation by MAC CE before cell switch command for one or more than one candidate cells is allowed

Agreement

- Absolute value and differential values are used for L1-RSRP reporting:

  - For absolute L1-RSRP, the L1-RSRP value is quantized to a 7-bit value in the range [-140, -44] dBm with 1dB step size

  - For differential L1-RSRP, the L1-RSRP value is quantized to a 4-bit value where the differential L1-RSRP value is computed with 2 dB step size from reference L1-RSRP value

- Agreement

- SSBRI among configured candidate cells is included for each L1-RSRP report

  - The bit size of SSBRI is where  is the number of configured SSBs in the corresponding resource set for the report

  - The following format is used for reporting

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n | SSBRI #1 as in Table 6.3.1.1.2-6, if reported |
|  | SSBRI #2 as in Table 6.3.1.1.2-6, if reported |
|  | : |
|  | SSBRI #L*M as in Table 6.3.1.1.2-6, if reported |
|  | RSRP #1 as in Table 6.3.1.1.2-6, if reported |
|  | Differential RSRP #2 as in Table 6.3.1.1.2-6, if reported |
|  | : |
|  | Differential RSRP #L*M as in Table 6.3.1.1.2-6, if reported |

## 1.2 Timing advance management to reduce latency

Agreement

For the power control of PDCCH-ordered CFRA in LTM, the UE can maintain only one power ramping counter

Agreement

For the power control of PDCCH-ordered CFRA in LTM, power-ramping counter is reset at least when UE receives a PDCCH order indicating the initial transmission of PRACH

Agreement

when a PDCCH order is sent for a candidate cell,

- The bit size of N in DCI format 1_0 for cell indicator is determined by the number (e.g., C) of configured candidate cells with RACH configuration provided for early TA acquisition, downselect one from the following alternatives.

    - Alt 1: N=

    - Alt 2: N= (update the equation with (C+1))

        - The number of cells used to calculate the bit width is the number of candidate cells with RACH configuration provided for early TA acquisition + 1 (serving cell)

Agreement

For the power control of PDCCH-ordered CFRA in LTM,

- When a UE receives a PDCCH order indicating a re-transmission of PRACH with the same associated SSB and same candidate cell as the previous PRACH, the counter is increased by 1.

- In addition to case 1, power-ramping counter is reset in the following cases:

  - Case 2: The candidate cell indicated in the PDCCH order, indicating retransmission, is different from that indicated in the last PDCCH order.

Note: the initial counter is 0 before receiving any PDCCH order.

Agreement

when a PDCCH order is sent for a candidate cell,

- The bit size of N in DCI format 1_0 for cell indicator is determined by the number (e.g., C) of configured candidate cells with RACH configuration provided for early TA acquisition, the following alternative is supported:

    - Alt 2: N=

- The number of cells used to calculate the bit width is the number of candidate cells with RACH configuration provided for early TA acquisition + 1 (serving cell)

Agreement

When the UE does not support simultaneous/parallel transmissions of PRACH in candidate cell and UL channels and signals in serving cell, support

  - serving cell UL TX is dropped.

Agreement

If the UE supports simultaneous/parallel transmissions of PRACH in candidate cell and UL channels and signals in serving cell in the same frequency range, support:

- A PRACH transmission to a LTM candidate cell has the highest priority for power allocation

Note: up to UE whether performs power scale-down or drop of UL transmission with lower priority when UL transmission power is insufficient.

2. Actions:

To RAN2 group.

ACTION: 	RAN1 respectfully asks RAN2 to take the RAN1 agreements into consideration for their work.

To RAN3 group.

ACTION: 	RAN1 respectfully asks RAN3 to take the RAN1 agreements into consideration for their work.

To RAN4 group.

ACTION: 	RAN1 respectfully asks RAN4 to take the RAN1 agreements into consideration for their work.

3. Date of Next TSG RAN WG1 Meetings:

TSG RAN WG1 Meeting #114-bis	9th Oct – 13th Oct 2023	Xiamen, CN

TSG RAN WG1 Meeting #115	13th Oct – 17th Oct 2023	Chicago, US