3GPP TSG-RAN WG4 Meeting #108bis	R4-2315042

Xiamen, China, October 09 – October 13, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Supporting simulations for 4Tx Demodulation

Agenda item:	5.3.3.2

Document for:	Discussion

Introduction

In this contribution we provide simulation results for 4Tx BS demodulation, based upon the parameters agreed at previous RAN4 meetings.

Discussion

This paper presents the results of simulations which have been conducted by Nokia based on the parameters defined for 4Tx Simulations at RAN4 #107 and distributed in the WF for 4Tx [2]. These parameters can be seen summarised in Table 1.

The simulation results are revisited based on the simulations presented at RAN4#108, and no new additional simulations have been added, but can be utilised to define performance requirements.

Table 1 : Extended Simulation Parameters for 4Tx following RAN4# 106

| Parameter |  | Value |  |
| --- | --- | --- | --- |
| Transform precoding |  | Disabled |  |
| Default TDD UL-DL pattern (Note 1) |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |  |
| HARQ | Maximum number of HARQ transmissions | 4 |  |
|  | RV sequence | 0, 2, 3, 1 |  |
| DM-RS | DM-RS configuration type | 1 |  |
|  | DM-RS duration | single-symbol DM-RS |  |
|  | Additional DM-RS position | pos1 |  |
|  | Number of DM-RS CDM group(s) without data | 2 |  |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3dB |  |
|  | DM-RS port | {0, 1, 2, 3} |  |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |  |
| Time domain | PUSCH mapping type | A and B |  |
| resource | Start symbol | 0 |  |
| assignment | Allocation length | 14 |  |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |  |
| assignment | Frequency hopping | Disabled |  |
| TPMI index for 4Tx four-layer spatial multiplexing transmission |  | 0 |  |
| Code block group based PUSCH transmission |  | Disabled |  |
| Antenna Configuration |  | 4T4R and 4T8R |  |
| Bandwidth and SCS |  | BW | SCS |
|  |  | 5 MHz | 15 kHz |
|  |  | 20 MHz (FFS) |  |
|  |  | 50 MHz |  |
|  |  | 10 MHz | 30 kHz |
|  |  | 40 MHz (FFS) |  |
|  |  | 100 MHz |  |
| Propagation Conditions and MCS |  | Propagation | MCS (Table 1) |
|  |  | TDLA 30-10 | 17 |
|  |  | TDLB 100-400 | 2 |
|  |  | TDLC 300-100 | 12 |
| NOTE 1: The same requirements are applicable to FDD and TDD with different UL-DL pattern. |  |  |  |

All the values which will be presented in the following tables represent the SNR value in dB at which 70% of the maximum throughput could be reached.

Results will be presented broken down by antenna configuration.

Results for 4T4R Antenna Configuration

Table 2 presents the results for 4T4R, the results are presented for both PUSCH mapping types, all propagation conditions and all results are using TPMI 0 and ‘low’ correlation.

Table 2 :4Tx Results for 4T4R (Rank 4)

| Parameter |  |  |  | SNR Value (@ 70% TPUT) ideal |  |
| --- | --- | --- | --- | --- | --- |
| SCS | BW | MCS | Propagation | PUSCH Mapping Type A | PUSCH Mapping Type B |
|  |  | 2 | TDLB100-400 | -1.23 | -1.16 |
|  | 5MHz | 12 | TDLC300-100 | 11.88 | 11.98 |
|  |  | 17 | TDLA30-10 | 16.23 | 16.22 |
|  |  | 2 | TDLB100-400 | -0.94 | -0.88 |
| 15kHz | 20MHz | 12 | TDLC300-100 | 12.28 | 12.26 |
|  |  | 17 | TDLA30-10 | 17.29 | 17.26 |
|  |  | 2 | TDLB100-400 | -0.87 | -0.73 |
|  | 50MHz | 12 | TDLC300-100 | 12.36 | 12.34 |
|  |  | 17 | TDLA30-10 | 18.48 | 18.50 |
|  |  | 2 | TDLB100-400 | -1.42 | -1.39 |
|  | 10MHz | 12 | TDLC300-100 | 12.34 | 12.48 |
|  |  | 17 | TDLA30-10 | 16.44 | 16.43 |
|  |  | 2 | TDLB100-400 | -1.13 | -1.17 |
| 30kHz | 40MHz | 12 | TDLC300-100 | 12.82 | 12.87 |
|  |  | 17 | TDLA30-10 | 17.11 | 17.10 |
|  |  | 2 | TDLB100-400 | -0.99 | -0.99 |
|  | 100MHz | 12 | TDLC300-100 | 13.36 | 13.42 |
|  |  | 17 | TDLA30-10 | 18.11 | 18.19 |

Results for 4T8R Antenna Configuration

Table 3 presents the results for 4T8R, the results are presented for both PUSCH mapping types, all propagation conditions and all results are using TPMI 0 and ‘low’ correlation.

Table 3 : 4Tx Results for 4T8R (Rank 4)

| Parameter |  |  |  | SNR Value (@ 70% TPUT) ideal |  |
| --- | --- | --- | --- | --- | --- |
| SCS | BW | MCS | Propagation | PUSCH Mapping Type A | PUSCH Mapping Type B |
|  |  | 2 | TDBLB100-400 | -4.55 | -4.47 |
|  | 5MHz | 12 | TDLC300-100 | 5.12 | 5.18 |
|  |  | 17 | TDLA30-10 | 9.01 | 9.03 |
| 15 KHz | 20 MHz | 17 | TDLA30-10 | 9.42 | 9.43 |
|  |  | 2 | TDBLB100-400 | -4.16 | -4.05 |
|  | 50MHz | 12 | TDLC300-100 | 5.39 | 5.40 |
|  |  | 17 | TDLA30-10 | 9.83 | 9.90 |
|  |  | 2 | TDBLB100-400 | -4.63 | -4.55 |
|  | 10MHz | 12 | TDLC300-100 | 5.62 | 5.74 |
|  |  | 17 | TDLA30-10 | 9.18 | 9.23 |
| 30kHz | 40MHz | 17 | TDLA30-10 | 9.30 | 9.33 |
|  |  | 2 | TDBLB100-400 | -4.10 | -4.04 |
|  | 100MHz | 12 | TDLC300-100 | 6.28 | 6.38 |
|  |  | 17 | TDLA30-10 | 9.83 | 9.91 |

Conclusion

In this contribution, simulation results were provided on the 4Tx parameters as agreed in [2], and down selected in [4].

- References

- RP-221496 Revised WID Further RF requirements enhancement for NR and EN-DC in frequency range 1 (FR1)

- R4-2305890 WF on FR1 4Tx demodulation requirement, Ericsson, RAN4 #106bis-e. Electronic Meeting, April 17 – 26, 2023

- R4-2311079, Discussion of 4Tx Demodulation Requirements, RAN4 #108, Toulouse, France, Aug 21 – Aug 25, 2023

- R4-2309798, WF on FR1 4Tx demodulation requirement, Ericsson, RAN4 #107. Incheon, Korea, May 22 – 26, 2023