3GPP TSG-RAN WG4 Meeting #108bis	R4-2315080

Xiamen, China, October 09 – October 13, 2023

Source:	China Telecom

Title:	Discussion on general aspects for PDSCH requirements for UE with 8Rx

Agenda Item:	5.3.3.1.1

Document for:	Discussion

- Introduction

In RAN4 #108 meeting, the 8Rx demodulation are discussed and the WF is approved in [1]. In this paper, we give our views on the remaining open issues.

- Discussion

Issue 1-3: Test applicability rules for CA demodulation requirements

Status in the WF in [1]:

- Option 1: For FR1 8Rx CA applicability rules of demodulation performance requirements, extend Table 5.1.1.7.2-1 to include tests cases in new Clause 5.2A.4.1.

- Other options not precluded

In general, we support to extend the existing 2Rx/4Rx CA test applicability rules to 8Rx CA tests. At the same time, we notice that the test configuration including channel bandwidth, duplex model coverage are still not decided in [1], therefore, we propose to make decision on the 8Rx CA test applicability rules together with or after the discussion on the test parameters.

Proposal 1: Make decision on the 8Rx CA test applicability rules together with or after the discussion on the test parameters.

Issue 1-4: Applicability rules for different number of RX antenna ports for CA demodulation requirements

- Option 1:

  - Within the CA configuration if any of the PCell and/or the SCells is a 4Rx supported RF band, 4 out of the 8Rx should be connected with data source from system simulator, depending on UE’s declaration and AP configuration. Requirements from Clause 5.2A.3.1 are applied.

  - Within the CA configuration if any of the PCell and/or the SCells is a 2Rx supported RF band, 2 out of the 8Rx should be connected with data source from system simulator, depending on UE’s declaration and AP configuration. Requirements from Clause 5.2A.2.1 are applied.

  - Within the CA configuration if any of the PCell and/or the SCells is a 8Rx supported RF band, all 8Rx should be connected with data source from system simulator. Requirements from Clause5.2A.4.1 are applied.

  - For 8Rx capable UEs, the 2Rx supported RF bands, 4Rx supported RF bands and 8Rx supported RF bands are up to UE’s declaration.

- Other options not precluded

Issue 1-5: Applicability rules for different number of RX antenna ports for CA CSI requirements

- Option 1: For FR1 8Rx CA applicability rules and antenna connection of CSI reporting requirements, below requirements should be specified.

  - All the requirements specified in clause 6.2A for CA with 4 RX are applied for 8 RX capable UEs by connecting all 8 RX with data source from system simulator and reducing the signal power density by 3 dB compared to the signal power density for 4 RX in the test configurations.

  - All the requirements specified in clause 6.2A for CA with 2 RX are applied for 8 RX capable UEs by connecting all 8 RX with data source from system simulator and reducing the signal power density by 6 dB compared to the signal power density for 2 RX in the test configurations.

- Other options not precluded

As for the applicability rules for 8Rx CA PDSCH and CSI tests, we support the previously proposed option1s as a reasonable extension from 2/4Rx to 8Rx.

Proposal 2: Support the previously proposed option1s as a reasonable extension from 2/4Rx to 8Rx.

- Conclusion

Proposal 1: Make decision on the 8Rx CA test applicability rules together with or after the discussion on the test parameters.

Proposal 2: Support the previously proposed option1s as a reasonable extension from 2/4Rx to 8Rx.

- References

- [1] R4-2313877, WF for 8Rx UE performance requirements, Huawei, HiSilicon, RAN4 #108, Aug, 2023