3GPP TSG-RAN WG4 Meeting # 108bis	R4-2315087

# Xiamen, China, October 09 – October 13, 2023

Source:	LG Electronics

Title:	Discussion on sidelink relay RRM core requirements

Agenda item:	5.32.1

Document for:	Discussion

Introduction

In this contribution, we provided our views on NR sidelink relay RRM core requirements.

Discussion

In the RAN4#108 meeting, following agreements were made.

- Issue 1-1: Interruption time in multipath scenario - Agreements - Clarify interruption requirements caused by the remote UE on its serving cell in TS 38.133 - Issue 1-2: Selection/reselection of relay UE in UE-to-UE relay scenario - Agreements - Applicability of requirements on selection/reselection of relay UE by the remote UE in U2U relay scenario should be explicitly captured in clause 12.10 of TS 38.133.

Regarding interruption time in multipath scenario, the interruptions at NR sidelink discovery configuration in multi-path relay scenario can be necessary to capture. But the interruptions to WAN at transitions between active and non-active during SL-DRX in multi-path relay scenario are not necessary. Because current contexts are described generally, so already cover every scenarios. If RAN4 should capture the interruptions to WAN at transitions between active and non-active during SL-DRX in multi-path relay scenario, RAN4 also should consider the interruptions to PCell/serving cell on the indirect path caused by a relay UE due to transitions between active and non-active times during SL-DRX in multi-path relay scenario as well. It can make the specification complicate unnecessarily. So, we propose that RAN4 to capture “the interruptions at NR sidelink discovery configuration in multi-path relay scenario” only, regarding interruption time in multipath scenario.

Proposal 1: Regarding interruption time, RAN4 to capture the interruptions at NR sidelink discovery configuration in multi-path relay scenario. And not to capture the interruptions to WAN at transitions between active and non-active during SL-DRX in multi-path relay scenario.

Conclusion

In this contribution, we provide our views on the RRM core requirements on NR sidelink relay.

Proposal 1: Regarding interruption time, RAN4 to capture the interruptions at NR sidelink discovery configuration in multi-path relay scenario. And not to capture the interruptions to WAN at transitions between active and non-active during SL-DRX in multi-path relay scenario.

Reference

[1] R4-2314352, “WF on SL Relay RRM Requirements”, LG Electronics

[2] R4-2313522, “Draft CR on requirements for discovery and reselection for UE-to-UE relay”, Ericsson