3GPP TSG-RAN WG4 Meeting # 108bis	R4-2315089

# Xiamen, China, October 09 – October 13, 2023

Source:	LG Electronics

Title:	Discussion on work scope and test cases for SL evolution demodulation performance

Agenda item:	5.30.5

Document for:	Discussion

Introduction

In this contribution, we provide our views on work scope and test cases for demodulation performance of SL evolution based on the WID [1].

Discussion

According to the WID [1], the objectives related to demodulation performance of this WI are

- Specify mechanism to support NR sidelink CA operation based on LTE sidelink CA operation [RAN2, RAN1, RAN4] (This part of the work is put on hold until further checking in RAN#99) - … - Study and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only [RAN1, RAN2, RAN4] - …

This meeting is first meeting for demodulation performance of SL evolution, so we should decide the work scope and test cases. According to the WID[1], Rel-18 SL has objectives to specify to support NR sidelink CA operation and sidelink on unlicensed spectrum. So, RAN4 should include the NR sidelink CA operation and unlicensed spectrum as work scope.

Proposal 1: RAN4 to consider the NR sidelink CA operation and sidelink on unlicensed spectrum as work scope for demodulation performance.

Regarding the sidelink on unlicensed spectrum, the physical channel design framework is updated in Rel-18. “The interlacing RB mapping” and “two candidate starting point in slot” are the updated features and should be evaluated. So, we propose that RAN4 need to evaluate the SL-U demodulation performance with the interlacing RB mapping and the two candidate starting point in slot.

Proposal 2: RAN4 need to evaluate the SL-U demodulation performance with the interlacing RB mapping and the two candidate starting point in slot.

Regarding test cases, we think the existing SL test cases and NR-U test parameters can be used as reference. So, the existing SL common test parameters can be reused. And at least PSSCH and PSFCH demodulation performance requirements are necessary for SL-U. But regarding other physical channels, RAN4 needs to disscuss whether the demodulation performance is necessary.

Proposal 3: SL-U demodulation performance can reuse the existing SL test parameters as much as possible considering interlacing RB mapping and two candidate starting point.

- For transmission model, reuse the existing NR-U transmission model as specified in TS38.101-4 B.5 as much as possible.

Proposal 4: It is necessary to define PSSCH and PSFCH demodulation performance requirements for SL-U.

The draft version of test parameters and cases can be like this.

Table 11.1.1.2-1: Common test parameters

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| Carrier configuration |  | Offset between Point A and the lowest usable subcarrier on this carrier (Note 1) | RBs | 0 |
|  |  | Subcarrier spacing | kHz | 30 |
| SL BWP configuration #1 |  | Cyclic prefix |  | Normal |
|  |  | RB offset | RBs | 0 |
|  |  | Number of contiguous PRB | PRBs | Maximum transmission bandwidth configuration as specified in clause 5.3.2 of TS 38.101-1 [6] for tested channel bandwidth and subcarrier spacing |
| PT-RS configuration |  |  |  | PT-RS is not configured |
| Resource pool configuration | PSCCH Time resource |  | Symbols | 2 |
|  | PSCCH Frequency resource |  | PRBs | 10 |
|  | PSFCH number of cyclic shift pairs |  |  | n1 |
|  | PSFCH hopping ID |  |  | 0 |
|  | PSFCH candidate resource type |  |  | allocSubCH |
|  | Set of PRBs for PSFCH transmission |  |  | ones(1,100) for 40 MHz and ones(1,50) for 20 MHz |
|  | PSSCH RSRP threshold |  |  | 66 (infinity dBm) |
|  | Synchronization reference |  |  | GNSS |
|  | Subchannel size |  | PRBs | 10 |
|  | Number of sub-channels |  |  | 5 for 20 MHz and 10 for 40 MHz |
|  | Start PRB for first sub-channel |  |  | 0 |
|  | Time resource bitmap |  |  | ones(1, 160) |
| Note 1: Point A coincides with minimum guard band as specified in Table 5.3.3-1 from TS 38.101-1 [6] for tested channel bandwidth and subcarrier spacing. |  |  |  |  |

For PSSCH test case, the NR-U test scenario can be referred.

Table 11.1.2.1.X-1: Test parameters

| Parameter |  | Unit | Value |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 |
| Active cell(s) |  |  | None |  |  |
| SL transmission model |  |  | As specified in B.X |  |  |
| SL model parameters | SL transmission duration values | Slots | {2,4,6,7} |  |  |
|  | Occupied OFDM symbols in slot other than the first slot of the transmission duration | Symbols | 12 (Except AGC, TxRx switching symbols) |  |  |
|  | Occupied OFDM symbols in the first slot of the transmission duration | Symbols | {6,7, …,12} |  |  |
|  | SL transmission period | ms | 5 |  |  |
|  | LBT failure probability (pLBT) |  | 0.25 |  |  |
| Sidelink UE 1 | Sidelink transmissions |  | PSCCH + PSSCH |  |  |
|  | PSSCH DMRS pattern (Note 1) |  | {3,4} |  |  |
|  | Index of sub-channel allocation |  | [0,1] | {2,3} | {2,2} |
|  | Timing offset (Note 2) | s | CP/2-12*64*Tc | [0,1] | [0] |
|  | Frequency offset (Note 3) | Hz | +600 |  |  |
|  | Synchronization |  | GNSS or GNSS-equivalent |  |  |
|  | Antenna configuration |  | 1x2 Low |  |  |
| PSFCH resource period |  | Slot | 4 | 4 | 4 |
| MinTimeGapPSFCH |  | Slot | 3 | 3 | 3 |
| Note 1: {x, y}: x and y means the number of DMRS symbols for slot with PSFCH transmission and without PSFCH transmission, respectively. Note 2: Time offset of transmitted Sidelink UE signal with respect to GNSS referring timing. Note 3: Frequency offset of transmitted Sidelink UE signal with respect to GNSS reference frequency. |  |  |  |  |  |

The minimum performance can scope down from existing case.

Table 11.1.2.1.X-2: Minimum performance

| Test num. | Reference channel | Bandwidth (MHz)/ Subcarrier spacing(kHz) | Modulation format and code rate | Propagation condition | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | PSSCH BLER (%) | SNR(dB) of PSSCH |
| 1 | R.PSSCH.2-1.3 | 20 / 30 | 64QAM, 0.43 | TDLA30-180 |  |  |

For PSFCH test case and minimum performance, the existing SL test parameters and test scenarios are based on NACK only case. But, for SL-U the NACK only scenario is not yet decided to be supported at RAN1. So, RAN4 need to check the status of RAN1.

Observation 1: For SL-U the NACK only scenario is not yet decided to be supported at RAN1. So, RAN4 need to check the status of RAN1.

Proposal 5: RAN4 need to discuss how to define PSFCH demodulation performance in Rel18.

Conclusion

In this contribution, we provide our views on work scope and test cases for demodulation performance of SL evolution based on the WID [1].

Observation 1: For SL-U the NACK only scenario is not yet decided to be supported at RAN1. So, RAN4 need to check the status of RAN1.

Proposal 1: RAN4 to consider the NR sidelink CA operation and sidelink on unlicensed spectrum as work scope for demodulation performance.

Proposal 2: RAN4 need to evaluate the SL-U demodulation performance with the interlacing RB mapping and the two candidate starting point in slot.

Proposal 3: SL-U demodulation performance can reuse the existing SL test parameters as much as possible considering interlacing RB mapping and two candidate starting point.

- For transmission model, reuse the existing NR-U transmission model as specified in TS38.101-4 B.5 as much as possible.

Proposal 4: At least PSSCH and PSFCH demodulation performance requirements for SL-U are necessary.

Proposal 5: RAN4 need to discuss how to define PSFCH demodulation performance in Rel18.

Reference

[1] RP-222806, “WID revision: NR sidelink evolution,” OPPO.

[2] 3GPP TS38.101-4 v18.0.0, “UE radio transmission and reception; Part4: Performance requirements (Release 18),” June, 2023.