3GPP TSG-RAN4 Meeting # 108-bis	R4-2315096

# Xiamen, Oct. 9 – Oct. 13, 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.106 | CR | Draft | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DRAFT CR for TS 38.106, Introduction of Operating band and channel arrangement for NCR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_netcon_repeater-Core |  |  |  |  |  | Date: |  |  | 2023-09-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To define operating band and channel arrangement requirements for NCR-MT. |  |  |  |  |  |  |  |  |
| Summary of change: |  | According to the work split, perating operating band and channel arrangement requirements for NCR-MT are to be introduced. 5.3A Channel arrangement for NCR-MT 5.3A.1 Channel spacing 5.3A.2 Channel raster 5.3A.3 Synchronization raster 5.3A.4 TX–RX frequency separation 5.4 NCR-MT channel bandwidth |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The operating band and channel arrangement requirements of NR NCR would be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New 5.3A, 5.3A.1, 5.3A.2, 5.3A.3, 5.3A.4, 5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change>

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.104: “NR; Base Station (BS) radio transmission and reception”.

[3]	3GPP TR 25.942: "RF system scenarios".

[4]	Recommendation ITU-R SM.328: "Spectra and bandwidth of emissions".

[5]	ITU-R Recommendation SM.329: "Unwanted emissions in the spurious domain".

[6]	ITU-R Recommendation M.1545: “Measurement uncertainty as it applies to test limits for the terrestrial component of International Mobile Telecommunications – 2000”.

[7]	3GPP TS 38.115-1: “NR; Repeater conformance testing - Part 1: Conducted conformance testing”.

[8]	3GPP TS 38.115-2: “NR; Repeater conformance testing - Part 2: Radiated conformance testing”.

[9]	ERC Recommendation 74-01, "Unwanted emissions in the spurious domain".

[10]	"Title 47 of the Code of Federal Regulations (CFR)", Federal Communications Commission.

[11]	Void

[12]	Void

[13]	3GPP TS 38.101-1: “NR User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”.

[14]	3GPP TS 38.101-2: “NR User Equipment (UE) radio transmission and reception: Part 2: Range 2 Standalone”.

[15]	Void

[16]	Void

[17]	Void

[18]	Void

[19]	3GPP TS 38.213: “NR; Physical layer procedures for control”.

[20]	3GPP TS 36.104: “Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception”

[21]	3GPP TS 38.104: “NR; Base Station (BS) radio transmission and reception”

[22]	3GPP TS 38.211: "NR; Physical channels and modulation".

<Next Change>

5.3A	Channel arrangement for NCR-MT

5.3A.1	Channel spacing

For NCR-MT, the channel spacing is the same as specified for UE in TS38.101-1 [13] for FR1 in subclause 5.4.1 and in TS38.101-2 [14] for FR2 in subclause 5.4.1

5.3A.2	Channel raster

5.3A.2.1	NR-ARFCN and channel raster

For NCR-MT, the NR-ARFCN and channel raster is the same as specified for UE in TS38.101-1 [13] for FR1 in subclause 5.4.2.1 and in TS38.101-2 [14] for FR2 in subclause 5.4.2.1.

5.3A.2.2	Channel raster to resourse element mapping

For NCR-MT, the Channel raster to resource element mapping is the same as specified for UE in TS38.101-1 [13] for FR1 in subclause 5.4.2.2 and in TS38.101-2 [14] for FR2 in subclause 5.4.2.2.

5.3A.2.3	Channel raster to resourse element mapping

For NCR-MT, the channel raster entries for NR bands for FR1 in Table 5.2-1 are the same as specified for UE in TS38.101-1 [13] in subclause 5.4.2.3 and for NR bands for FR2 defined in TS38.104 [21] are the same as specified for UE in TS38.101-2 [14] in subclause 5.4.2.3.

5.3A.2.4	Channel raster entries for each operating band

For NCR-MT, the channel raster entries for NR bands for FR1 in Table 5.2-1 are the same as specified for UE in TS38.101-1 [13] in subclause 5.4.2.3 and for NR bands for FR2 defined in TS38.104 [21] are the same as specified for UE in TS38.101-2 [14] in subclause 5.4.2.3.

5.3A.3	Synchronization raster

5.3A.3.1	Synchronization raster and numbering

For NCR-MT, the synchronization raster and numbering are the same as specified for UE in TS38.101-1 [13] for FR1 in subclause 5.4.3.1 and in TS38.101-2 [14] for FR2 in subclause 5.4.3.1.

5.3A.3.2	Synchronization raster to synchronization block resource element mapping

For NCR-MT, the synchronization raster to synchronization block resource element mapping is the same as specified for UE in TS38.101-1 [13] for FR1 in subclause 5.4.3.2 and in TS38.101-2 [14] for FR2 in subclause 5.4.3.2.

5.3A.3.3	Synchronization raster entries for each operating band

For NCR-MT, the synchronization raster entries for NR bands for FR1 in Table 5.2-1 are the same as specified for UE in TS38.101-1 [3] in subclause 5.4.3.3 and for NR bands for FR2 defined in TS38.104 [2] are the same as specified for UE in TS38.101-2 [4] in subclause 5.4.3.3.

5.3A.4	TX–RX frequency separation

The default TX channel (carrier centre frequency) to RX channel (carrier centre frequency) separation for operating bands is specified in Table 5.3A.4-1.

Table 5.3A.4-1: NCR-MT TX-RX frequency separation

| NR Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| n1 | 190 MHz |
| n2 | 80 MHz |
| n3 | 95 MHz |
| n5 | 45 MHz |
| n7 | 120 MHz |
| n8 | 45 MHz |
| n12 | 30 MHz |
| n13 | -31 MHz |
| n14 | -30 MHz |
| n18 | 45 MHz |
| n20 | -41 MHz |
| n24 | -101.5, -120.5 MHz |
| n25 | 80 MHz |
| n26 | 45 MHz |
| n28 | 55 MHz |
| n30 | 45 MHz |
| n65 | 190 MHz |
| n66 | 400 MHz |
| n70 | 300 MHz |
| n71 | -46 MHz |
| n74 | 48 MHz |
| n85 | 30 MHz |
| n91 | 570 MHz – 595 MHz (NOTE 2) |
| n92 | 575 MHz – 680 MHz (μ = 0) 580 MHz – 675 MHz (μ = 1) (NOTE 2) |
| n93 | 517 MHz – 547 MHz (NOTE 2) |
| n94 | 522 MHz – 632 MHz (μ = 0) 527 MHz – 627 MHz (μ = 1) (NOTE 2) |
| n100 | 45 MHz |
| n105 | -51 MHz |
| NOTE 1: Void NOTE 2: The range of TX-RX frequency separation given paired UL and DL channel bandwidths BWUL and BWDL is given by the respective lower and upper limit FDL_low – FUL_high + 0.5(BWDL + BWUL) and FDL_high – FUL_low – 0.5(BWDL + BWUL). The UL and DL channel bandwidth combinations specified in Table 5.4.5-1 and 5.4.6-1 depend on the subcarrier spacing configuration μ [22]. |  |

5.4	NCR-MT channel bandwidth

5.4.1	General

The NCR-MT channel bandwidth supports a single NR RF carrier in the uplink or downlink at the NCR-MT. From a BS perspective, different NCR-MT channel bandwidths may be supported within the same spectrum for transmitting to and receiving from NCR-MT connected to the BS. Transmission of multiple carriers to different NCR-MT within the BS channel bandwidth can be supported.

From a NCR-MT perspective, the NCR-MT is configured with one or more BWP / carriers. The NCR-MT does not need to be aware of the BS channel bandwidth or how the BS allocates bandwidth to different NCR-MT.

The placement of the NCR-MT channel bandwidth for NCR-MT carrier is flexible but can only be completely within the NCR-Fwd passband.

The relationship between the NCR-MT channel bandwidth, the guardband and the transmission bandwidth configuration is shown in Figure 5.4.1-1.

Figure 5.4.1-1: Definition of channel bandwidth and transmission bandwidth configuration for one NR channel

### 5.4.2	Transmission bandwidth configuration

For NCR-MT, the transmission bandwidth configuration is the same as specified for UE in TS 38.101-1 [13] for FR1 in subclause 5.3.2 and in TS 38.101-2 [14] for FR2 in subclause 5.3.2.

5.4.3	Minimum guardband and transmission bandwidth configuration

For NCR-MT, the minimum guardband and transmission bandwidth configuration is the same as specified for UE in TS38.101-1 [13] for FR1 and in TS 38.101-2 [14] for FR2 in subclause 5.3.3.

5.4.4	RB alignment

For NCR-MT, the RB alignment is the same as specified for UE in TS38.101-1 [3] for FR1 in subclause 5.3.4 and in TS 38.101-2 [4] for FR2 in subclause 5.3.4

5.4.5	NCR-MT channel bandwidth per operating band

For NCR-MT, the channel bandwidth for NR bands for FR1 in Table 5.2-1 is the same as specified for UE in TS38.101-1[13] in subclause 5.3.5 and for NR bands for FR2 defined in TS38.104 [21] is the same as specified for UE in TS38.101-2[14] in subclause 5.3.5.

<End of Change>