3GPP RAN WG4 Meeting #108bis	R4-2315100

Xiamen, China, October 09 – October 13, 2023

Title: 	Discussion on RRM requirements of RedCap UE Positioning

Source: 	CATT

Agenda item:	5.22.2.4

Document for:	Discussion

Introduction

In RAN4#108 meeting, RAN4 has discussed general RRM impacts of the RedCap UE positioning with conclusions captured in [1]. In this paper, we will further discuss the potential RRM requirements for RedCap UE Positioning.

Discussion

General aspects/scenarios for RedCap positioning

Issue 2-1-1: Relation with Rel-16/Rel-17 positioning for PRS measurements with FH

In last meeting, RAN4 discussed the relation between Rel-18 RedCap positioning and Rel-16/Rel-17 positioning features for PRS measurements with FH, and the following agreements are achieved with TEG FFS.

Agreements: - All Rel-16 features are supported for RedCap positioning measurement with FH. - In addition, reduced number of samples with gaps and reduced Rx beam sweeping factor with gaps are supported for RedCap positioning measurement with FH. TEG is FFS.

TEG has an effect on the measurement period requirements and according to the definition agreed in RAN4#100 meeting, the timing errors are grouped by UE/TRP based on RF chains and antenna panel such that timing error difference in the same group is within a certain margin.

Issue 1-2-1 Feasibility of TEGs for timing error mitigation mechanism Agreements: - UE/TRP may group the timing errors for UE/TRP Rx/Tx (e.g., based on RF chains and antenna panel) such that timing error difference in the same group is within a certain margin

TEG therefore does not apply to 1Rx RedCap UEs, i.e., the TEG scaling factor in measurement period requirements kmultiTEG,i=1. For 2Rx RedCap UEs, it is possible to have 1 or 2 TEG(s) with each group including two or one RF chain(s)/antenna panel(s). If 2Rx RedCap UE has only one TEG, kmultiTEG,i equals to 1. If it has two TEGs, the value of kmultiTEG,i depends on if UE is capable of receiving/transmitting the same DL PRS/UL SRS with multiple TEGs simultaneously, and the potential values are {1, 2}. Hence, as a mechanism to mitigate timing errors and without any technical issues identified for FH case, we think Tx/Rx TEG can be supported for Rel-18 RedCap positioning with FH.

Proposal 1: TEG can be supported for Rel-18 RedCap positioning with FH.

PRS measurements for RedCap without FH

Issue 2-2-1: Side conditions for 1Rx RedCap UE PRS measurements without FH

In last meeting, RAN4 made agreements on the side conditions for 1Rx RedCap UE PRS measurements without FH in fading channel with 4 samples. However, the side conditions for fading channel with 1 sample are still TBD [2]. In this section, we provide our simulation results for 1Rx RedCap UE PRS measurements without FH in fading channel with 1 sample to evaluate the measurement performance. The side conditions for 4 samples are reused for 1 sample evaluation.

Table 1. Measurement errors of RSTD for fading channel（Tc）

| SINRs [dB] |  | <-6,-10,-10> (Nsample = 1) |  |  |  |  |  | <-6,-6,-6> (Nsample = 1) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCS |  | 15kHz |  |  | 30kHz |  | 60kHz | 15kHz |  |  | 30kHz |  | 60kHz |
| BW |  | 5M | 10M | 20M | 10M | 20M | 20M | 5M | 10M | 20M | 10M | 20M | 20M |
| DL PRS Repetition Factor |  | 4 | 1 | 1 | 4 | 1 | 4 | 4 | 1 | 1 | 4 | 1 | 4 |
| Cell2 | TDL-A | 28.61 | 18.71 | 7.12 | 20.37 | 11.01 | 16.23 | 32.08 | 11.21 | 5.03 | 17.63 | 6.01 | 11.23 |
| Cell3 |  | 162.9 | 83.94 | 49.3 | 88.8 | 50.5 | 56.83 | 180.8 | 91.81 | 60.28 | 103.1 | 61.04 | 62.5 |

Table 2. Measurement errors of PRS RSRP for fading channel（dB）

| SINRs [dB] |  | <-3,-10,-10> (Nsample = 1) |  |  |  |  |  | <-3,-6,-6> (Nsample = 1) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCS |  | 15kHz |  |  | 30kHz |  | 60kHz | 15kHz |  |  | 30kHz |  | 60kHz |
| BW |  | 5M | 10M | 20M | 10M | 20M | 20M | 5M | 10M | 20M | 10M | 20M | 20M |
| DL PRS Repetition Factor |  | 4 | 1 | 1 | 4 | 1 | 4 | 4 | 1 | 1 | 4 | 1 | 4 |
| Cell1 | TDL-A | 1.3 | 0.36 | 0.4 | 0.86 | 0.61 | 0.73 | 0.76 | 0.44 | 0.27 | 0.56 | 0.42 | 0.49 |
| Cell2 |  | 4.76 | 2 | 1.07 | 3.6 | 1.66 | 2.57 | 1.13 | 0.68 | 0.39 | 0.94 | 0.55 | 0.69 |
| Cell3 |  | 4.48 | 1.97 | 1.08 | 3.55 | 1.85 | 2.62 | 1.21 | 0.58 | 0.39 | 0.94 | 0.6 | 0.74 |

Table 3. Measurement errors of UE Rx-Tx time difference for fading channel（Tc）

| SINRs [dB] |  | <-3,-10,-10> (Nsample = 1) |  |  |  |  |  | <-3,-6,-6> (Nsample = 1) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCS |  | 15kHz |  |  | 30kHz |  | 60kHz | 15kHz |  |  | 30kHz |  | 60kHz |
| BW |  | 5M | 10M | 20M | 10M | 20M | 20M | 5M | 10M | 20M | 10M | 20M | 20M |
| DL PRS Repetition Factor |  | 4 | 1 | 1 | 4 | 1 | 4 | 4 | 1 | 1 | 4 | 1 | 4 |
| Cell1 | TDL-A | 124.9 | 69.58 | 45.5 | 71.28 | 46.6 | 45.14 | 125.4 | 69.5 | 45.43 | 70.26 | 45.8 | 44.22 |
| Cell2 |  | 119.9 | 76.9 | 48.62 | 76.41 | 68.7 | 73.26 | 120.1 | 71.5 | 45.15 | 72.05 | 45.7 | 43.66 |
| Cell3 |  | 128.1 | 65.04 | 41.85 | 73.59 | 52.1 | 62.79 | 114.2 | 61.4 | 38.61 | 64.6 | 40.2 | 41.34 |

Observation 1: No obvious performance degradation observed with 1 sample in fading channel, compared with the results for 4 samples in fading channel.

In current RAN4 spec, there are no measurement accuracy requirements for PRS based RSTD, PRS-RSRP and UE Rx-Tx time difference with reduced number of samples for fading channel. Based on the simulation results in the tables above, we think the measurement accuracy requirements for fading channel with reduced number of samples can be defined since the measurement delay can be shortened and the measurement errors are acceptable.

Proposal 2: RAN4 to define measurement accuracy requirements for PRS based positioning measurements with reduced number of samples for fading channel. The details, e.g., accuracy and side conditions, can be FFS.

PRS measurements for RedCap with FH

Issue 2-3-1: Side conditions for 1Rx/2Rx RedCap UE PRS measurements with FH

In lasting meeting, simulation assumptions for 1Rx/2Rx RedCap UE PRS measurements with FH were determined. Companies are encouraged to evaluate the measurement errors based on the side conditions agreed for FH-less case. We provided our simulation results in [3].

Proposal 3: The side conditions for measurements without FH can be reused for measurements with FH.

Issue 2-3-2: PRS measurement period requirements for RedCap with FH

There are two cases according to RAN1’s agreements on the FH-based measurement and reporting methods:

Case1: A single measurement based on receiving multiple hops of the DL PRS or UL SRS for positioning.

Case2: One measurement where a measurement is associated with one received hop.

The main idea of frequency hopping is to achieve higher measurement accuracy by using larger bandwidth. Hence the measurement requirements for Case1 should be prioritized. In last meeting, we suggest to use Rel-17 measurement requirements as baseline for RedCap positioning with FH. Besides, RAN1 sent LS to request RAN4 to confirm the feasibility of using a single measurement gap to receive all DL PRS hops and RAN4 confirmed it is feasible. Based on this confirmation and the following achieved agreements, we think the Rel-17 measurement period requirements can be reused with a few modifications, e.g., the measurement duration Tlast,i for the last measurement sample is Tlast,i = Ti +MGL. For Case2, we think the Rel-17 measurement period requirements are applicable and can be reused directly.

Agreements: - All Rel-16 features are supported for RedCap positioning measurement with FH. - In addition, reduced number of samples with gaps and reduced Rx beam sweeping factor with gaps are supported for RedCap positioning measurement with FH. TEG is FFS.

Proposal 4: Measurement requirements for the single measurement based on multiple hops shall be prioritized.

Proposal 5: The Rel-17 measurement period requirements can be reused for RedCap positioning with FH with a few modifications, e.g., Tlast,i = Ti +MGL.

Issue 2-3-3: Effective BW for PRS measurement with FH

Regarding the calculation of effective BW, as we proposed in the last meeting, the effective BW can be derived by the equation below for Case1 assuming the measured hops have the same BW:

BWeffective=12*15*2μ*NRB_hop-NRB_overlap*Nhop+NRB_overlap

Where NRB_hop is the number of RBs per hop, Nhop is number of hops and NRB_overlap is the number of overlapping RBs between hops.

For Case2, the effective BW is equal to the BW occupied by the hop on which the measurement is performed. It can also be obtained by using the above equation with NRB_overlap=0 and Nhop=1.

Proposal 6: The effective bandwidth for PRS measurements for RedCap with FH can be derived by the equation: BWeffective=12*15*2μ*NRB_hop-NRB_overlap*Nhop+NRB_overlap.

Conclusions

This paper discussed the general issues for RedCap UE positioning, and the following proposals are provided:

Proposal 1: TEG can be supported for Rel-18 RedCap positioning with FH.

Observation 1: No obvious performance degradation observed with 1 sample in fading channel, compared with the results for 4 samples in fading channel.

Proposal 2: RAN4 to define measurement accuracy requirements for PRS based positioning measurements with reduced number of samples for fading channel. The details, e.g., accuracy and side conditions, can be FFS.

Proposal 3: The side conditions for measurements without FH can be reused for measurements with FH.

Proposal 4: Measurement requirements for the single measurement based on multiple hops shall be prioritized.

Proposal 5: The Rel-17 measurement period requirements can be reused for RedCap positioning with FH with a few modifications, e.g., Tlast,i = Ti +MGL.

Proposal 6: The effective bandwidth for PRS measurements for RedCap with FH can be derived by the equation: BWeffective=12*15*2μ*NRB_hop-NRB_overlap*Nhop+NRB_overlap.

Reference

[1] R4-2314353, WF on R18 NR Positioning – RedCap positioning and PRS/SRS BW aggregation, Ericsson, RAN4#108.

[2] R4-2310075, Updated simulation assumptions for 1Rx RedCap positioning, Ericsson, RAN4#107.

[3] R4-2315101, Simulation results for RedCap UE PRS measurements with FH, CATT, RAN4#108bis.