3GPP TSG-RAN WG4 Meeting # 108bis                                                         R4-2315106			R4-22xxxxx

# Xiamen, China, October 9 – October 13, 2023

Title: 	SBFD adjacent channel co-existence simulation results

Source: 	CATT

Agenda item:	5.19.2.1

Document for:	Discussion

- Background

This contribution provides the SBFD adjacent channel co-existence simulation results of scenario 2(FR1 Urban Hotspot -> Urban Hotspot), scenario 8(FR2 Urban Micro -> Urban Micro) and scenario 9(FR2 Indoor -> Indoor).

- Discussion

  - SBFD adjacent channel co-existence simulation scenarios

SBFD adjacent channel co-existence simulation scenarios summarized in table 2.1-1, relevant case are specified in table 2.1-2. SBFD subband configuration is {DU}.

Table 2.1-1: SBFD adjacent channel co-existence simulation scenarios

| FR | Scenario No. | Deployment Scenario1 | Priority |
| --- | --- | --- | --- |
|  |  | (Aggressor -> Victim) |  |
| FR1 (4GHz) | 1 | Urban Macro -> Urban Macro | High |
|  | 2 | Urban Hotspot -> Urban Hotspot | Note 4 |
|  | 3 | Indoor -> Indoor | Low |
|  | 4 | CableLabs：UMa-to-UMi | Note 5 |
|  | 5 | UMi-to-UMi | Note 6 |
| FR2 (30GHz) | 6 | Urban Macro -> Urban Macro | High |
|  | 7 | Urban Hotspot -> Urban Hotspot | Note 4 |
|  | 8 | Urban Micro -> Urban Micro | Low |
|  | 9 | Indoor -> Indoor | Low |
| Note 1: The Urban Macro is agreed as baseline scenario for SBFD co-ex study with high priority in RAN4#104-e, while it does not preclude other scenarios. |  |  |  |
| Note 2: The Urban Hotspot uses the same assumption as Urban Macro, except that Urban Macro uses random dropping method for UE while Urban Hotspot uses cluster-based dropping method for UE. Both random dropping and cluster-based dropping for calibration. |  |  |  |
| Note 3: Consider Urban Macro scenario first for calibration purpose. |  |  |  |
| Note 4: Companies are encouraged to provide simulation results for Urban Hotspot scenario as 2nd priority. [Editor’s Note: Agreement 2.2.1 of R4-2302888] |  |  |  |
| Note 5: Companies also encouraged to simulate Uma-to-UMi co-existence scenario as 2nd priority. [Editor’s Note: Agreement 2.2.3 of R4-2302888] |  |  |  |
| Note 6: Use UMi simulation assumption R4-2305922 as starting point. Consider Tx power refer to 3GPP UMi output power. Detailed simulation assumptions will be discussed after RAN4#107 meeting. |  |  |  |

Table 2.1-2: SBFD adjacent channel co-existence simulation case

| Case | Victim | Aggressor | Figures: | Aggressor baseline | Priority |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Aggressor(left) and Victim(right) |  |  |
| 1 | NR TDD DL | SBFD (DU) |  | NR TDD DL | High |
| 2 | NR TDD UL | SBFD (DU) |  | NR TDD UL | Low |
| 3 | SBFD (DU) | NR TDD DL |  | No system in adj channel | High |
| 4 | SBFD (DU) | NR TDD UL |  |  | Low |

  - Simulation calibration results

Scenario 2: Urban Hotspot -> Urban Hotspot

Table 2.2.1-1:  Victim with aggressor ACI / victim with baseline ACI compared to victim without ACI (Scenario 2)

Table 2.2.1-2:  TP degradation corresponding to ACIR (Scenario 2)

| Case | Victim | TP degradation corresponding to ACIR |  |
| --- | --- | --- | --- |
|  |  | ACIR to meet 5% TP degradation @5% | ACIR to meet 5% TP degradation @Mean |
| 1 | NR TDD DL | Very high ACIR | Relative ACIR+5dB |
| 2 | NR TDD UL | Very high ACIR | Very high ACIR |
| 3 | SBFD (UL) | Very high ACIR | Very high ACIR |
|  | SBFD (DL) | Relative ACIR | Relative ACIR |
| 4 | SBFD (UL) | Relative ACIR | Relative ACIR |
|  | SBFD (DL) | Relative ACIR | Relative ACIR |

From the above simulation results, it can be observed,

Observation 1: For scenarios 2,

- For case 4, SINR/throughput degradation is acceptable using relative ACIR.

- For case 1, case 2 and case 3, higher than 5% throughput loss is observed. Very high ACIR is needed to meet 5% TP loss.

Scenario 8: Urban Micro -> Urban Micro

Table 2.2.2-1:  Victim with aggressor ACI / victim with baseline ACI compared to victim without ACI (Scenario 8)

Table 2.2.2-2:  TP degradation corresponding to ACIR (Scenario 8)

| Case | Victim | TP degradation corresponding to ACIR |  |
| --- | --- | --- | --- |
|  |  | ACIR to meet 5%TP degradation @5% | ACIR to meet 5% TP degradation @Mean |
| 1 | NR TDD DL | Relative ACIR | Relative ACIR |
| 2 | NR TDD UL | Very high ACIR | Relative ACIR+2dB |
| 3 | SBFD (UL) | Very high ACIR | Relative ACIR+2dB |
|  | SBFD (DL) | Relative ACIR | Relative ACIR |
| 4 | SBFD (UL) | Relative ACIR | Relative ACIR |
|  | SBFD (DL) | Relative ACIR | Relative ACIR |

From the above simulation results, it can be observed,

Observation 2: For scenarios 8,

- For case 1 and case 4, SINR/throughput degradation is acceptable using relative ACIR.

- For case 2 and case 3, higher than 5% throughput loss is observed. Very high ACIR is needed to meet 5% TP loss for 5%. Relative ACIR + 2dB is needed to meet 5% TP loss for average, and relative ACIR + 2dB may be achieved if ACS capabilities of NR TDD BS and SBFD BS can be improved to 32dB and 28dB separately.

Scenario 9: Indoor -> Indoor

Table 2.2.2-1:  Victim with aggressor ACI / victim with baseline ACI compared to victim without ACI (Scenario 9)

Table 2.2.2-2:  TP degradation corresponding to ACIR (Scenario 9)

| Case | Victim | TP degradation corresponding to ACIR |  |
| --- | --- | --- | --- |
|  |  | ACIR to meet 5% TP degradation @5% | ACIR to meet 5% TP degradation @Mean |
| 1 | NR TDD DL | Relative ACIR | Relative ACIR |
| 2 | NR TDD UL | Relative ACIR+2dB | Relative ACIR |
| 3 | SBFD (UL) | Relative ACIR+2dB | Relative ACIR |
|  | SBFD (DL) | Relative ACIR | Relative ACIR |
| 4 | SBFD (UL) | Relative ACIR | Relative ACIR |
|  | SBFD (DL) | Relative ACIR | Relative ACIR |

From the above simulation results, it can be observed,

- For case 1 and case 4, SINR/throughput degradation is acceptable using relative ACIR.

- For case 2 and case 3, Relative ACIR + 2dB is needed to meet 5% TP loss for 5%, and relative ACIR + 2dB may be achieved if ACS capabilities of NR TDD BS and SBFD BS can be improved to 32dB and 28dB separately. Relative ACIR can meet 5% TP loss for average.

  - Co-existence simulation results

Table 2.3-1:  TP degradation corresponding to ACIR

|  |  | TP degradation corresponding to ACIR |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case | Victim | Scenario 2 |  | Scenario 8 |  | Scenario 9 |  |
|  |  | ACIR to meet 5% TP degradation @5% | ACIR to meet 5% TP degradation @Mean | ACIR to meet 5% TP degradation @5% | ACIR to meet 5% TP degradation @Mean | ACIR to meet 5% TP degradation @5% | ACIR to meet 5% TP degradation @Mean |
| 1 | NR TDD DL | Very high ACIR | Relative ACIR+5dB | Relative ACIR | Relative ACIR | Relative ACIR | Relative ACIR |
| 2 | NR TDD UL | Very high ACIR | Very high ACIR | Very high ACIR | Relative ACIR+2dB | Relative ACIR+2dB | Relative ACIR |
| 3 | SBFD (UL) | Very high ACIR | Very high ACIR | Very high ACIR | Relative ACIR+2dB | Relative ACIR+2dB | Relative ACIR |
|  | SBFD (DL) | Relative ACIR | Relative ACIR | Relative ACIR | Relative ACIR | Relative ACIR | Relative ACIR |
| 4 | SBFD (UL) | Relative ACIR | Relative ACIR | Relative ACIR | Relative ACIR | Relative ACIR | Relative ACIR |
|  | SBFD (DL) | Relative ACIR | Relative ACIR | Relative ACIR | Relative ACIR | Relative ACIR | Relative ACIR |

Table 2.3-2:  Realizability of every case

| Case | Victim | Scenario 2 |  | Scenario 8 |  | Scenario 9 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | TP degradation @5% | TP degradation @Mean | TP degradation @5% | TP degradation @Mean | TP degradation @5% | TP degradation @Mean |
| 1 | NR TDD DL | No | No | Yes | Yes | Yes | Yes |
| 2 | NR TDD UL | No | No | No | ACS of NR TDD BS need to be 32dB | ACS of NR TDD BS need to be 32dB | Yes |
| 3 | SBFD (UL) | No | No | No | ACS of SBFD BS need to be 28dB | ACS of SBFD BS need to be 28dB | Yes |
|  | SBFD (DL) | Yes | Yes | Yes | Yes | Yes | Yes |
| 4 | SBFD (UL) | Yes | Yes | Yes | Yes | Yes | Yes |
|  | SBFD (DL) | Yes | Yes | Yes | Yes | Yes | Yes |
| Note 1：“Yes”: the performance degradation is acceptable when relative ACIR is used. Note 2：“No”: the performance degradation is not acceptable when relative ACIR is used. |  |  |  |  |  |  |  |

Victim with aggressor ACI compared to victim without ACI,

- For scenarios 2,

- For case 4, SINR/throughput degradation is acceptable using relative ACIR.

- For case 1, case 2 and case 3, higher than 5% throughput loss is observed. Very high ACIR is needed to meet 5% TP loss.

- For scenarios 8,

- For case 1 and case 4, SINR/throughput degradation is acceptable using relative ACIR.

- For case 2 and case 3, higher than 5% throughput loss is observed. Very high ACIR is needed to meet 5% TP loss for 5%. Relative ACIR + 2dB is needed to meet 5% TP loss for average, and relative ACIR + 2dB may be achieved if ACS capabilities of NR TDD BS and SBFD BS can be improved to 32dB and 28dB separately.

- For scenarios 9,

- For case 1 and case 4, SINR/throughput degradation is acceptable using relative ACIR.

- For case 2 and case 3, Relative ACIR + 2dB is needed to meet 5% TP loss for 5%, and relative ACIR + 2dB may be achieved if ACS capabilities of NR TDD BS and SBFD BS can be improved to 32dB and 28dB separately. Relative ACIR can meet 5% TP loss for average.

- According to assumption of SBFD system, ACLR and ACS of UE, also of NR TDD BS are fixed value, only ACS of SBFD BS can be improved, the improved ACS capability of TDD BS need to be considered.

- Summary

This contribution summarizes the simulation results from our company; the realizability of every case is specified in table 2.3-2.

Observation 1: For scenarios 2,

- For case 4, SINR/throughput degradation is acceptable using relative ACIR.

- For case 1, case 2 and case 3, higher than 5% throughput loss is observed. Very high ACIR is needed to meet 5% TP loss.

Observation 2: For scenarios 8,

- For case 1 and case 4, SINR/throughput degradation is acceptable using relative ACIR.

- For case 2 and case 3, higher than 5% throughput loss is observed. Very high ACIR is needed to meet 5% TP loss for 5%. Relative ACIR + 2dB is needed to meet 5% TP loss for average, and relative ACIR + 2dB may be achieved if ACS capabilities of NR TDD BS and SBFD BS can be improved to 32dB and 28dB separately.

Observation 2: For scenarios 9,

- For case 1 and case 4, SINR/throughput degradation is acceptable using relative ACIR.

- For case 2 and case 3, Relative ACIR + 2dB is needed to meet 5% TP loss for 5%, and relative ACIR + 2dB may be achieved if ACS capabilities of NR TDD BS and SBFD BS can be improved to 32dB and 28dB separately. Relative ACIR can meet 5% TP loss for average.

Reference

[1] R4-2313872, “WF for SBFD co-existence study”, RAN4#108