3GPP TSG-RAN WG4 Meeting # 108bis                                                          R4-2315107

Xiamen, China, October 9 – October 13, 2023

Title: 	Discussion on BS RF requirements impact for SBFD

Source: 	CATT

Agenda item:	5.19.2.3

Document for:	Discussion

- Background

WF [1] summarized the discussion status in last meeting. Views on some requirements were agreed, this contribution provides our further analysis of the requirements based on the WF [1].

- RF Requirement Impact

  - New RF requirements

In last meeting, the following new RF requirements are FFS.

  - in-channel adjacent subband leakage ratio

  - in-channel adjacent subband blocking

  - in-channel adjacent subband selectivity

From co-existence aspect consideration, we think the following two requirements are necessary,

- In-channel adjacent sub-band leakage power ratio

- In-channel adjacent sub-band selectivity

The above requirement can use the co-existence study conclusion.

Proposal 1: The following new RF requirements are needed for SBFD BS, the requirements can be derived from the co-existence simulation.

- In-channel adjacent sub-band leakage power ratio

- In-channel adjacent sub-band selectivity

For the in-channel adjacent subband blocking requirement, it may also be needed considering the co-existence with other BS. For legacy system, non-colocated scenario may be the worst case for blocking signal if the the co-located base stations are close in the distance. But for SBFD deployment, the co-located BS sub-band Tx could also cause blocking to victim BS sub-band Rx if the isolation is not large enough. This case may need more discussion in WI phase pending on decision of which one of WA, MA and LA BS will be defined.

Proposal 2: In-channel adjacent subband blocking requirement may be needed. Both colocation and non-colocation scenario should be analyzed or simulated for the blocking signal level.

  - Transmitter spurious emissions

Issue 3-2-5: Unwanted emissions

- Agreement:

  - Co-location/co-existence:

    - Option 1: Co-location requirement can’t use 30 dB coupling loss as the coupling loss assumption for SBFD capable gNB co-location related requirement.

    - Option 2: No update on existing requirements, it’s declaration basis whether BS need to follow the requirements. FFS whether applicable for SBFD symbols/slots.

The legacy transmitter spurious emission limits apply from 9 KHz to 12.75 GHz, excluding the frequency range from ΔfOBUE below the lowest frequency of each supported downlink operating band, up to ΔfOBUE above the highest frequency of each supported downlink operating band. As this requirement is applied out of operating band, there’re no difficulties for SBFD capable BS to meet the emission limits. The 30 dB coupling loss colocation assumption can also apply for transmitter spurious emissions co-location requirements. So we have the following proposal.

Proposal 3: The existing additional spurious emissions requirements and co-location with other base stations requirements can be reused on SBFD slots.

  - Transmitter transient period between SBFD and non-SBFD

For transmitter transient period, discussion is needed on whether this requirement is defined or not. Our understanding is that this time can be provided to RAN1 for the possible guard period design but it may not be very necessary to define it as a RF requirement. Similarly with the NR transition time, it defined in 38.211 specification as following,

Table 4.3.2-3: Transition time NRx-Tx and NTx-Rx

| Transition time | FR1 | FR2 |
| --- | --- | --- |
| NTx-Rx | 25600 | 13792 |
| NRx-Tx | 25600 | 13792 |

The RF capability was provided by RAN4. The similar approach can be used for transient period between SBFD and non-SBFD, reconfigurations, etc. There’s another reason for not defining it as an RF requirement is that it’s difficult to measure it. For SBFD, it’s no sufficient only measuring the Tx power. In theory, Rx performance should also be measured. And the transient period cases can be very large, it’s not possible to measure all of them. So using the similar approach of transition time can be considered in RAN4.

Proposal 4: Transmitter transient period is not defined as a RF requirement, the capability can be provided to RAN1.

  - TX intermodulation requirement

Traditionally for the transmitter intermodulation requirement is to address the coexistence between the transmitter antenna from one BS and the transmitter antenna from another BS in case the antennas are co-located with assumption that the worst-case coupling loss between them is 30dB. The requirement assumes that the co-located BSs transmit the same power, and the transmitted signals are adjacent to each other in the frequency domain. For SBFD gNB, if co-located BS Tx power is 46 dBm and the coupling loss is 30 dB, then SBFD UL band received power from other co-located BS is 16 dBm because there’s no RF filter rejection for this signal. It will block SBFD Rx path. So the legacy Tx IMD requirement can’t be reused for SBFD capable BS. How the co-location BS works on the SBFD slots should be considered.

Observation 1: For transmitter intermodulation requirement, co-location coupling loss assumption can’t use 30 dB for SBFD capable gNB.

Proposal 5: The transmitter intermodulation co-location scenario should be revisited for SBFD deployment.

  - Receiver intermodulation

The following is the agreement for receiver IMD in last meeting,

- Issue 3-3-6: Receiver intermodulation

- Agreement:

  - RX intermodulation requirement and the interference levels shall be determined by RAN4 co-existence study, and for the definition of RX intermodulation requirement:

    - Conducted RX intermodulation: Take the existing wanted signal of RX intermodulation requirement by using the existing reference sensitivity level.

    - OTA RX intermodulation: The OTA sensitivity degradation shall be taken into account to determine the level of wanted signal and interference signal mean power.

  - FFS whether an additional requirement based on a single input signal placed to cause IM with the RX sub-band provides any additional robustness, and whether such a requirement is anyhow implicitly captured by the SBFD RX blocking requirement.

There’s one issue that if additional requirement based on a single input signal is needed and if Rx blocking requirement can cover this requirement. For the single input interference scenario, there’s possibility that the IM product of interference signal and Tx sub-band signal falls in UL subband for some UL/DL subband configurations.

If the SBFD configuration is DUD, there could be two IMD interfering scenarios. One is two interfering signals input, the other is only single interfering signal input.

Figure 1:  SBFD DUD configuration

As shown in Figure 1, f1, f2 and f4 are interfering signals, f3 is the Tx subband signal received by Rx path. For the two interference signals scenario, the whole IM product for f1 and f2 falls in the UL subband, but only part of the IM product of f2 and f3 fall in the UL subband if the f1 and f3 BW is the same as UL subband. For single interference signal scenario, the IM product of f3 and f4 can be overlapped with the whole UL subband. It’s difficult to say the two interfering signal scenario can cover the one interfering signal scenario if f3 signal level is big. According the SI feasibility analysis provided by companies, usually IBB blocking signal level is assumed for the Tx subband signal level received in Rx path. So the f3 signal level may be bigger than IMD interference signals, in that case one interfering signal scenario may be needed.

If the SBFD configuration is DU or UD, there may be also two IMD interfering scenarios. One is two interfering signals input, the other is only single interfering signal input, as shown in Figure 2, the IM analysis and results are similar with scenario which SBFD configuration is DUD.

Figure 2: SBFD DU configuration

Observation 2: For receiver IMD requirement, it’s difficult to say the two interfering signal scenario can cover the single interfering signal scenario for both DUD and DU subband configuration.

For the issue of if single interfering signal scenario is covered by IBB requirement, it depends on the frequency position of the single interfering signal. There’s also a difference between the IBB and single interfering signal that IBB signal is modulated signal but single interfering signal for Rx IMD could be CW. So we have the following observation.

Observation 3: For receiver IMD requirement, single interfering signal scenario may not be equal to IBB scenario.

- Summary

In this contribution, the analysis for the open issues of BS RF requirements for SBFD BS is provided. We have the following proposals and observations.

Proposal 1: The following new RF requirements are needed for SBFD BS, the requirements can be derived from the co-existence simulation.

- In-channel adjacent sub-band leakage power ratio

- In-channel adjacent sub-band selectivity

Proposal 2: In-channel adjacent subband blocking requirement may be needed. Both colocation and non-colocation scenario should be analyzed or simulated for the blocking signal level.

Proposal 3: The existing additional spurious emissions requirements and co-location with other base stations requirements can be reused on SBFD slots.

Proposal 4: Transmitter transient period is not defined as a RF requirement, the capability can be provided to RAN1.

Observation 1: For transmitter intermodulation requirement, co-location coupling loss assumption can’t use 30 dB for SBFD capable gNB.

Proposal 5: The transmitter intermodulation co-location scenario should be revisited for SBFD deployment.

Observation 2: For receiver IMD requirement, it’s difficult to say the two interfering signal scenario can cover the single interfering signal scenario for both DUD and DU subband configuration.

For the issue of if single interfering signal scenario is covered by IBB requirement, it depends on the frequency position of the single interfering signal. There’s also a difference between the IBB and single interfering signal that IBB signal is modulated signal but single interfering signal for Rx IMD could be CW. So we have the following observation.

Observation 3: For receiver IMD requirement, single interfering signal scenario may not be equal to IBB scenario.

- Reference

[1] R4-2313867, “WF for feasibility and BS requirements impact on SBFD operation”, RAN4#108

[2] 3GPP TS 38.104, Base Station (BS) radio transmission and reception

[3] 3GPP TR 38.817-02, General aspects for Base Station (BS) Radio Frequency (RF) for NR