3GPP TSG-RAN WG4 Meeting #108bis                                                                                   R4-2315119

Xiamen, China, October 09 – October 13, 2023

Source: 	CATT

Title: 	Further discussion on co-existence study for above 10GHz bands

Agenda item:	5.26.2

Document for:	Approval

- Introduction

This contribution provides our views on co-existence study for above 10GHz bands based on our co-existence simulation result.

- Discussion

According to WF [1], the co-existence simulation scenarios are show as below:

| No. | Combination | Aggressor | Victim | Frequency band | Scope of Coexistence Simulation |
| --- | --- | --- | --- | --- | --- |
| 1 | TN with NTN | NTN UL | TN UL | 27 GHz | ACLR NTN UE to be varied/defined ACS TN gNB fixed |
| 2 | TN with NTN | TN UL | NTN UL | 27 GHz | ACLR TN UE fixed ACS NTN SAN to be varied/defined |
| 3 | TN with NTN | NTN UL | TN DL | 27 GHz | ACLR NTN UE to be varied/defined ACS TN UE fixed |
| 4 | TN with NTN | TN DL | NTN UL | 27 GHz | ACLR TN gNB fixed ACS NTN SAN to be varied/defined |
| 5 | TN with NTN | TN DL | NTN DL | 17 GHz | ACLR TN gNB fixed ACS NTN UE to be varied/defined |
| 6 | TN with NTN | NTN DL | TN DL | 17 GHz | ACLR NTN SAN to be varied/defined ACS TN UE fixed |
| 7 | TN with NTN | NTN DL | TN UL | 17 GHz | ACLR NTN SAN to be varied/defined ACS TN gNB fixed |
| 8 | TN with NTN | TN UL | NTN DL | 17 GHz | ACLR TN UE fixed ACS NTN UE to be varied/defined |
| NOTE 1: For coexistence between Ka-Band DL and adjacent TN bands, there are no 3GPP defined/specified TN bands. NOTE 2: To deprioritize scenario 7 and 8 for SAN elevation angle as 90-degree cases. Scenario 7 and 8 should still be studied for SAN elevation angle as 25-degree cases (computed from the center of the central beam). |  |  |  |  |  |

Referring to our co-existence simulation result in excel [2], we plot required ACIR curve of case 1, 5 and 6 as following.

Case 1: Aggressor NTN UL to Victim TN UL

Figure 2-1: Case 1: Aggressor NTN UL to Victim TN UL (GEO, LEO-1200, LEO-600)

For case 1: aggressor NTN UL to victim TN UL, as shown in Figure 2-1, for 90 and 25 degree elevation angle, required ACIR @ 5% throughput loss for GEO, LEO-1200 and LEO-600 is 6.8, 6.8 and 6.75, respectively. From figure 2-1, curve of 2.5dB UE NF and curve of 6dB UE NF almost coincide, so impact of UE NF is small.

Case 5: Aggressor TN DL to Victim NTN DL

Figure 2-2: Case 5: Aggressor TN DL to Victim NTN DL (GEO, LEO-1200, LEO-600)

For case 5: aggressor TN DL to victim NTN DL, as shown in Figure 2-2, for 90 degree elevation angle, the required ACIR for GEO, LEO-1200 and LEO-600 is 38, 32 and 34, respectively. Comparing to 28dB ACLR of FR2-1 TN BS, for case 5, for 90 degree SAN elevation angle, the required ACIR is larger. From figure 2-2, curve of 2.5dB UE NF and curve of 6dB UE NF almost coincide, so impact of UE NF is small. For 25 degree elevation angle, the required ACIR for GEO, LEO-1200 and LEO-600 is 100% (actually it is NaN, 100% is used for plotting curve). The reason for NaN is that CL between VSAT and SAN is quite larger, which cause the throughput of NTN DL is zero, so throughput loss is NaN.

Observation 1:

Observation 1: For case 5: aggressor TN DL to victim NTN DL, for 25 degree SAN elevation angle, CL between VSAT and SAN is quite larger, which cause the throughput of NTN DL is zero, so throughput loss is NaN.

Case 6: Aggressor NTN DL to Victim TN DL

Figure 2-3: Case 6: Aggressor NTN DL to Victim TN DL(GEO, LEO-1200, LEO-600)

For case 6: aggressor NTN DL to victim TN DL, as shown in Figure 2-3, for 90 and 25 degree elevation angle, required ACIR @ 5% throughput loss for GEO, LEO-1200 and LEO-600 is 0. From figure 2-3, curve of 2.5dB UE NF and curve of 6dB UE NF almost coincide, so impact of UE NF is small.

- Conclusion

This contribution provides our co-existence simulation result for above 10GHz bands. The following observation is provided as follows:

Observation 1: For case 5: aggressor TN DL to victim NTN DL, for 25 degree SAN elevation angle, CL between VSAT and SAN is quite larger, which cause the throughput of NTN DL is zero, so throughput loss is NaN.

- Reference

- R4-2313890, Simulation assumption for NTN co-existence study, Samsung, RAN4#108

- R4-2315120, Co-existence study result for above 10GHz bands, CATT, RAN4#108bis