3GPP TSG-RAN WG4 Meeting #108bis                                                                                   R4-2315121

Xiamen, China, October 09 – October 13, 2023

Source: 	CATT

Title: 	Further discussion on SAN RF requirements for above 10GHz bands

Agenda item:	5.26.3

Document for:	Approval

- Introduction

In RAN4#108 meeting, WF [1] on NTN enhancements SAN RF requirement was agreed. Some remaining issues in WF need further discussion.

This contribution provides our views on the remaining issues for SAN RF requirements for above 10GHz bands.

- Discussion

OTA reference sensitivity level

As per the WF [1, 2], the agreements related to OTA reference sensitivity level are shown as below:

PkT = -174 dBm/Hz.

IM = 2 dB.

G = [50.5-58.5] dBi, [30.5-38.5] dBi, and [30.5-38.5] dBi for GEO, LEO-1200, and LEO-600.

BW for EISREFSENS_50M for Ka-band SAN is 66RB, i.e. 66*12*60*1000Hz.

-1dB SNR can be reused for EIS for Ka-band SAN.

NF for GEO and LEO SAN: 3.5 dB.

Because the EISREFSENS_50M is the worst EIS in OTA REFSENS RoAoA, so 3dB off peak margin need to be considered. Referring to TR 38.817-2, EISREFSENS_50M can be calculated by the following,

EISREFSENS_50M = PkT +10*log10(BW) +IM+SNR+NF –G+3dB                                                                  (Equation 1)

Using equation 1 with agreed parameter,

For GEO class SAN,

EISREFSENS_50M = -174+10*log10(66*12*60*1000)+2+(-1)+3.5-50.5+3=-140.2312 dBm

EISREFSENS_50M = -174+10*log10(66*12*60*1000)+2+(-1)+3.5-58.5+3=-148.2312 dBm

For LEO class SAN,

EISREFSENS_50M = -174+10*log10(66*12*60*1000)+2+(-1)+3.5-30.5+3=-120.2312 dBm

EISREFSENS_50M = -174+10*log10(66*12*60*1000)+2+(-1)+3.5-38.5+3=-128.2312 dBm

So,

For GEO class SAN, EISREFSENS_50M is an integer value in the range [-140] to [-149] dBm. The specific value is declared by the vendor.

For LEO class SAN, EISREFSENS_50M is an integer value in the range [-120] to [-129] dBm. The specific value is declared by the vendor.

Proposal 1: The declared ranges for EISREFSENS_50M are defined as follows:

For GEO class SAN, EISREFSENS_50M is an integer value in the range [-140] to [-149] dBm. The specific value is declared by the vendor.

For LEO class SAN, EISREFSENS_50M is an integer value in the range [-120] to [-129] dBm. The specific value is declared by the vendor.

For ΔFR2_REFSENS is defined for FR2 TN BS as following,

For FR2 requirements which are to be met over the OTA REFSENS RoAoA absolute requirement values are offset by the following term:

ΔFR2_REFSENS = -3 dB for the reference direction

and

ΔFR2_REFSENS = 0 dB for all other directions

For FR2-NTN SAN RX requirement, to simplify, we think that ΔFR2_REFSENS defined for FR2 TN BS can be reused for FR2-NTN.

Proposal 2: ΔFR2_REFSENS defined for FR2 TN BS can be reused for FR2-NTN SAN.

In RAN4#108 meeting, it is agreed that the G-FR2-A1-1, G-FR2-A1-2 and G-FR2-A1-3 for FR2-1 TN BS can be reused for Ka-band SAN and -1dB SNR can be reused for EIS for Ka-band SAN. Since the SCS and CBW for FR2-1 TN BS were agreed to be reused for Ka-band SAN, so the OTA reference sensitivity requirement in Table 10.3.3-1 of TS 38.104 can be reused for Ka-band NTN as following Table 2-1.

Table 2-1: FR2-NTN OTA reference sensitivity requirement

| SAN channel Bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | OTA reference sensitivity level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 50, 100, 200 | 60 | G-FR2-A1-1 | EISREFSENS_50M + ΔFR2_REFSENS |
| 50 | 120 | G-FR2-A1-2 | EISREFSENS_50M + ΔFR2_REFSENS |
| 100, 200, 400 | 120 | G-FR2-A1-3 | EISREFSENS_50M + 3 + ΔFR2_REFSENS |
| NOTE 1: EISREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full SAN channel bandwidth. NOTE 2: The declared EISREFSENS_50M shall be within the range specified above. |  |  |  |

Proposal 3: Agree the following Table 2-1 for SAN type 2-O OTA reference sensitivity requirement.

Table 2-1: FR2-NTN OTA reference sensitivity requirement

| SAN channel Bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | OTA reference sensitivity level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 50, 100, 200 | 60 | G-FR2-A1-1 | EISREFSENS_50M + ΔFR2_REFSENS |
| 50 | 120 | G-FR2-A1-2 | EISREFSENS_50M + ΔFR2_REFSENS |
| 100, 200, 400 | 120 | G-FR2-A1-3 | EISREFSENS_50M + 3 + ΔFR2_REFSENS |
| NOTE 1: EISREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full SAN channel bandwidth. NOTE 2: The declared EISREFSENS_50M shall be within the range specified above. |  |  |  |

OTA dynamic range

As per the WF [3], the remaining open issue concerning OTA dynamic range for SAN type 2-O is shown as below:

| OTA Rx requirement | Applicability notes |
| --- | --- |
| OTA Dynamic range | FFS pending further evaluation |

According to our contribution [3], the IoT simulation results for dynamic range are shown in the following table 2-2.

Table 2-2 IoT and NFR at 95% CDF

|  | XPR=20dB |  | XPR=30dB |  |
| --- | --- | --- | --- | --- |
| Cases | IoT [dB] @ 95% CDF | NFR [dB] @ 95% CDF | IoT [dB] @ 95% CDF | NFR [dB] @ 95% CDF |
| GEO | -14.65 | 0.15 | -24.65 | 0.01 |
| LEO-600 | 5.33 | 6.45 | -4.668 | 1.28 |
| LEO-1200 | -0.6906 | 2.68 | -10.7 | 0.35 |

Since the 3dB beamwidth of SAN is very smaller, so the XPR of SAN antenna would be larger than 20dB. As shown in above Table 2-2 for XPR=20dB, the NFR [dB] @ 95% CDF for GEO, LEO-600 and LEO-1200 is 0.15, 6.45 and 2.68dB, respectively, which are quite smaller, for 6.45dB NFR, it can verified by demodulation requirement. So we make the following proposal:

Proposal 4: Not to define OTA Rx dynamic range requirements for GEO, LEO-600, LEO-1200 for Ka-band.

- Conclusion

This contribution provides our general consideration on SAN RF requirement for above 10GHz bands. The following observations and proposals are provided as follows:

Proposal 1: The declared ranges for EISREFSENS_50M are defined as follows:

For GEO class SAN, EISREFSENS_50M is an integer value in the range [-140] to [-149] dBm. The specific value is declared by the vendor.

For LEO class SAN, EISREFSENS_50M is an integer value in the range [-120] to [-129] dBm. The specific value is declared by the vendor.

Proposal 2: ΔFR2_REFSENS defined for FR2 TN BS can be reused for FR2-NTN SAN.

Proposal 3: Agree the following Table 2-1 for SAN type 2-O OTA reference sensitivity requirement.

Table 2-1: FR2-NTN OTA reference sensitivity requirement

| SAN channel Bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | OTA reference sensitivity level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 50, 100, 200 | 60 | G-FR2-A1-1 | EISREFSENS_50M + ΔFR2_REFSENS |
| 50 | 120 | G-FR2-A1-2 | EISREFSENS_50M + ΔFR2_REFSENS |
| 100, 200, 400 | 120 | G-FR2-A1-3 | EISREFSENS_50M + 3 + ΔFR2_REFSENS |
| NOTE 1: EISREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full SAN channel bandwidth. NOTE 2: The declared EISREFSENS_50M shall be within the range specified above. |  |  |  |

Proposal 4: Not to define OTA Rx dynamic range requirements for GEO, LEO-600, LEO-1200 for Ka-band.

- References

- R4-2313996, WF on SAN RF, Ericsson, RAN4#108

- R4-2309770, WF on above 10 GHz SAN RF requirements, CATT, RAN4#107

- R4-2302877, WF for SAN RF requirements on above 10 GHz, Moderator (THALES), RAN4#106

- R4-2315124, Simulation results for Ka-band NTN SAN dynamic range, CATT, RAN4#108bis