3GPP TSG RAN WG4 #108BIS meeting 	 R4-2315150

Xiamen, China, 09th – 13th October, 2023

Agenda Item:	5.3.1.4.2

Source:	Meta Ireland

Title:	The remaining open issues for lower MSD capability signalling

Document for:	Approval

- Introduction

In RAN4 #106 meeting, RAN4 concluded that lower MSD is feasible [1] and RAN approved the updated objectives in the WI [2] for FR1 RF further enhancements. Especially, RAN2 WG was involved to specify the lower MSD capability signalling. In RAN4 #108 meeting, the remaining open issues will be continually discussed and addressed in 2nd phase discussion in the WI based on the approved the WF [3] as follow:

Issue 1-2-2: MSD type - Proposals - Option 1: As long as the order is within 5, the IMD could be considered in Rel-18 regardless of the IMD mechanism (which means intra-band UL CA and intra+ inter UL CA are also taken into account in Rel-18) in terms of lower MSD capability reporting. (Samsung) - Option 2: (Skyworks) - MSD types are communicated per UL configuration and victim band - 4 UL configuration types are needed as of today: - 1UL 1CC/2CC - 2UL 2CC/3CC - 8 MSD types per UL configuration is sufficient (currently it is 4 per MSD type) - UL harmonic, Harmonic mixing and cross band isolation MSD types for 1UL 1CC UL configuration type - IMD type for 1UL 2CC UL configuration type - IMD type for 2UL 2CC UL configuration type - Triple beat type for 2UL 3CC UL configuration type - Option 3: Six different low MSD types signaling for R18 + 3 in R19 (MTK) - Option 4: If RAN4 can keep the existing UL/DL configuration for the MSD test, then RAN4 don’t need to report the detail UL/DL configuration information (# of CC, # of bands of each UL/DL) and RAN4 can merge the MSD types based on the actual MSD values from UE (Meta) - WF FFS in next meeting. Issue 1-2-3: MSD order - Proposals - Option 1: previous agreements on MSD order - No need to report order for harmonic/ harmonic mixing/cross band isolation - IMD order up to 5 in Rel-18 - Option 2: Instead of MSD order, an MSD index is used (Skyworks) - For Harmonic, harmonic mixing and cross band isolation, index 0 is the worst-case MSD test point, index 1 is the optional second test point for the 1UL 1CC UL configuration type - For IMDs, index 0 correspond to the lowest IMD order (largest MSD) and index 1 to the next higher IMD order: - Within IM2 to IMD5 for the 2UL 2CC UL configuration type related IMDs - Within IMD3 to IMD9 for the 1UL 2CC UL configuration type related IMDs (but maximum 2 order possible) - IMD3 for triple beat related MSD for 2UL 3CC UL configuration type. - WF FFS in next meeting. Sub-Topic 1-5: Signaling overhead reduction - Proposals - Option 1: allow gNB query UE capability and UE only report certain capability filtered by gNB’s query information. (CMCC, vivo, Skyworks, Meta, OPPO) - Option 2: The potential singaling overhead and complexity on lower MSD capability indication shall be carefully considered in RAN4 while the solution details are being worked out (Apple) - Option 3: (MTK) - To reduce signalling overhead during connection and save UE maximum memory size for storing low-MSD information, an adaptive signalling approach that network can require UE only to report the top K largest MSD values together with its mechanism indexing and improved MSD values is proposed as option 5. - UE is also allowed to report top K’ largest low-MSD information where K’<K. For the low-MSD terms that are not responded, the MSD in existing specs applies. - Option 4: For the sake of signalling overhead reduction, further study if the victim band for a given band combination can be omitted from reporting at least for some of the MSD types (Huawei) - WF FFS in next meeting. Sub-Topic 1-6: lower MSD capability for higher order combination - Proposals - Option 1: No need to change or discuss what RAN4 agreed before, i.e., “lower MSD capability for higher order combination is inherited from lower order fallback combinations”, unless RAN4 receives an LS from RAN2 to ask RAN4 to take a specific action on the agreement. (Nokia, vivo, Skyworks, Samsung) - Option 2: Reply RAN2 with the clarification on RAN4’s assumptions on MSD requirements for higher order combinations together with other agreements in this meeting. (Huawei) - RAN4 defines the MSD requirements using band combinations consisting of 2 or 3 bands. UEs supporting high order band combinations shall meet the same MSD requirements as the fallbacks. When reporting the lower-MSD capability, the indicated MSD improvement shall be maintained regardless of the order of band combinations, and no new test points would be added. - WF FFS in next meeting. Sub-topic 1-7: Others Issue 1-7-1: other approaches for lower MSD reporting - Proposals - Option 1: Consider other approaches for lower MSD capability reporting once the primary method is finalized (Qualcomm) - Option 2 (Skyworks) - The UE reports the MSD class per UL configuration, Victim band, UL configuration type, MSD type and MSD index for the highest supported power class for this UL configuration - 0dB MSD region concept is not developed further in Release 18 and no dedicated test or signalling is designed for it - in Release 19, a set of rules are studied to derive MSD improvement compared to specified or declared lower MSD class versus (for example purposes, and not intended to be exhaustive): - Offset from the test point frequencies (DL and UL) - CBW of the UL aggressor(s) and DL CBW of the victim band - UL RB allocation - Output power. - Option 3: separate the reporting from the BC lists (Ericsson) for each affected DL band, report either - “remaining MSD” given uplink operation in given UL bands only for the cases in which the MSD is not eliminated, e.g. insufficient harmonic suppression between the an UL band and a DL band, or - “improved MSD” e.g. the cases the MSD is eliminated, e.g. that the degradation due to harmonics between an UL band and a DL band with allowed MSD in conformance tests is reduced to zero in a UE design with good isolation similarly to IDC assistance reporting. Actual values of isolation/MSD would not be indicated. - Option 4: allow UE report under which Tx power all the MSD values would be negligible/acceptable. This information could help gNB to know which UE could be allocated to MSD-victim RB since the MSD is negligible for this UE when it is at cell center with less target Tx power (CMCC) - Option 5: Single bit indication and 2-bits MSD reporting bitmap for the small MSD capability (MSD <= [3] dB) can be considered to apply the improved MSD level by the high order IMD/harmonic problems (Meta) - Option 6: For long term, MSD tests should take antenna coupling impacts into account, and evaluate the MSD in OTA conditions (OPPO) - WF FFS in next meeting.

In this paper, we propose how to solve the open issues and share our preference and propose the small MSD capability on the high order IMD/harmonic problems.

- Signalling for lower MSD capability

  - MSD types and orders for lower MSD Capability

The reported MSD types decided at last RAN4 meeting as follows and new MSD types will be further discussed in Rel-19.

The following MSD type will be defined in Rel-18

- Harmonic

- Harmonic mixing

- Cross band isolation

- IMD 2/3/4/5

- New type will be further discussed in Rel-19 i.e. IMD6/7/8/9

But, RAN4 did not have intensive discussion on the detail UL/DL configuration as mentioned in [5][6].

Here, the detail number of CC and bands configuration was already defined in specification. So, option 2 by SKW is almost the same as option 4 to keep current specifications of MSD with Rel-18 MSD types. Furthermore, similar MSD levels can be merged based on the reported MSD levels of the UE.

If RAN4 supports the high order IMD products in future, RAN4 needs to define small granularity MSD reporting capability signalling.

Hence, RAN4 agree to follow the existing configuration in Rel-18. So we believe that the additional information of detail UL/DL configuration is quite a burden to report MSD capability. So, if RAN4 keeps the UL/DL configuration in the existing specification, then we don’t need to provide the additional information.

Therefore, we propose as follow

Proposal #1: If RAN4 can keep the existing UL/DL configuration for the MSD test, then RAN4 don’t need to report the detail UL/DL configuration information (# of CC, # of bands of each UL/DL).

Proposal #2: In future release, RAN4 can support high order IMD reporting such as IMD 6/7/8/9, then RAN4 can define the small granularity of MSD reporting capability signalling.

For the MSD order, we prefer to keep the previous RAN4 agreements as follow

  - Option 1: previous agreements on MSD order

    - No need to report order for harmonic/ harmonic mixing/cross band isolation

    - IMD order up to 5 in Rel-18

Proposal #3: For the MSD order, we prefer to keep the previous RAN4 agreements as option 1 in Rel-18.

  - Signalling overhead reduction

In the last RAN4 meeting, following options as were listed in WF[3].

- Candidate option for signalling overhead reduction

  - Option 1: allow gNB query UE capability and UE only report certain capability filtered by gNB’s query information. (CMCC, vivo, Skyworks, Meta, OPPO)

  - Option 2: The potential singaling overhead and complexity on lower MSD capability indication shall be carefully considered in RAN4 while the solution details are being worked out (Apple)

  - Option 3: (MTK)

    - To reduce signalling overhead during connection and save UE maximum memory size for storing low-MSD information, an adaptive signalling approach that network can require UE only to report the top K largest MSD values together with its mechanism indexing and improved MSD values is proposed as option 5.

    - UE is also allowed to report top K’ largest low-MSD information where K’<K. For the low-MSD terms that are not responded, the MSD in existing specs applies.

  - Option 4: For the sake of signalling overhead reduction, further study if the victim band for a given band combination can be omitted from reporting at least for some of the MSD types (Huawei)

To reduce the signalling overhead, it is important point which MSD type is easier to achieve the system benefits and improved system capacity by gNB scheduling aspect. But it is quite difficult to decide the priority of the among MSD information. Hence, we prefer to allow gNB query UE capability and UE only report the certain filtered capability signalling to reduce the signalling overhead.

Also, RAN4 can merge the MSD values according to harmonic/harmonic mixing/cross band isolation/ IMD 2/3/4/5 when the UE supports the same MSD values among these MSD types to reduce signalling overhead.

Therefore, we propose as follow

Proposal #4: RAN4 can allow gNB query UE capability signalling and UE only reported the filtered capability signalling to reduce the signalling overhead. Also can merge MSD types with the same MSD values.

  - Lower MSD capability for higher order combination

This topic is related to RAN2 LS (R2-2306866/R4-2311021). RAN4 already had concrete agreements of MSD requirements for the high order CA/DC band combinations.

There are candidate options in WF [3] as follow

  - Option 1: No need to change or discuss what RAN4 agreed before, i.e., “lower MSD capability for higher order combination is inherited from lower order fallback combinations”, unless RAN4 receives an LS from RAN2 to ask RAN4 to take a specific action on the agreement. (Nokia, vivo, Skyworks, Samsung)

  - Option 2: Reply RAN2 with the clarification on RAN4’s assumptions on MSD requirements for higher order combinations together with other agreements in this meeting. (Huawei)

    - RAN4 defines the MSD requirements using band combinations consisting of 2 or 3 bands. UEs supporting high order band combinations shall meet the same MSD requirements as the fallbacks. When reporting the lower-MSD capability, the indicated MSD improvement shall be maintained regardless of the order of band combinations, and no new test points would be added.

The option 2 described the reason why RAN4 only specify MSD requirements up to 3DL CA band combinations and the other high order CA/DC band combinations should meet the same MSD requirements when CA/DC band combinations UE is fallback to lower order CA/DC band combinations. This is exactly the same as option 1 as previous agreements.

Hence, we believe that if RAN2 needs more information about the RAN4 agreement, then RAN4 can send the detail reason and agreement as mentioned in option 2. But, in this stage, RAN2 should be aligned and consistent with RAN2 and RAN4 specifications.

Proposal #5: RAN4 can keep the current agreements and RAN2 shall be aligned and consistent with RAN2 and RAN4 specifications for MSD requirements of high order CA/DC band combinations.

  - Specification impact for lower MSD capability

RAN4 agreed to specify the detail lower MSD capability classes in RAN4 specification.

Furthermore, we prefer to keep the current CA/DC UE conformance test’s burden for the lower MSD capability UE on the related conformance perspectives such as test time and the number of test cases. Also, prefer to keep the current test configuration in the existing MSD requirements and do not define the additional minimum requirements in TS38.101-1 and TS38.101-3 for lower MSD capability UE. Hence, RAN4 can add the note in the MSD exceptions in REFSENS requirements in TS38.101.

NOTE X: If the UE supports the lower MSD capability, then a reported MSD threshold shall be tested and verified as the REFSENS exception requirements with the appropriate test point in each reference sensitivity exceptions in section 7.3A.4, 7.3A.5 and 7.3A.6.

Proposal #6: RAN4 can add the Note in section 7.3A.4, 7.3A.5 and 7.3A.6 to support lower MSD capability requirements without the explicit additional MSD requirements in TS38.101-1 and TS38.101-3.

  - Other approaches for lower MSD capability reporting

For the lower MSD capability, we think that the additional small MSD capability reporting is needed and separately RAN4 can treat the small MSD capability. In our perspective, the small MSD capability (MSD <= [3] dB) reporting is also useful to apply the actual improved MSD level by high order IMD/harmonic problems (e.g., H5, IMD5) and cross band isolation issues. Even though the improved MSD level is quite small, the MSD enhancement is also important to keep the legacy UE performance in single carrier to overcome the self-interference problems.

Observation #1: A bit indication of small MSD capability (MSD <= [3] dB) can be useful to apply the improved MSD level by the high order IMD/harmonic problems (e.g. 4th/ 5th / 6th / 7th/ 8th IMD or harmonic orders) in the inter-band CA/DC band combinations.

Based on the observation, RAN4 can discuss separately between general lower MSD capability for all type MSD sources & orders and small MSD capability. The latter MSD capability is related to the specific CA/DC band combinations to overcome the high order IMD/harmonic interference problems and crossband isolations.

Based on our analysis, we propose the 2-bits MSD reporting as follows.

Table 1: MSD reporting bitmap for lower MSD capability when a bit indication of small MSD capability is enabled (MSD <= [3]dB)

| Bit map | MSD range (i.e. Thresholds) | Note |
| --- | --- | --- |
| 00 | - | Not supported the lower MSD capability. Only apply the existing MSD requirements in TS38.101-1 and TS38.101-3. |
| 01 | [1] dB | 0 ≤ Actual MSD ≤ [1] |
| 10 | [2] dB | 1 < Actual MSD ≤ [2] |
| 11 | [3] dB | 2 < Actual MSD ≤ [3] |

Proposal #7: Single bit indication and 2-bits MSD reporting bitmap for the small MSD capability (MSD <= [3] dB) can be considered to apply the improved MSD level by the high order IMD/harmonic problems.

For the 2Rx/4Rx indication, we believe that the diversity gain is the only different point between 2Rx and 4Rx architecture. So, the diversity gain of 4Rx will be considered based on 2Rx MSD requirements.

Hence, the 2Rx MSD requirement is the reference MSD values and 4Rx MSD will be decreased by the actual diversity gain.

- Conclusions

In this contribution, how to solve the open issues and share our preference and propose the small MSD capability on the high order IMD/harmonic problems.  If RAN4 considers the indication of small MSD capability (MSD <= [3] dB), the reported MSD thresholds in Table 1 are considered to apply actual improved MSD levels for the CA/DC band combination. Furthermore, in the specification impact, we prefer to keep the current test burden of the inter-band CA/DC UE for the lower MSD capability UE without the explicit additional MSD requirements. Based on our analysis for the remaining open issues, we proposed as follow

Proposal #1: If RAN4 can keep the existing UL/DL configuration for the MSD test, then RAN4 don’t need to report the detail UL/DL configuration information (# of CC, # of bands of each UL/DL).

Proposal #2: In future release, RAN4 can support high order IMD reporting such as IMD 6/7/8/9, then RAN4 can define the small granularity of MSD reporting capability signalling.

Proposal #3: For the MSD order, we prefer to keep the previous RAN4 agreements as option 1 in Rel-18.

Proposal #4: RAN4 can allow gNB query UE capability signalling and UE only reported the filtered capability signalling to reduce the signalling overhead. Also can merge MSD types with the same MSD values.

Proposal #5: RAN4 can keep the current agreements and RAN2 shall be aligned and consistent with RAN2 and RAN4 specifications for MSD requirements of high order CA/DC band combinations.

Proposal #6: RAN4 can add the Note in section 7.3A.4, 7.3A.5 and 7.3A.6 to support lower MSD capability requirements without the explicit additional MSD requirements in TS38.101-1 and TS38.101-3.

Proposal #7: Single bit indication and 2-bits MSD reporting bitmap for the small MSD capability (MSD <= [3] dB) can be considered to apply the improved MSD level by the high order IMD/harmonic problems.

Reference

[1] R4-2303695, “WF on study for lower MSD,” Huawei

[2] RP-230753, “Revised WID on Further RF requirements enhancement for NR and EN-DC in FR1,” Huawei

[3] R4-2314923, “WF on lower MSD,” Huawei

[4] TR38.881: Study on lower MSD for inter-band CA/EN-DC/DC combinations

[5] R4-2307476, “Signaling approach for low MSD,” SKW

[6] R4-2307919, “On the signalling design for lower-MSD capability,” Huawei

[7] R4-2218298, “On lower MSD capability signaling,” Meta Ireland

[8] TS38.101-1

[9] TS38.101-3