3GPP TSG-RAN WG4 Meeting # 108bis		R4-2315171

Xiamen, China, October 09 - October 13, 2023

Agenda Item:	5.13.7.3

Source: 	CMCC

Title: 			 Simulation results for ATG PUSCH demodulation

Document for:	Discussion

- Introduction

After RAN4#108 meeting, the simulation assumption is updated [1]. In this contribution, we provide our simulation results.

- Simulation results

Simulation results

Our simulation results of FR1 FDD cases are provided in the table below

| FDD 15kHz AWGN+200Hz doppler |  |  |  |  |
| --- | --- | --- | --- | --- |
| Antenna | BWP(MHz) | PRBs | MCS | SNR（70%TP） |
| 1T2R | 5 | 25 | 28（Table1） | 16.32 |
|  |  |  | 22（Table2） | 16.55 |

Our simulation results of FR1 TDD cases are provided in the table below

| TDD 30kHz AWGN+500Hz doppler typeA for 7DS2U and 30D4S6U |  |  |  |  |
| --- | --- | --- | --- | --- |
| Antenna | BWP(MHz) | PRBs | MCS | SNR（70%TP） |
| 1T2R | 10 | 24 | 28（Table1） | 16.09 |
|  |  |  | 22（Table2） | 16.94 |

- Simulation assumption

| Parameter |  | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| Frequency range |  |  | FR1 | FR1 |
| Bandwidth |  | MHz | 5 | 10 |
| Subcarrier spacing |  | kHz | 15 | 30 |
| Duplex Mode |  |  | FDD | TDD |
| TDD Slot Configuration |  |  | N/A | 7D1S2U S:6D+4G+4U |
| Channel model |  |  | AWGN+200Hz | AWGN+500Hz |
| MCS/Rank |  |  | Rank 1 64QAM MCS 28 in Table 1 256QAM MCS 22 in Table 2 | Rank 1 64QAM MCS 28 in Table 1 256QAM MCS 22 in Table 2 |
| Antenna configuration |  |  | 1T2R | 1T2R |
| Transform precoding |  |  | CP-OFDM |  |
| HARQ | Maximum number of HARQ transmissions |  | 4 |  |
|  | RV sequence |  | 0, 2, 3, 1 |  |
| DM-RS | DM-RS configuration type |  | 1 |  |
|  | DM-RS duration |  | single-symbol DM-RS |  |
|  | Additional DM-RS position |  | pos 1 |  |
|  | Number of DM-RS CDM group(s) without data |  | 2 |  |
|  | Ratio of PUSCH EPRE to DM-RS EPRE |  | -3 dB |  |
|  | DM-RS port |  | {0} |  |
|  | DM-RS sequence generation |  | NID0=0, nSCID =0 |  |
| Time domain resource assignment | PUSCH mapping type |  | A |  |
|  | Start symbol |  | 0 |  |
|  | Allocation length |  | 14 |  |
| Frequency domain resource assignment | RB assignment |  | Full applicable test bandwidth |  |
|  | Frequency hopping |  | Disabled |  |
| Code block group based PUSCH transmission |  |  | Disabled |  |
| Test metric |  |  | 70% of max throughput at target SNR. |  |

- Conclusion

In this contribution, we provide our simulation results about NR BS ATG demodulation requirements.

- Reference

[1] R4-2313875, WF for ATG demodulation requirements, CMCC