3GPP TSG-RAN WG4 Meeting # 108-bis							          R4-2315183

Xiamen, China, October 9 - October 13, 2023

Agenda item:	5.13.2.2

Source: 	CMCC

Title: 	Discussion on ATG UE Tx requirements

Document for:	Discussion

# 1. Introduction

After the RAN4 #108 meeting, a good process was reached for ATG UE Tx requirements. In the WF [1]. Some issues are left after the last meeting and this paper will focus on these issues.

# 2. Discussion

## 2.1. Minimum distance assumption

Last meeting, we still did not reach an agreement on the assumption of minimum distance between ATG BS and ATG UE, and left two proposals for this meeting.

Issue 2-1-1: Minimum distance assumption between ATG BS and ATG UE - Proposal: - Option 1: Considering the possibility of ATG serving the aircraft directly above in the future still exists, 3km could be used as the minimum distance between ATG UE from BS. - Option 2: 20km referring to coexistence study. - Option 3: Others

Our analysis has been shown in the ATG UE Rx paper. Based on the analysis the minimum distance for the non sub-array model and the sub-array model is different.

Observation 1: The minimum distance for the non sub-array model is 6.7 km, and the minimum distance for the sub-array model is 15.1 km.

20km is not the best choice for non sub-array model. In addition, considering that the possibility of ATG BS serving the aircraft directly above in the future still exists, we could use 3km as the minimum distance between ATG UE from BS.

Proposal 1: 3km could be used as the minimum distance between ATG UE from BS.

## 2.2. Minimum output power

Last meeting we reached an agreement in the WF that uses the following table as a baseline. Some parameters are still TBD, and we have analyzed some parameters in the Rx paper.

Table 1. Table of Minimum input power

|  | Omni-direction antenna | Antenna array |
| --- | --- | --- |
| Frequency | 2GHz | 4GHz |
| Distance | TBD | TBD |
| PL | TBD | TBD |
| ATG BS antenna gain | TBD | TBD |
| ATG UE antenna gain | 0dB | TBD |
| Thermal noise power density | -114dBm/MHz | -114dBm/MHz |
| ATG BS Noise figure | 5dB | 5dB |
| Minimum output power | TBD | TBD |

Pass loss:

For omni-direction antenna model (2GHz), PL = 32.5 + 20×log(3) + 20×log(2000) ≈ 108 (dB).

For antenna array model (4GHz), PL = 32.5 + 20log(3) + 20log(4000) ≈ 114 (dB).

ATG BS antenna gain: using antenna parameters for non sub-array model in TR 38.876.

BS antenna gain = 7.1(Element gain) + 10×log(8×8) = 25.16 ≈ 25.2 (dBi)

ATG UE antenna gain:

For Omni-direction antenna model, ATG UE antenna gain = 0(dBi)

For Antenna array model, ATG UE antenna gain = 5(Element gain) + 10×log(8×2) = 17.04 ≈ 17(dBi).

Thermal noise power: in the table, the thermal noise power density is -114dBm/MHz. We assume 20MHz bandwidth for 2GHz and 100MHz bandwidth for 4GHz. We can calculate the thermal noise power as below.

For omni-direction antenna model (2GHz), thermal noise power = -114 + 10×log(20) = -101(dBm)

For Antenna array model (4GHz), thermal noise power = -114 + 10×log(100) = -94(dBm)

Based on the above analysis, we could use the following model to calculate the Maximum input level.

Proposal 2: Minimum output power = PL+ Thermal noise power + BS Noise figure - BS antenna gain - UE antenna gain.

Table 2. Minimum output power for ATG UE Tx

|  | Omni-direction antenna | Antenna array |
| --- | --- | --- |
| Frequency | 2GHz | 4GHz |
| Distance | 3km | 3km |
| PL | 108dB | 114dB |
| ATG BS antenna gain | 25.2dBi | 25.2dBi |
| ATG UE antenna gain | 0dBi | 17dBi |
| Thermal noise power density | -114dBm/MHz | -114dBm/MHz |
| ATG BS Noise figure | 5dB | 5dB |
| Minimum output power | -13.2dBm | -17.2dBm |

Considering the situation that ATG BS serving the aircraft directly above, the antenna gain of the sidelobes could be smaller than the BS antenna gain listed in the table. The minimum output power could be larger than the result listed in Table 2.

Observation 2: When ATG BS serves the aircraft directly above, the antenna gain of the sidelobes could be smaller than the BS antenna gain listed in Table 2.

Proposal 3: Based on the above analysis, the parameters of the maximum input level are shown in Table 2. Considering the antenna gain of the side lobes, the minimum output power could be assumed larger than -13dBm for the omni-direction antenna model and -17dBm for the antenna array.

## 2.3. P-Max

The legacy P-Max in TS 38.331 is INTEGER (-30..33), which could represent 6 bits indication. For ATG UE, its max output power is extended to 40dBm which is larger than the current P-max upper limit. Therefore legacy P-max range is not applicable anymore. Considering the upper limit of ATG UE is 42dBm and the lower limit is -21dBm, the two bound is near the limit of ATG UE output power. So we could update the upper and lower bound of the P-max indication instead of extension.

Proposal 4: After updating the upper as 42 and lower bound as -21 for P-max, the current 6-bit P-max indication for ATG UE is enough.

# 3. Conclusions

In this contribution, ATG UE Tx requirements are discussed with the following proposals:

Observation 1: The minimum distance for the non sub-array model is 6.7 km, and the minimum distance for the sub-array model is 15.1 km.

Proposal 1: 3km could be used as the minimum distance between ATG UE from BS.

Proposal 2: Minimum output power = PL+ Thermal noise power + BS Noise figure - BS antenna gain - UE antenna gain.

Observation 2: When ATG BS serves the aircraft directly above, the antenna gain of the sidelobes could be smaller than the BS antenna gain listed in Table 2.

Proposal 3: Based on the above analysis, the parameters of the maximum input level are shown in Table 2. Considering the antenna gain of the sidelobes, the minimum output power could be assumed larger than -13dBm for the omni-direction antenna model and -17dBm for the antenna array.

Table 2. Minimum output power for ATG UE Tx

|  | Omni-direction antenna | Antenna array |
| --- | --- | --- |
| Frequency | 2GHz | 4GHz |
| Distance | 3km | 3km |
| PL | 108dB | 114dB |
| ATG BS antenna gain | 25.2dBi | 25.2dBi |
| ATG UE antenna gain | 0dBi | 17dBi |
| Thermal noise power density | -114dBm/MHz | -114dBm/MHz |
| ATG BS Noise figure | 5dB | 5dB |
| Minimum output power | -13.2dBm | -17.2dBm |

Proposal 4: After updating the upper as 42 and lower bound as -21 for P-max, the current 6-bit P-max indication for ATG UE is enough.

# 4. References

[1] R4-2314761, WF on NR_ATG_UERF_part2, Huawei