3GPP TSG-RAN WG4 Meeting #108-bis	R4-2315216

# Xiamen, China, Oct. 09-13, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | <CR#> | rev |  | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | × | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft CR on genearl aspects for MUSIM gaps and collision handling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MG_enh2-Core |  |  |  |  |  | Date: |  |  | 2023-09-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | <Rel-18> |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Add the general aspects of MUSIM and MUSIM collisions handling. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Requirements applicability The conditions when MUSIM requirements apply MUSIM gap priority preference indication Introduction Priority preference indicated by UE priority preference order Collision handling within MUSIM gaps Introduction Priority based collision handling rules within MUSIM gaps “Keep” solution based collision handling within MUSIM gaps Collision handling between MUSIM gaps and legacy gaps Introduction Collision between MUSIM gaps and gap configured via GapConfig-r17 without preConfigInd-r17 or ncsgInd-r17 (Type-2) gaps Collision between MUSIM gaps and gap configured via GapConfig (Type-1) gaps |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements of MUSIM will not be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.10.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--- Start of change 1 ---

9.1.10	MUSIM gaps

If the UE requires gap patterns for MUSIM purpose, such as cell identification and measurement, paging monitoring, SIB acquisition, and/or on-demand SI request of the target cell in the target network, then the network may provide one or more per-UE MUSIM gap pattern(s) for concurrent monitoring of all frequency layers for MUSIM via MUSIM-GapConfig [2]. The UE can be configured with no more than three periodic MUSIM gap patterns and/or one aperiodic MUSIM gap pattern for MUSIM via MUSIM-GapConfig [2]. The MUSIM gap patterns specified in Table 9.1.10-1 are applicable only for MUSIM operation.

The UE is not required to perform cell identification and measurement, paging monitoring, SIB acquisition, and/or on-demand SI request of the target cell in the target network that is outside the MUSIM gaps.

UE supporting MUSIM capability shall support the MUSIM gap patterns listed in Table 9.1.10-1 based on UE’s capability specified in TS38.306[14] and the applicability specified in Table 9.1.10-2.

UE determines MUSIM gap timing based on gap offset configuration from serving cell provided by higher layer signalling as specified in TS 38.331 [2].

Table 9.1.10-1: MUSIM Gap Pattern Configurations

| MUSIM Gap Pattern Id | MUSIM Gap Length (MGL, ms) | MUSIM Gap Repetition Period (MGRP, ms) |
| --- | --- | --- |
| 0 | 6 | 40 |
| 1 | 6 | 80 |
| 2 | 3 | 40 |
| 3 | 3 | 80 |
| 4 | 6 | 20 |
| 5 | 6 | 160 |
| 6 | 4 | 20 |
| 7 | 4 | 40 |
| 8 | 4 | 80 |
| 9 | 4 | 160 |
| 10 | 3 | 20 |
| 11 | 3 | 160 |
| 12 | 10 | 80 |
| 13 | 20 | 160 |
| 14 | 6 | 320 |
| 15 | 6 | 640 |
| 16 | 6 | 1280 |
| 17 | 6 | 2560 |
| 18 | 10 | 320 |
| 19 | 10 | 640 |
| 20 | 10 | 1280 |
| 21 | 10 | 2560 |
| 22 | 20 | 320 |
| 23 | 20 | 640 |
| 24 | 20 | 1280 |
| 25 | 20 | 2560 |
| 26 | 20 | 5120 |
| 27 | 10 | NA |
| 28 | 20 | NA |
| Note 1: Measurement gap pattern #27, #28 are the aperiodic gap pattern without MGRP. |  |  |

Table 9.1.10-2: Applicability for MUSIM Gap Pattern Configurations supported by the UE with NR standalone operation (with single carrier, NR CA configuration)

| MUSIM gap pattern configuration | Serving cell | Gap Purpose | Applicable MUSIM Gap Pattern Id |
| --- | --- | --- | --- |
| Per-UE | FR1, FR2, or | MUSIM Note1 | 0-13, 14-26, 27, 28 |
| MUSIM gap | FR1 + FR2 |  |  |
| NOTE 1: Inclusion of MUSIM procedures for per-UE MUSIM gaps only in NR single carrier, NR CA mode: MUSIM purpose which includes cell identification and measurement, paging monitoring, SIB acquisition, and/or on-demand SI request of the target cell in the target network. |  |  |  |

9.1.10.1	Introduction

This clause contains the requirements on the UE supporting MUSIM capability, requirements in this section are applicable for UE in NR SA (with CA) or NR DC operation mode.

[A slot is considered as interrupted if it is interrupted by an occasion of any of the configured MUSIM gaps following the measurement gap interruption requirements in clause 9.1.10.5, except for a dropped MUSIM occasion.]

9.1.10.2	Priorities for MUSIM gaps

Prioriy is introduced for each periodic MUSIM gap. A UE can request its preferred priority for a particular MUSIM gap via MUSIM-GapConfig-r17 [2]. Network A assigns priority levels to all configured periodic MUSIM gaps even if UE does not indicate preferred priority for one or some periodic MUSIM gaps.

Equal priority between different periodic MUSIM gaps is not allowed when UE request periodic MUSIM gaps, network A will not allocate equal priority between different periodic MUSIM gaps and between a periodic MUSIM gap and a gap configured via GapConfig-r17 without preConfigInd-r17 or ncsgInd-r17.

When receiving priorities for periodic MUSIM gap(s), the UE may receive changed priority values. If network A does not retain the relative priorities among MUSIM gaps, UE behaviour is not specified

Aperiodic MUSIM gap is always kept (not dropped) from UE perspective in case of collisions with other gaps, i.e. all gaps including MUSIM gaps and measurement gaps. The gap priority level of an aperiodic MUSIM gap is not configured by the network A.

9.1.10.3	Collisions between different MUSIM gaps

MUSIM gap occasions are considered colliding if at least one of the following conditions is met:

-	the MUSIM occasions are fully or partially overlapping in time domain, or

-	the distance between the two MUSIM occasions is equal to or smaller than 4ms.

The distance between two MUSIM gap occasions is defined as the time difference between the ending point of the first occasion and the starting point of the second occasion, where the first MUSIM gap occasion occurs earlier in time than the second MUSIM gap occasion.

In case of collision between two MUSIM gap occasions, a UE can request to use “keep solution” for collision handling via [corresponding RAN2 signalling]. If the usage of “keep solution” is granted, the UE keeps all colliding MUSIM gaps [including periodic and aperiodic MUSIM gaps] irrespective of the priority of the periodic MUSIM gaps.

When “keep solution” is not granted, the occasion of the MUSIM gap with lower priority shall be dropped. [The UE shall be able to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI in the corresponding NR serving cells in the slots that are not interrupted according to requirements in clause 9.1.10.5.]

9.1.10.4	Collisions between MUSIM gaps and measurement gaps

MUSIM gap and measurement gap occasions are considered colliding if at least one of the following conditions is met:

-	the MUSIM gap and measurement gap occasions are fully or partially overlapping in time domain, or

-	the distance between any two occasions is equal to or smaller than 4ms.

The distance between two gap occasions is defined as the time difference between the ending point of the first occasion and the starting point of the second occasion, where the first gap occasion occurs earlier in time than the second gap occasion. The gap occasion can be either a MUSIM gap occasion or a measurement gap occasion.

Collisions between MUSIM gaps and measurement gaps configured via GapConfig-r17 without preConfigInd-r17 or ncsgInd-r17 is handled based on priority.

When “keep solution” is not used, in case of collision between gap occasions, the UE shall perform network A measurements in the occasion of the gap with the highest priority if this gap is a network A measurement gap; the UE shall perform network B measurements in the occasion of the gap with the highest priority if this gap is a MUSIM gap. All other gap occasions in this collision with lower priority shall be dropped. The UE shall be able to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI in the corresponding NR serving cells in the slots that are not interrupted according to requirements in clause 9.1.10.5.

When “keep solution” is used, in case of collision between gap occasions, the UE shall perform network A measurements in the occasion of the gap with the highest priority if this gap is a network A measurement gap; [the UE shall perform network B measurements in the occasion of all MUSIM gaps in this collision when one of MUSIM gap in this collision has the highest priority among all gaps involved in this collision.] All other gap occasions in this collision with lower priority shall be dropped. The UE shall be able to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI in the corresponding NR serving cells in the slots that are not interrupted according to requirements in clause 9.1.10.5.

[The requirements of MUSIM gaps in section 9 shall not apply when priority is not assigned for a measurement gaps configured via GapConfig-r17 without preConfigInd-r17 or ncsgInd-r17.]

Collisions between MUSIM gaps and measurement gaps Gap(s) configured via GapConfig without suffix is [FFS]

9.1.10.5	Interruption of MUSIM gaps

A slot is considered as interrupted if it is interrupted by an occasion of any of the configured MUSIM gaps following the measurement gap interruption requirements in clause 9.1.2, except for a dropped MUSIM gap occasion.

--- End of change 1 ---