3GPP TSG-RAN WG4 Meeting #108bis                                        R4-2315226

Xiamen, China, October 09 – October 13, 2023

Source: 	Huawei, HiSilicon

Title: 	Sidelink on a single unlicensed spectrum

Agenda Item:	5.30.2.1.2

Document for:	Approval

Introduction

The WID revision [1] was approved in RAN#99e with the updated scope for the objective of sidelink CA operation in NR sidelink evolution.

- Specify mechanism to support NR sidelink CA operation based on LTE sidelink CA operation [RAN2, RAN1, RAN4] - Support only LTE sidelink CA features for NR (i.e., SL carrier (re-)selection, synchronization of aggregated carriers, power control for simultaneous sidelink TX, packet duplication) - The work is limited to intra-band CA for the ITS band in FR1 (Band n47). - No specific enhancements of Rel-17 sidelink features with sidelink CA support. - This feature is backwards compatible in the following regards - A Rel-16/Rel-17 UE can receive Rel-18 sidelink broadcast/groupcast transmissions with CA for the carrier on which it receives PSCCH/PSSCH and transmits the corresponding sidelink HARQ feedback (when SL-HARQ is enabled in SCI) - Only Mode 2 operation - Same subcarrier spacing (SCS) among CA carriers to avoid resource selection enhancements and AGC issues - Time resources for PSFCH are aligned among the carriers for CA - No enhancement related to SCI transmissions on PSCCH/PSSCH, PSFCH transmission, RSRP feedback, CSI feedback and congestion control compared to Rel-16 (i.e., per-carrier operation) - SL resource indication remains to be per-resource pool and per-carrier basis (no cross-carrier scheduling in SCI) - UE transmits SL HARQ feedback on the same carrier on which it receives the associated PSSCH - No consideration for limited transmission and reception capability - No primary/secondary carrier differentiation - Reuse the LTE sidelink CA design for the following aspects: - Sidelink carrier (re-)selection, synchronization of aggregated carriers, Tx power split for simultaneous sidelink transmissions, packet duplication - The CA band combination work in RAN4 is limited to intra-band contiguous CA in Rel-18. - Note: The SL CA work in Rel-18 mainly targets some V2X use cases

The TP on the MPR/A-MPR simulation assumptions for SL-U was approved in [2]. In this contribution, we provide our initial simulation evaluations on MPR for SL-U based on the agreed assumptions.

MPR evaluation on PSSCH/PSCCH

Simulation assumptions

Table 1 Evaluation assumptions on SL-U

| Frequency band | n46 |
| --- | --- |
| Bandwidth | 20/40/60/80/100MHz |
| Maximum output power | Power class 5: 20dBm |
| numerology | 15 kHz/30kHz |
| Modulation | QPSK/16QAM/64QAM/256QAM |
| Waveform | CP-OFDM for SL-U |
| PSCCH/PSSCH multiplexing |  |
| PSCCH size | 10RB*3 Symbols |
| CIM3 | 60dBc |
| PSD | 10 dBm/MHz |
| Resource allocation | Full allocation/ Interlace-based allocation/ wideband operation |
| PA calibration | PA calibrated to deliver -30dBc ACLR for a fully allocated RBs in 20MHz QPSK DFT- S-OFDM waveform at 1 dB MPR. This is based to share PA between LTE V2X and NR V2X at 5.9GHz as worst case. |

Analysis and proposal

Referring to the simulation cases listed in Table 2, the MPR value are shown in Table 3 and Table 4 for below.

Table 2 Simulation cases for SL-U

|  | case | Waveform | BW | RB Setup | SCS |
| --- | --- | --- | --- | --- | --- |
| Full Allocation Single CC | 1 | CP-OFDM | 20 | 105RB0 | 15 |
|  | 2 | CP-OFDM | 20 | 50RB0 | 30 |
|  | 3 | CP-OFDM | 40 | 216RB0 | 15 |
|  | 4 | CP-OFDM | 60 | 160RB0 | 30 |
|  | 5 | CP-OFDM | 80 | 216RB0 | 30 |
|  | 6 | CP-OFDM | 100 | 270RB0 | 30 |
| Interlaced Allocation Single CC | 7 | CP-OFDM | 20 | 1 RB0 every 10RBs | 15 |
|  | 8 | CP-OFDM | 20 | 1RB0 every 5RBs | 30 |
|  | 9 | CP-OFDM | 40 | 1RB0 every 10RBs | 15 |
|  | 10 | CP-OFDM | 60 | 1RB0 every 5RBs | 30 |
|  | 11 | CP-OFDM | 80 | 1RB0 every 5RBs | 30 |
|  | 12 | CP-OFDM | 100 | 1RB0 every 5RBs | 30 |
| Wide band operation | 13 | CP-OFDM | 40 | Bitmap 10 | 15 |
|  | 14 | CP-OFDM | 60 | Bitmap 100 | 30 |
|  | 15 | CP-OFDM | 60 | Bitmap 110 | 30 |
|  | 16 | CP-OFDM | 80 | Bitmap 1000 | 30 |
|  | 17 | CP-OFDM | 80 | Bitmap 1100 | 30 |
|  | 18 | CP-OFDM | 80 | Bitmap 1110 | 30 |
|  | 19 | CP-OFDM | 80 | Bitmap 0100 | 30 |
|  | 20 | CP-OFDM | 80 | Bitmap 0110 | 30 |
|  | 21 | CP-OFDM | 100 | Bitmap 10000 | 30 |
|  | 22 | CP-OFDM | 100 | Bitmap 11000 | 30 |
|  | 23 | CP-OFDM | 100 | Bitmap 11100 | 30 |
|  | 24 | CP-OFDM | 100 | Bitmap 11110 | 30 |
|  | 25 | CP-OFDM | 100 | Bitmap 01000 | 30 |
|  | 26 | CP-OFDM | 100 | Bitmap 01100 | 30 |
|  | 27 | CP-OFDM | 100 | Bitmap 01110 | 30 |
|  | 28 | CP-OFDM | 100 | Bitmap 00100 | 30 |

Table 3 Maximum power reduction (MPR) for SL-U with Full Allocation

| Modulation | Case: Bandwidth/SCS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 20MHz/15k | 20MHz/30k | 40MHz/15k | 60MHz/30k | 80MHz/30k | 100MHz/30k |
| QPSK | 4.0 | 3.8 | 4.1 | 4.0 | 4.0 | 4.1 |
| 16 QAM | 4.1 | 3.9 | 4.1 | 4.1 | 3.8 | 4.2 |
| 64 QAM | 6.1 | 6.1 | 6.1 | 6.1 | 6.1 | 6.1 |
| 256 QAM | 7.5 | 7.7 | 7.6 | 7.8 | 7.5 | 7.9 |

Table 4 Maximum power reduction (MPR) for SL-U with Interlaced Allocation

| Modulation | Case: Bandwidth/SCS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 20MHz/15k | 20MHz/30k | 40MHz/15k | 60MHz/30k | 80MHz/30k | 100MHz/30k |
| QPSK | 4.0 | 4.1 | 4.0 | 4.1 | 4.1 | 4.1 |
| 16 QAM | 4.1 | 4.2 | 4.2 | 4.1 | 4.2 | 4.2 |
| 64 QAM | 5.8 | 5.9 | 5.7 | 5.6 | 5.9 | 6.1 |
| 256 QAM | 7.0 | 7.3 | 7.1 | 7.4 | 7.7 | 7.4 |

Table 5 Maximum power reduction (MPR) for SL-U with Wide band operation

| Modulation | Case: Bandwidth/Bitmap pattern |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 40MHz | 60MHz |  | 80MHz |  |  | 100MHz |  |  |  |  |  |
|  | 10 | 100 | 110 | 1000 | 1110 | 0110 | 10000 | 11110 | 01000 | 01100 | 01110 | 00100 |
| QPSK | 4.0 | 3.7 | 3.9 | 3.8 | 4.0 | 3.8 | 3.9 | 4.1 | 3.0 | 3.2 | 3.9 | 3.4 |
| 16 QAM | 4.0 | 3.9 | 4.1 | 3.9 | 4.1 | 3.9 | 3.9 | 4.2 | 3.7 | 3.8 | 4.0 | 3.7 |
| 64 QAM | 5.1 | 5.1 | 5.5 | 5.2 | 5.7 | 5.7 | 5.1 | 5.8 | 5.4 | 6.0 | 5.9 | 5.5 |
| 256 QAM | 6.4 | 6.8 | 6.9 | 6.3 | 7.0 | 7.1 | 6.5 | 7.3 | 6.7 | 7.4 | 7.3 | 7.0 |

Proposal 1: The proposed MPR values for SL-U with single CC is as below

Table 3: Proposed MPR for SL-U with single CC

| Precoding | Modulation | Full | Partial |
| --- | --- | --- | --- |
| CP-OFDM | QPSK | ≤ 4.1 | ≤ 4.1 |
|  | 16 QAM | ≤ 4.2 | ≤ 4.2 |
|  | 64 QAM | ≤ 6.1 | ≤ 6.1 |
|  | 256 QAM | ≤ 8.0 | ≤ 7.7 |

MPR for PSFCH

RAN1 has already several complicated resource allocation for PSFCH on unlicensed band.

- Without the consideration of OCB requirement: RB allocation is same as the legacy style in Rel-16.

- Common interlace: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s) as shown in Figure 1. Each UE transmits HARQ-ACK information on one PRB as in R16 NR-V, and additionally transmits on the common PSFCH interlace to meet OCB requirement.

Figure 1 Illustration of common interlace for PSFCH transmission

- Dedicated interlace: each PSFCH transmission occupies 1 dedicated interlace

Figure 2 Illustration of dedicated interlace for PSFCH transmission

Among the three resource allocation methods for PSFCH on unlicensed band, common interlace may outperform. The UEs would transmit their HARQ-ACK information at different PRBs respectively as in Rel-16 NR-V, and they transmit on the same common PSFCH interlace (i.e. same PSFCH resource) so that the resource overhead is minimized. According to RAN1 discussion, the PSFCH capacity of this common interlace is close to existing R16/R17 design and it achieves highest capacity among all the alternatives. For the PSFCH allocation without consideration of OCB, it is not different from R16/17 design.

Observation 1: The common interlace allocation for PSFCH is kind of equivalent to that in existing R16/17 design. And PSFCH allocation without consideration of OCB is not different from R16/17 design.

Therefore, to reduce workload in RAN4, we propose to only consider the PSFCH resource allocation same as that in R16/17 but on unlicensed spectrum with power class 5 in one RB set in Rel-18. And the simulation assumption in section 8.1.2 in TR38.886 can be reused.

Proposal 2: Only consider the PSFCH resource allocation same as that in R16/17 but on unlicensed spectrum with power class 5 in one RB set in Rel-18.

According to TR38.786 v1.0.0, the MPR simulation assumptions for PSFCH transmission should be updated as follows

Table 6 SL-U UE’s MPR simulation assumptions for PSFCH transmission

| Items | Assumption |
| --- | --- |
| Modulation for PSSCH | QPSK |
| PSFCH | ZC sequence |
| Structure of Slot | Baseline is follow RAN1 agreements |
| RB allocation | - Power per RB is same in PSFCH for all users - Total power is 20dBm - Single RB-set will be considered based on RAN1 decision. For single RB-set, RAN4 consider common interlace RBs for PSFCH |

Conclusion

This contribution provides our initial evaluation results on MPR for SL-U with singl CC including observation and proposals:

Observation 1: The common interlace allocation for PSFCH is kind of equivalent to that in existing R16/17 design. And PSFCH allocation without consideration of OCB is not different from R16/17 design.

Proposal 1: The proposed MPR values for SL-U with single CC is as below

Table 3: Proposed MPR for SL-U with single CC

| Precoding | Modulation | Full | Partial |
| --- | --- | --- | --- |
| CP-OFDM | QPSK | ≤ 4.1 | ≤ 4.1 |
|  | 16 QAM | ≤ 4.2 | ≤ 4.2 |
|  | 64 QAM | ≤ 6.1 | ≤ 6.1 |
|  | 256 QAM | ≤ 8.0 | ≤ 7.7 |

Proposal 2: Only consider the PSFCH resource allocation same as that in R16/17 but on unlicensed spectrum with power class 5 in one RB set in Rel-18.

- References

[1] RP-230077 WID revision: NR sidelink evolution, OPPO, TSG RAN Meeting #99-e, March. 20-23, 2023

[2] R4-2310378 TP for TR 38.786 on the MPR/A-MPR simulation assumptions, Facebook Japan K.K. TSG RAN4 Meeting #107, May. 22-27, 2023