3GPP TSG-RAN WG4 Meeting #108bis                                        R4-2315227

Xiamen, China, October 09 – October 13, 2023

Source: 	Huawei, HiSilicon

Title: 	MPR evaluation for sidelink CA

Agenda Item:	5.30.2.3

Document for:	Approval

Introduction

The WID revision [1] was approved in RAN#99e with the updated scope for the objective of sidelink CA operation in NR sidelink evolution.

- Specify mechanism to support NR sidelink CA operation based on LTE sidelink CA operation [RAN2, RAN1, RAN4] - Support only LTE sidelink CA features for NR (i.e., SL carrier (re-)selection, synchronization of aggregated carriers, power control for simultaneous sidelink TX, packet duplication) - The work is limited to intra-band CA for the ITS band in FR1 (Band n47). - No specific enhancements of Rel-17 sidelink features with sidelink CA support. - This feature is backwards compatible in the following regards - A Rel-16/Rel-17 UE can receive Rel-18 sidelink broadcast/groupcast transmissions with CA for the carrier on which it receives PSCCH/PSSCH and transmits the corresponding sidelink HARQ feedback (when SL-HARQ is enabled in SCI) - Only Mode 2 operation - Same subcarrier spacing (SCS) among CA carriers to avoid resource selection enhancements and AGC issues - Time resources for PSFCH are aligned among the carriers for CA - No enhancement related to SCI transmissions on PSCCH/PSSCH, PSFCH transmission, RSRP feedback, CSI feedback and congestion control compared to Rel-16 (i.e., per-carrier operation) - SL resource indication remains to be per-resource pool and per-carrier basis (no cross-carrier scheduling in SCI) - UE transmits SL HARQ feedback on the same carrier on which it receives the associated PSSCH - No consideration for limited transmission and reception capability - No primary/secondary carrier differentiation - Reuse the LTE sidelink CA design for the following aspects: - Sidelink carrier (re-)selection, synchronization of aggregated carriers, Tx power split for simultaneous sidelink transmissions, packet duplication - The CA band combination work in RAN4 is limited to intra-band contiguous CA in Rel-18. - Note: The SL CA work in Rel-18 mainly targets some V2X use cases

In this contribution, we provide our consideration and analysis on RAN4 work for NR sidelink CA operation.

Discussion

In the approved WF R4-2310314 in RAN4#107 and WF R4-2314739 in RAN4#108, the Tx requirements for SL CA were captured

Topic #1: System parameter for SL intra-band contiguous CA <Agreement>: Capture the following bandwidth combinations in TS Sidelink CA configuration / Bandwidth combination set. Sidelink CA configuration; Sidelink CA configuration for TX; Component carriers in order of increasing carrier frequency; Maximum aggregated bandwidth [MHz]; Bandwidth combination set. Channel bandwidths for carrier [MHz]; Channel bandwidths for carrier [MHz]; Channel bandwidths for carrier [MHz]; Channel bandwidths for carrier [MHz]; Channel bandwidths for carrier [MHz]. SL_n47B; SL_n47B; 10; 10; 70; 0. 30; [30], 40. Topic #2: Tx Requirements for SL intra-band contiguous CA Issue 2-1: Methodology on Tx requirements for SL intra-band contiguous CA <Way forward >: FFS: Take the requirements for - LTE V2X intra-band contiguous CA or LTE intra-band contiguous CA - NR intra-band contiguous CA as baseline. Encourage companies to provide results based on the preferred methodology. Make decision for the suitable manner for SL CA in next meeting based on the inputs. Issue 2-6: MPR/A-MPR Simulation assumptions <Way forward >: Update the simulation assumptions to Center frequency; 5.9GHz. Bandwidth; per CC: 10/20/30/40MHz aggregated CBW: 20+40MHz/ 30+40MHz, 10MHz + 10MHz. Maximum output power for aggregated CBW; 23dBm. Numerology; 15 kHz/30kHz/60kHz. Modulation per CC; QPSK/16QAM/64QAM/256QAM. Waveform; CP-OFDM. Carrier leakage; 25dBc. IQ image; 25dBc for QPSK, 16QAM, 64QAM 34dBc for 256QAM. CIM3; 60dBc. PA calibration; PA calibrated to deliver 30dBc ACLR for a fully allocated RBs in 20MHz QPSK DFT- S-OFDM waveform at 1 dB MPR. This is based to share PA between LTE V2X and NR V2X at 5.9GHz as worst case..

In LTE, the standardization for MPR of intra-band CA is based on contiguously aggregated transmit bandwidth configuration (resource blocks). In NR, the MPR of intra-band contiguous CA was specified in Rel-16 based on both contiguous and non-contiguous RB allocations to accommodate flexible frequency resource allocation on both the carriers for large bandwidth, up to 100MHz.

In Sidelink CA, the work is limited to the ITS band in FR1 (Band n47) with channel bandwidth 10, 20, 30 and 40MHz. The aggregated CBW for sidelink CA can be 10+10MHz, 30+40 MHz, and could be 20+20 MHz, etc. Considering the utilized bandwidths are not very large, and they are closed to the bandwidth in LTE, it is more suitable to specify MPR of SL CA based on the methodology of LTE intra-band CA with contiguous RB allocation only.

Table 1 Channel bandwidths for band n47

| NR Band | SCS (kHz) | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
| n4710 | 15 |  | 10 |  | 20 |  | 30 |  | 40 |  |  |  |  |  |  |  |
|  | 30 |  | 10 |  | 20 |  | 30 |  | 40 |  |  |  |  |  |  |  |
|  | 60 |  | 10 |  | 20 |  | 30 |  | 40 |  |  |  |  |  |  |  |
| NOTE 10: For this band, UE channel bandwidths which are applicable to sidelink operation are specified in Table 5.3E.1-1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Observation 1: The channel bandwidth for n47 in SL CA is limited to 10, 20, 30 and 40MHz.

Proposal 1: To specify MPR of SL CA based on the methodology of LTE intra-band CA with contiguous RB allocation only.

The contiguous allocation rule of inner and outer for intra-band contiguous CA are as follow referring to TS38.101-1

In contiguous CA, a contiguous allocation is an inner allocation if RBStart,Low ≤ RBStart_CA ≤ RBStart,High, and NRB_alloc ≤ ceil(NRB,agg /2), where RBStart,Low = max(1, floor(NRB_alloc /2)) RBStart,High = NRB,agg – RBStart,Low – NRB,alloc, with NRB_alloc= LCRB1 ∙ 2^µ1 + LCRB2 ∙ 2^µ2, NRB,agg=NRB1∙2^µ1+ NRB2∙2^µ2. If LCRB1 =0, RBStart_CA = NRB1∙2^µ1+ RBStart2∙2^µ2, if LCRB1 > 0, RBStart_CA = RBStart1∙2^µ1. A contiguous allocation that is not an Inner contiguous allocation is an Outer contiguous allocation.

The MPR for power class 3 NR V2X with single cell and intra-band CA of contiguous RB allocation with bandwidth calls B in CP-OFDM waveform are as follows in Table 2 and Table 3 separately.

Table 2 Maximum Power Reduction (MPR) for power class 3 NR V2X

| Modulation |  | Channel bandwidth/MPR (dB) |  |
| --- | --- | --- | --- |
|  |  | Inner RB allocations | Outer RB allocations |
| CP-OFDM | QPSK | ≤ 2.5 | ≤ 4.5 |
|  | 16QAM | ≤ 2.5 | ≤ 4.5 |
|  | 64 QAM | ≤ 4.5 |  |
|  | 256 QAM | ≤ 7.0 |  |

Table 3 MPR for intra-band contiguous CA with contiguous RB allocation in power class 3

| Modulation |  | MPR for bandwidth class B(dB) |  |
| --- | --- | --- | --- |
|  |  | inner | outer |
| CP-OFDM | QPSK | 2.0 | 4.0 |
|  | 16QAM | 2.5 | 4.0 |
|  | 64QAM | 3.5 | 4.0 |
|  | 256QAM | 6.5 | 6.5 |

Referring to the updated MPR simulation assumption in the last meeting, we use the following simulation scenarios in Table 4.

Table 4 SLCA PSSCH/PSCCH MPR simulation scenarios

| Scenario | Aggregated CBW | Subcarrier spacing | Waveform | Modulation |
| --- | --- | --- | --- | --- |
| 1 | 10MHz + 10MHz | 15kHz+15kHz | CP-OFDM | QPSK/16QAM/64QAM/256QAM |
| 2 | 10MHz + 10MHz | 30kHz+30kHz | CP-OFDM | QPSK/16QAM/64QAM/256QAM |
| 3 | 30MHz + 40MHz | 30kHz+30kHz | CP-OFDM | QPSK/16QAM/64QAM/256QAM |
| 4 | 20MHz + 20MHz | 30kHz+30kHz | CP-OFDM | QPSK/16QAM/64QAM/256QAM |

The simulation results for the scenarios listed in are illustrated below, and the evaluation figures are shown in the Annex with Figure 1-Figure 4.

| Modulation | 10MHz + 10MHz/ 30kHz+30kHz |  | 10MHz + 10MHz/ 15kHz+15kHz |  | 30MHz + 40MHz / 30kHz+30kHz |  | 20MHz + 20MHz / 30kHz+30kHz |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | inner | outer | inner | outer | inner | outer | inner | outer |
| QPSK | 1.8 | 2.9 | 1.9 | 3.1 | 1.7 | 3.0 | 1.8 | 3.2 |
| 16QAM | 2.3 | 2.9 | 2.2 | 3.0 | 2.5 | 3.2 | 2.3 | 3.2 |
| 64QAM | 3.3 | 3.9 | 3.3 | 4.0 | 3.4 | 4.0 | 3.4 | 4.0 |
| 256QAM | 5.2 | 6.3 | 5.3 | 6.4 | 5.4 | 6.4 | 5.5 | 5.4 |

Proposal 2: The proposed MPR of SL CA for PSSCH/PSCCH is as Table 5, taking margin into consideration.

Table 5 MPR for SL CA with in power class 3

| Modulation |  | MPR for bandwidth class B(dB) |  |
| --- | --- | --- | --- |
|  |  | inner | outer |
| CP-OFDM | QPSK | 2.5 | 4.5 |
|  | 16QAM | 2.5 | 4.0 |
|  | 64QAM | 4.5 | 4.5 |
|  | 256QAM | 6.0 | 7.0 |

Conclusion

This contribution provides our consideration and initial analysis on RAN4 work for Rel-18 SL evolution with the following observation and proposals:

Observation 1: The channel bandwidth for n47 in SL CA is limited to 10, 20, 30 and 40MHz.

Proposal 1: To specify MPR of SL CA based on the methodology of LTE intra-band CA with contiguous RB allocation only.

Proposal 2: The proposed MPR of SL CA for PSSCH/PSCCH is as Table 5, taking margin into consideration.

Table 5 MPR for SL CA with in power class 3

| Modulation |  | MPR for bandwidth class B(dB) |  |
| --- | --- | --- | --- |
|  |  | inner | outer |
| CP-OFDM | QPSK | 2.5 | 4.5 |
|  | 16QAM | 2.5 | 4.0 |
|  | 64QAM | 4.5 | 4.5 |
|  | 256QAM | 6.0 | 7.0 |

- Annex

Scenario#1:  10MHz + 10MHz/ 30kHz+30kHz

Figure 1 Scenario#1: 10MHz + 10MHz/ 30kHz+30kHz

Figure 2 Scenario#2:  10MHz + 10MHz/ 15kHz+15kHz

Figure 3 Scenario#3:  30MHz + 40MHz/ 30kHz+30kHz

Figure 4 Scenario#4:  20MHz + 20MHz/ 30kHz+30kHz

- References

[1] RP-230077 WID revision: NR sidelink evolution, OPPO, TSG RAN Meeting #99-e, March. 20-23, 2023