# 3GPP TSG-RAN WG4 Meeting # 108bis	R4-2315249

# Xiamen, China, October 09 – October 13, 2023

Agenda item:			8.1

Source:	MediaTek Inc.

Title:	Discussion on NTN testing

Document for:	Discussion

- Introduction

One LS is received from RAN5 regarding clarifications on the R17 NTN test cases is received for the WI NR_NTN_solutions and LTE_NBIoT_eMTC_NTN_req [1]. In RAN4#108, Reply LS on clarifications for Non-Terrestrial Networks is approved [2]. In RANP#101, further conclusion has been reached [3]:

- RAN tasks RAN4 to discuss and conclude in Oct meeting on

  - The condition for testing the frequency error requirements to verify NGSO UE pre-compensation functionality for Doppler shift, which will be implemented in the RAN5 NTN specifications

    - For example, setting an artificial fixed Doppler shift, which is randomly selected out of a range of Doppler shift values decided by RAN4, as the condition in a test

  - The condition for RRM uplink timing test cases to verify NGSO UE delay pre-compensation functionality, which can be implemented in RAN5 NTN specifications

    - For example, setting an artificial fixed delay, which is randomly selected out of a range of delay values decided by RAN4, as the condition in a test

    - Note: For all the other NTN RRM test cases (except NGSO scenario in UE timing TCs), zero doppler shift and zero timing shift are assumed to be configured for RAN5 conformance testing

- Further discuss TE-emulated channel model with delay and Doppler shifts matching the satellite propagator model in future release i.e. Rel-19

- No changes to the RAN4 core requirements

- Discussion

The testing condition for NGSO to verify UE pre-compensation functionality for Doppler shift / delay has been discussed. An artificial fixed Doppler shift / delay can be set, wherein the range of values need to be decided.

Doppler shift for testing RF frequency error requirements

According to Table 4.2-2 in TR 38821-g20, as the min. elevation angle for both sat- user equipment is equal to 10 degree, the Max Doppler is 24 ppm. However, if we consider elevation angle is not smaller 30 deg (the value used in B.5 in 38.133), the Max Doppler can be X=[20] ppm.

Table 4.2-2: Reference scenario parameters (excerpted)

| Max Doppler shift (earth fixed user equipment) | 24 ppm (600km) 21ppm(1200km) |
| --- | --- |

Proposal 1: For NGSO the condition for testing the frequency error requirements, setting an artificial fixed Doppler shift, which is randomly selected out of a range of (- X ppm, X ppm), where X=[20]m  assuming the elevation angle for satellite - user equipment is not smaller than 30 deg.

Timing delay for RRM uplink timing test

Fixed delay parameter (Koffset and kmac) can refer to Table 3.36.2-1 in 38.133 for NR NTN and Table A.3.28-1 in 36.133 for IoT NTN, wherecellSpecificKoffset/ k-Offset-r17 of 8 slots are used as RTT of the service link.

Table A.3.36.2-1: Test cases for NTN specific requirements

| Parameter | Unit | Test 1 |
| --- | --- | --- |
| Interval between adjacent epoch time | s | LEO: 2.56 GEO: 10.24 |
| ntn-UlSyncValidityDuration | s | LEO: 5s GEO: 900s |
| cellSpecificKoffset | slot | LEO: 8 GEO: 256 |
| kmac | slot | Not configured |
| ta-Common |  | 0 |
| ta-CommonDrift |  | 0 |
| ta-CommonDriftVariant |  | 0 |
| ntn-PolarizationDL |  | linear |
| ntn-PolarizationUL |  | linear |
| ephemerisInfo |  |  |
| ta-Report |  | Not configured |

Table A.3.28-1: SIB31 parameters setup for Serving satellite

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference configuration for serving satellite |  | SSC.1 | SSC.2 |
| Scenario |  | GSO | NGSO |
| Interval between adjacent epoch time | s | 10.24 | 2.56 |
| ul-SyncValidityDuration-r17 | s | 900 | 5 |
| k-Offset-r17 | slot | 256 | 8 |
| k-Mac-r17 | slot | Not configured | Not configured |
| nta-Common-r17 |  | [0] | [0] |
| nta-CommonDrift-r17 |  | [0] | [0] |
| nta-CommonDriftVariation-r17 |  | [0] | [0] |
| ephemerisInfo |  |  |  |

Thus, for LEO-600km, we propose the one way of service link at elevation of 90 deg can be 2 ms (600km/c), and the one way of service link delay at elevation of 30 degrees is 4 ms (1200km /c).

Observation 1: The current value of cellSpecificKoffset/ k-Offset-r17 is 8 slots, as in Table 3.36.2-1 in 38.133 for NR NTN and Table A.3.28-1 in 36.133 for IoT NTN.

Proposal 2: For NGSO the condition for testing RRM uplink timing test cases, setting an artificial fixed delay, where one way of service link delay is ranged from 2ms to 4ms, as the max cellSpecificKoffset/ k-Offset-r17 is 8 ms and TA common is zero.

For RRM test cases for GSO, as TE-emulated channel model with delay and Doppler shifts matching the satellite propagator model can be further discussed in future release i.e. Rel-19, zero doppler shift and zero timing shift are assumed to be configured for RAN5 conformance testing for GSO tests.

Observation 2: TE-emulated channel model with delay and Doppler shifts matching the satellite propagator model can be further discussed in future release i.e. Rel-19.

Proposal 3: For RF frequency error test cases for GSO, zero doppler shift and zero timing shift are assumed to be configured for RAN5 conformance testing.

Proposal 4: For RRM test cases for GSO, zero doppler shift and zero timing shift are assumed to be configured for RAN5 conformance testing.

# Summary

Our observations and proposals are also summarized as below:

Proposal 1: For NGSO the condition for testing the frequency error requirements, setting an artificial fixed Doppler shift, which is randomly selected out of a range of (- X ppm, X ppm), where X=[20]m  assuming the elevation angle for satellite - user equipment is not smaller than 30 deg.

Observation 1: The current value of cellSpecificKoffset/ k-Offset-r17 is 8 slots, as in Table 3.36.2-1 in 38.133 for NR NTN and Table A.3.28-1 in 36.133 for IoT NTN.

Proposal 2: For NGSO the condition for testing RRM uplink timing test cases, setting an artificial fixed delay, where one way of service link delay is ranged from 2ms to 4ms, as the max cellSpecificKoffset/ k-Offset-r17 is 8 ms and TA common is zero.

Observation 2: TE-emulated channel model with delay and Doppler shifts matching the satellite propagator model can be further discussed in future release i.e. Rel-19.

Proposal 3: For RF frequency error test cases for GSO, zero doppler shift and zero timing shift are assumed to be configured for RAN5 conformance testing.

Proposal 4: For RRM test cases for GSO, zero doppler shift and zero timing shift are assumed to be configured for RAN5 conformance testing.

# Reference

- R5-233672, LS on clarifications for Non-Terrestrial Networks, RAN5#99, May. 2023.

- R4-2314001, Reply LS on clarifications for Non-Terrestrial Networks, RAN4#108, Aug. 2023.

- RP-232682, Offline discussion summary on NTN testing, RAN WG4 Chair, RAN WG5 Chair, Sept. 2023.