3GPP TSG-RAN WG4 Meeting #108-bis	R4-2315255

Xiamen, China, Oct 9 – Oct 13, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on UE Demodulation for non-colocated FR1 intra-band EN-DC/NR-CA

Agenda item:	5.11.4

Document for:	Discussion

Introduction

In RAN4#108 it was agreed to base requirements following the framework of previous CA requirements, i.e., the KPI will be based on SNR numbers [1].

Test setup and parameters Agreement: - Reuse the existing PDSCH CA demodulation requirements (e.g., TS 38.101-4 5.2A.2.1)

Additionally, we agreed on many simulation parameters, with only the MCS values and #Tx being left open:

Other parameter configurations Agreement: - Reusing test set-up from existing CA demodulation requirements for TRS and SSB configuration. - Configure 33us received time difference (RTD) between PCell and SCell. Type 2 UE NR-CA demodulation requirements. Received power difference; 25 dB. Received time difference; 33 µs. Channel model; [AWGN]. Maximum number of HARQ transmission; [1]. Signal/Noise setting; Set both Es and Noc to set SNR test points. Throughput measurement procedure; Measure both carriers. Tx antenna configuration and rank; Option1: 2Tx, rank2 for both carriers Option 2: 1Tx with rank1 for carrier with lower power 2Tx with rank2 for another carrier Option 3: rank 1 for both carriers with 1Tx. MCS table and MCS index; FFS. Test metric; 70% of the maximum throughput. MCS pair and SNR test point pair Way forward: Interested companies are encouraged to bring the simulation results in RANM4#108bis to decide the MCS index pair and corresponding SNR test point pair according to the agreed test setup and test metrics. Note the required SNR difference between the carriers with lower power and higher power should not exceed 25dB. Tx antenna and rank; MCS table; Test metric. Carrier with lower power; Option 1: 2Tx, Rank 2 Option 2: 1Tx, Rank 1; Option 1: MCS index 1 Option 2: MCS index 2; 70% of the maximum throughput. Carrier with higher power; Option 1: 2Tx, Rank 2 Option 2: 1Tx, Rank 1; Option 1: MCS index 1 Option 2: MCS index 2; 70% of the maximum throughput.

In this paper, we present Nokia’s view on the open issues of non-colocated FR1 intra-band EN-DC/NR-CA demodulation requirements, specifically focusing MCS (pair) selection and number of transmitters/layers.

RAN4 shall also decide a CR split in RAN4#108b, such that we can discuss CRs in RAN4#109.

Discussion

MCS pairs

Following the prior agreements to have requirements using two simultaneous demodulation links for PCell and SCell, with a power difference of 25dB (and 33us time difference), it has been agreed that a pair of MCS values needs to be defined to allow testing under these assumptions [1]:

MCS pair and SNR test point pair Way forward: Interested companies are encouraged to bring the simulation results in RANM4#108bis to decide the MCS index pair and corresponding SNR test point pair according to the agreed test setup and test metrics. Note the required SNR difference between the carriers with lower power and higher power should not exceed 25dB. Tx antenna and rank; MCS table; Test metric. Carrier with lower power; Option 1: 2Tx, Rank 2 Option 2: 1Tx, Rank 1; Option 1: MCS index 1 Option 2: MCS index 2; 70% of the maximum throughput. Carrier with higher power; Option 1: 2Tx, Rank 2 Option 2: 1Tx, Rank 1; Option 1: MCS index 1 Option 2: MCS index 2; 70% of the maximum throughput.

As AWGN channel model was agreed, we can quickly estimate the required SNR for each MCS (i.e., target spectral efficiency) using the Shannon bound. For 1 layer this results in the following estimations for required SNR.

| MCS Index | SNR_Shannon [dB] |  |  |  |
| --- | --- | --- | --- | --- |
|  | table 1 | table 2 | table 3 | table 4 |
| 0 | -7.5 | -7.5 | -13.8 | -7.5 |
| 1 | -6.3 | -5.2 | -12.5 | -5.2 |
| 2 | -5.2 | -2.9 | -11.5 | -0.8 |
| 3 | -3.9 | -0.8 | -10.4 | 2.5 |
| 4 | -2.9 | 1.0 | -9.5 | 4.4 |
| 5 | -1.7 | 2.5 | -8.4 | 6.3 |
| 6 | -0.8 | 3.5 | -7.5 | 7.5 |
| 7 | 0.2 | 4.4 | -6.3 | 8.6 |
| 8 | 1.0 | 5.4 | -5.2 | 9.5 |
| 9 | 1.8 | 6.3 | -3.9 | 10.5 |
| 10 | 1.8 | 6.9 | -2.9 | 11.4 |
| 11 | 2.5 | 7.5 | -1.7 | 12.4 |
| 12 | 3.5 | 8.6 | -0.8 | 13.4 |
| 13 | 4.4 | 9.5 | 0.2 | 14.3 |
| 14 | 5.4 | 10.5 | 1.0 | 15.3 |
| 15 | 6.3 | 11.4 | 1.8 | 15.9 |
| 16 | 6.9 | 12.4 | 2.5 | 16.6 |
| 17 | 6.9 | 13.4 | 3.5 | 17.7 |
| 18 | 7.5 | 14.3 | 4.4 | 18.7 |
| 19 | 8.6 | 15.3 | 5.4 | 19.7 |
| 20 | 9.5 | 15.9 | 6.3 | 20.8 |
| 21 | 10.5 | 16.6 | 6.9 | 21.5 |
| 22 | 11.4 | 17.7 | 7.5 | 22.3 |
| 23 | 12.4 | 18.7 | 8.6 | 23.7 |
| 24 | 13.4 | 19.7 | 9.5 | 25.1 |
| 25 | 14.3 | 20.8 | 10.5 | 26.5 |
| 26 | 15.3 | 21.5 | 11.4 | 27.9 |
| 27 | 15.9 | 22.3 | 12.4 |  |
| 28 | 16.6 |  | 13.4 |  |

Table 1: AWGN channel assumption-based Shannon bounds for MCS index-based SE targets (1 layer, 1Tx)

Based on this analysis, we have selected 3 potential MCS pairs with ~25dB SNR difference and presence in real deployments, which we then have verified in link level simulations (see our simulation companion document). These simulations included provisions on the residual timing offset error induced effects, due to the 33 us time difference.

Resulting in the following three potential MCS pairs:

| Pair 1 | Table 1 – MCS 2 | Table 2 – MCS 24 | SNR delta |
| --- | --- | --- | --- |
| Pair 1: 70%TPUT | -5.1 dB | 19.6 dB | 24.7 dB |
| Pair 2 | Table 2 – MCS 2 | Table 2 – MCS 26 |  |
| Pair 2: 70%TPUT | -3.2 dB | 21.9 dB | 25.1 dB |
| Pair 3 | Table 3 – MCS 4 | table 2 – MCS 20 |  |
| Pair 3: 70%TPUT | -9.6 dB | 16 dB | 25.6 dB |

Table 1: 70%TPUT SNR values AWGN 1Tx/1layer for 3MCS pairs

All these pairs offer a ~25dB SNR difference, hence power difference in the test setup with fixed noise density.

Since either using only table 2 or table1+2 is more common in practice, we me the following proposal:

RAN4 shall chose the MCS pair as either {Table 1 – MCS 2, Table 2 – MCS 24}, or {Table 2 – MCS 2, Table 2 – MCS 26}.

Tx antenna number and rank

Another open issue from RAN#108 is which Tx antenna number and rank to define requirements for.

There are two options currently being discussed [1]:

Tx antenna and rank; MCS table; Test metric. Carrier with lower power; Option 1: 2Tx, Rank 2 Option 2: 1Tx, Rank 1; Option 1: MCS index 1 Option 2: MCS index 2; 70% of the maximum throughput. Carrier with higher power; Option 1: 2Tx, Rank 2 Option 2: 1Tx, Rank 1; Option 1: MCS index 1 Option 2: MCS index 2; 70% of the maximum throughput.

In the previous section, we showed that 1TX/rank1 can achieve the agreed conditions in practical deployments, as such we should have requirements for this case.

RAN4 shall define requirements for 1Tx/rank1.

However, if found feasible, rank2 should also be included.

RAN4 shall additionally define requirements for 1Tx/rank2, if found to be feasible in practically relevant SNR operating points.

Conclusion

Within this contribution we discuss the demodulation requirements for non-colocated FR1 intra-band EN-DC/NR-CA.

Specifically, in the paper, the following Observations and Proposals were made:

MCS pairs

- RAN4 shall chose the MCS pair as either {Table 1 – MCS 2, Table 2 – MCS 24}, or {Table 2 – MCS 2, Table 2 – MCS 26}.

Tx antenna number and rank

RAN4 shall define requirements for 1Tx/rank1.

RAN4 shall additionally define requirements for 1Tx/rank2, if found to be feasible in practically relevant SNR operating points.

- References

- R4-2313876 “WF for NonCol_intraB_ENDC_NR_CA_Demod”, Ericsson, RAN4#108-e, Toulouse.