3GPP TSG-RAN WG4 Meeting #108-bis	R4-2315257

Xiamen, China, Oct 9 – Oct 13, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on Network energy saving for NR UE demodulation performance and CSI requirements

Agenda item:	5.34.5

Document for:	Discussion

Introduction

This meeting RAN4#108-bis is the first time Network energy savings for NR are treated in the demodulation performance session. According to the WID [1], there are several objectives that could potentially have demodulation performance impact:

- Specify SSB-less SCell operation for inter-band CA for FR1 and co-located cells, if found feasible by RAN4 study, where a UE measures SSB transmitted on PCell or another SCell for an SCell’s time/frequency synchronization (including downlink AGC), and L1/L3 measurements, including potential enhancement on SCell activation procedures if necessary [RAN4, RAN2] - Specify enhancement on cell DTX/DRX mechanism including the alignment of cell DTX/DRX and UE DRX in RRC_CONNECTED mode, and inter-node information exchange on cell DTX/DRX [RAN2, RAN1, RAN3] - Note: No change for SSB transmission due to cell DTX/DRX. - Note: The impact to IDLE/INACTIVE UEs due to the above enhancement should be avoided. - Specify the following techniques in spatial and power domains - Specify necessary enhancements on CSI and beam management related procedures including measurement and report, and signaling to enable efficient adaptation of spatial elements (e.g. antenna ports, active transceiver chains) [RAN1, RAN2] - Specify necessary enhancements on CSI related procedures including measurement and report, and signaling to enable efficient adaptation of power offset values between PDSCH and CSI-RS [RAN1, RAN2] - Note: Above objectives are only for UE specific channels/signals - Note: Legacy UE CSI/CSI-RS capabilities applies when considering total number of CSI reports and requirements

In this contribution we will analyze the objectives of the WID and make proposals concerning which objectives should be covered by demod and how eventual requirements shall be set up.

Discussion

SSB-less SCell operation

Specification of SSB-less SCell operation for inter-band CA for FR1 and co-located cells was discussed at length in RAN4 RRM and found to be feasible in, at least, in scenarios with maximum receive time difference (MRTD) of less than CP length between SSB in the PCell and the corresponding resource grid in the SSB-less SCell [2]:

Issue 1-2-1: RTD conditions for scenario 1 - Agreements - Define requirements for RTD ≤ CP - note: the CP corresponding to the [largest SCS across CCs or SCell SCS] - FFS if additional requirements shall be considered for RTD ≤ 260ns - Performance requirements (RRM and/or demodulation) need to be introduced for the scenario with RTD ≤ CP to guarantee proper performance on the SCell

In many standard implementations the UE uses the SSB of a serving cell for “coarse” time/frequency synchronization. Then CSI-RS for tracking (TRS) signals are used for “fine” synchronization and continuous tracking of changes. It has been previously argued that TRS fine synchronization is, at least, preferred for system configuration and SSB alone is not sufficient, albeit that it is Nokia’s understanding that TRS only synchronization can be implemented [3]:

Issue 1-2-5: TRS related - Periodic TRS/A-TRS - Option 1a: Periodic TRS is available for fine time/freq tracking purpose. (QC, Huawei, Apple) - Option 1b: A-TRS can be consider for SSB-less SCell activation (Apple, Huawei, Vivo) - Option 1c: RAN4 to clarify what type of TRS is intended for SSB-less SCell operation in Scenario 1. (Nokia) - Whether TRS is needed for SSB-less SCell activation: - Option 1: TRS monitoring is not needed for SSB-less SCell activation (i.e. 3ms SCell activation delay is achieved). (Nokia) - Option 1a: RAN4 to study whether transmitting no TRS would work or not (Ericsson) - Option 2: TRS monitoring is needed for SSB-less SCell activation (wherein >3ms SCell activation delay is expected). (Nokia)

TRS based time/frequency (T/F) synchronization algorithms have a limited capture range, i.e., a coarse synchronization is needed to set the TRS based algorithm on a sufficiently close starting point. This capture range varies from between implementation, and it is reasonable to assume that:

a) The initial coarse synchronization in the SSB-less SCell is worse than in SSB containing cells, as the PCell T/F sync needs to be re-used and it laden with the T/F difference between PCell and SCell.

b) TRS based fine synchronization algorithms with enhanced capture ranges, when compared to SSB containing cells, need to be used to maintain demodulation performance in SCells with degraded coarse synchronization.

c) Enhance capture range implementation often sacrifice accuracy for enhanced ranges.

TRS based fine synchronization algorithms with enhanced capture ranges, when compared to SSB containing cells, need to be used to maintain demodulation performance in SCells with degraded coarse synchronization.

Hence, we assume that there is implementation dependent demodulation performance impact in SSB-less SCells. As such we need to define requirements for this case.

RAN4 shall define demodulation minimum performance requirements for SSB-less SCell operation for inter-band CA for FR1 and co-located cells.

Following the deployment envisioned for SSB-less SCell operation, the demod requirement setup shall be as follows:

RAN4 shall configure a PCell cell with SSB and TRS, and an SCell with only TRS (i.e., two RMCs are needed at the same time). A time offset, close to the CP length, shall be applied between PCell and SCell. Demodulation performance shall be measured on the SCell only.

Enhancement on cell DTX/DRX mechanism

The next objective in the WID concerns enhancements on cell DTX/DRX mechanisms [R4-2314381, WF on RRM requirements for NR network energy saving, RAN4#108]:

- Specify enhancement on cell DTX/DRX mechanism including the alignment of cell DTX/DRX and UE DRX in RRC_CONNECTED mode, and inter-node information exchange on cell DTX/DRX [RAN2, RAN1, RAN3] - Note: No change for SSB transmission due to cell DTX/DRX. - Note: The impact to IDLE/INACTIVE UEs due to the above enhancement should be avoided.

In the past, DTX/DRX cycles were not taken into account for demodulation performance requirements, as DTX/DRX is known to the receiver and demodulation is not enabled during these periods.

The enhancements for DTX/DRX can be summarized as introducing cell based DTX/DRX via RRC or group-common DCI (as opposed to per UE DRX via RRC or UE specific DCI). I.e., only the (de-)activation signaling is changed from a receiver demodulation pov, not the implementation of the demodulation algorithms and/or performance.

As such we propose:

RAN4 shall not define demodulation minimum performance requirements for enhancement on cell DTX/DRX mechanisms.

Spatial and power domain techniques

The final objectives concern spatial and power domain techniques, which mainly targets providing the network with assistance information, in forms of CSIs provided by the UE(s), in order to assist the network in deciding of a suitable spatial pattern and/or power level for DL data transmissions [2][R4-2314381, WF on RRM requirements for NR network energy saving, RAN4#108]:

- Specify the following techniques in spatial and power domains - Specify necessary enhancements on CSI and beam management related procedures including measurement and report, and signaling to enable efficient adaptation of spatial elements (e.g. antenna ports, active transceiver chains) [RAN1, RAN2] - Specify necessary enhancements on CSI related procedures including measurement and report, and signaling to enable efficient adaptation of power offset values between PDSCH and CSI-RS [RAN1, RAN2] - Note: Above objectives are only for UE specific channels/signals - Note: Legacy UE CSI/CSI-RS capabilities applies when considering total number of CSI reports and requirements

In summary, the CSI enhancements are threefold:

a) Multi-CSI reporting for spatial adaptation

Enable one CSI report configuration to include a list of sub-configurations, each of which corresponding to a spatial pattern (at the gNB). This would then allow the gNB to indicate/trigger a UE to provide CSIs (e.g., CQI, PMI, RI, CRI) for all or a subset of those sub-configurations and thus spatial patterns. Two types of spatial adaptation patterns were agreed, namely Type 1 and Type 2, as can be seen from the agreement copied below. With Type 1, each sub-configuration would correspond to/include a (different) subset of CSI-RS antenna ports of a CSI-RS resource(s), where the resource with a large number of antenna ports is transmitted and UE is requested to measure CSIs for different set/subset of antenna ports from this transmission. Whereas with Type 2, each sub-configuration would correspond to/include a CSI-RS resource(s) where the same number of antenna ports is assumed for all sub-configurations/resources.

Agreement (RAN1#112 Athens) For the purpose of further discussions in RAN1 on NES spatial domain adaptations, consider the following cases - Type 1: all antenna elements associated to a logical antenna port is disabled/enabled - Type 2: part/subset of antenna elements associated to a logical antenna port is disabled/enabled Agreement (RAN1#112bis-e) At least support A2-2, i.e., one CSI report configuration contains multiple CSI report sub-configurations where each sub-configuration corresponds to one spatial adaptation pattern.​ - FFS: impact on CSI processing requirement

b) Multi-CSI reporting for power adaptation

Enable one CSI report configuration to include a list of sub-configurations, each of which basically corresponding to a power offset which then translates to a specific power control offset (i.e., which is the PDSCH EPRE to NZP CSI-RS EPRE). This would then allow the gNB to indicate/trigger the UE to provide CSIs for all or a subset of those sub-configurations and thus power levels.

Agreement (RAN1#114 Toulouse) For power domain adaptation only, all CSI-RS resource(s) (which can be one or more) in the CSI-RS resource set for channel measurement are associated with each sub-configuration provided in a CSI report configuration - Each sub-configuration contains an offset value (e.g. X) that is commonly applied to all the resources within the resource set. For a CSI-RS resource in CSI resource configuration, the PDSCH to CSI-RS EPRE offset (e.g. Y) for CSI calculation is determined based on powerControlOffset (e.g. Z) value in CSI resource configuration and the offset value configured in CSI sub-configuration in the report configuration. - Only legacy values are applicable for the resulted power control offset values - It is expected that the sub-configuration leads to a value no larger than power control offset value provided in CSI resource configuration

c) Joint operation of power and spatial domain adaptations, which is enabled by jointly considering the features in a) and b).

Agreement (RAN1#113 Incheon) Joint operation of SD and PD adaptation is supported.

RAN1 and RAN2 have not yet fully completed the sub-configuration specification, but discussions point towards re-using the legacy reports and meanings, just >= one CSI hypothesis, for >= one sub-configuration, per one reportConfigId/CSI report is possible. Each of these sub-configurations will have a separate ID.

Note for SD that in type 1 there are usually more than one hypothesis per CSI-RS transmission, while for type 2 there are different CSI-RS transmissions with usually one hypothesis each.

As such we don’t see a demodulation performance impact, as the new sub-configuration each have a legacy interpretation, and so the legacy reporting requirements shall apply for each.

The new sub-configuration each have a legacy interpretation, and so the legacy reporting requirements shall apply for each.

Which leads us to the following proposals.

Concerning (a) “multi-CSI reporting for spatial adaptation”, i.e., different antenna port values:

RAN4 shall not define demodulation minimum performance requirements for multi-CSI reporting for spatial adaptation.

Concerning (b) “multi-CSI reporting for power adaptation”, i.e., different CSI-RS power offset values

RAN4 shall not define demodulation minimum performance requirements for multi-CSI reporting for power adaptation.

Conclusion

Within this contribution we discuss the demodulation requirements for Network energy savings for NR.

Specifically, in the paper, the following Observations and Proposals were made:

SSB-less SCell operation

- TRS based fine synchronization algorithms with enhanced capture ranges, when compared to SSB containing cells, need to be used to maintain demodulation performance in SCells with degraded coarse synchronization.

- RAN4 shall define demodulation minimum performance requirements for SSB-less SCell operation for inter-band CA for FR1 and co-located cells.

RAN4 shall configure a PCell cell with SSB and TRS, and an SCell with only TRS (i.e., two RMCs are needed at the same time). A time offset, close to the CP length, shall be applied between PCell and SCell. Demodulation performance shall be measured on the SCell only.

Enhancement on cell DTX/DRX mechanism

RAN4 shall not define demodulation minimum performance requirements for enhancement on cell DTX/DRX mechanisms.

Spatial and power domain techniques

RAN4 shall not define demodulation minimum performance requirements for multi-CSI reporting for spatial adaptation.

RAN4 shall not define demodulation minimum performance requirements for multi-CSI reporting for power adaptation.

- References

- R4-230566, WID revision: Network energy saving NR, Huawei, RAN #99, March 22 – 23, 2023.

- R4-2314381, WF on RRM requirements for NR network energy saving, Huawei, HiSilicon, RAN4 #108, August 21-25, 2023.

- R4-2314344, Ad-hoc minutes for NES, Huawei, RAN4 #108, August 21-25, 2023.