3GPP TSG-RAN WG4 Meeting # 108bis		R4-2315263

Xiamen, China, 9 – 13 October 2023

Agenda Item:	4.34.4

Source: 	Nokia, Nokia Shanghai Bell

Title: 	BS RF specification impact for introduction of NR bands n31 and n72

Document for:	Approval

# 1.	Introduction

The WID on New WID on introduction of NR bands n31 and n72 was approved at TSG RAN #101 [1]. One of the objectives of this WI is to evaluate potential impacts of NR based on LTE bands 31 and 72, and if any impact on performance or any changes w.r.t. existing requirements of bands 31 and 72 are found, address those during the work.

This contribution provides the necessary changes of the BS RF requirements in TS 38.104 [2] for introduction of NR bands n31 and n72, based on the existing requirements of LTE bands 31 and 72 in TS 36.104 [3].

# 2.	Specification Impact

Table 1 provides all BS RF Tx and Rx requirements one by one and analyzes which requirements need modification with introduction of NR bands n31 and n72, and presents as base the existing requirements of LTE bands 31 and 72.

Table 1: BS RF specification impact

| NR BS RF Tx/Rx requirement | Current LTE specification for bands 31 and 72 | Proposal for NR bands n31 and n72 |
| --- | --- | --- |
| 5.2 Operating bands | Band 31 (UL 452.5 – 457.5, DL 462.5 – 467.5 MHz) Band 72 (UL 451 – 456, DL 461 – 466 MHz) | Band n31 (UL 452.5 – 457.5, DL 462.5 – 467.5 MHz) Band n72 (UL 451 – 456, DL 461 – 466 MHz) |
| 5.3 BS channel bandwidth | 1.4, 3 and 5 MHz channel bandwidth | 3 and 5 MHz channel bandwidth in Table 5.3.5-1 |
| 5.3A BS channel bandwidth for CA | Operating band agnostic | No specification impact |
| 5.4.1 Channel spacing | Operating band agnostic | No specification impact |
| 5.4.2 Channel raster | Band 31 (UL 27760 – 27809, DL 9870 – 9919) Band 72 (UL 133472-133521, DL 68936 - 68985) | Band n31 (UL 90500 – <20> – 91500, DL 92500 – <20> – 93500) Band n72 (UL 90200 – <20> – 91200, DL 92200 – <20> – 93200) |
| 5.4.3 Synchronization raster | Not applicable | Band 31 (5MHz ChBW 1161 – <1> – 1162, 3MHz ChBW 28955 – <1> – 28967) Band 72 (5MHz ChBW 1157 – <1> – 1159, 3MHz ChBW 28947 – <1> – 28959) |
| 6.2 Base station output power | Minimum requirements are operating band agnostic, no additional regional requirement for bands 31 and 72 | No specification impact |
| 6.3.2 RE power control dynamic range | Operating band agnostic | No specification impact |
| 6.3.3 Total power dynamic range | Operating band agnostic | No specification impact |
| 6.3.4 NB-IoT RB power dynamic range for NB-IoT operation in NR in-band | Operating band agnostic | No specification impact |
| 6.4 Transmit ON/OFF power | Operating band agnostic | No specification impact |
| 6.5 Transmitted signal quality | Operating band agnostic | No specification impact |
| 6.6.2 Occupied bandwidth | Operating band agnostic | No specification impact |
| 6.6.3 Adjacent Channel Leakage Power Ratio | Operating band agnostic | No specification impact |
| 6.6.4 Operating band unwanted emissions | Bands 31 and 72 are included in the band lists for Wide Area Category A and Category B (option 1) | Add Bands 31 and 72 in the band lists for Wide Area Category A and Category B (option 1) in clause 6.6.4.2.1 and clause 6.6.4.2.2.2.1 |
| 6.6.5 Transmitter spurious emissions | Minimum requirements are operating band agnostic, no coexistence and co-location requirements are operating band specific | Add coexistence and co-location requirements in Table 6.6.5.2.3-1 and Table 6.6.5.2.4-1 with the same requirements as LTE bands 31 and 72 |
| 6.7 Transmitter intermodulation | Operating band agnostic | No specification impact |
| 7.2 Reference sensitivity level | Operating band agnostic | No specification impact |
| 7.3 Dynamic range | Operating band agnostic | No specification impact |
| 7.4.1 Adjacent Channel Selectivity (ACS) | Operating band agnostic | No specification impact |
| 7.4.2 In-band blocking | Operating band agnostic | No specification impact |
| 7.5 Out-of-band blocking | Operating band agnostic | No specification impact |
| 7.6 Receiver spurious emissions | Operating band agnostic | No specification impact |
| 7.7 Receiver intermodulation | Operating band agnostic | No specification impact |
| 7.8 In-channel selectivity | Operating band agnostic | No specification impact |

# 4.	Conclusion

This contribution provides the necessary changes of the BS RF requirements in TS 38.104 [2] for introduction of NR bands n31 and n72, based on the existing requirements of LTE bands 31 and 72 in TS 36.104 [3]. It is proposed to approve the proposals for NR bands n31 and n72 in Table 1 to guide the specification changes.

# References

[1]	RP-232691, “New WID on introduction of NR bands n31 and n72”, Nokia.

[2]	3GPP TS 38.104 v18.3.0: “NR; Base Station (BS) radio transmission and reception”.

[3]	3GPP TS 36.104 v18.3.0: “Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception”.