3GPP TSG-RAN WG4 Meeting#108-bis	R4-2315347

Xiamen, China 9th Oct – 13th Oct, 2023

Source: 	Qualcomm Incorporated

Title: 	Discussion on the demodulation performance requirements for enhanced NR dynamic spectrum sharing

Agenda item:	5.36.2

Document for:	Discussion

# 1. Introduction

The enhancement of NR dynamic spectrum sharing (DSS) will be discussed during RAN4#108-bis. Current specification does not allow more than one NR PDCCH symbols within the first 3 symbols of a slot in cells with four LTE CRS ports as well as overlap between NR PDCCH and LTE CRS symbols. As a result, NR PDCCH becomes the bottleneck for DSS operation with increasing NR traffic. The goal of the enhancement for NR DSS in Rel-18 would be to allow NR PDCCH reception to be overlapped with LTE CRS. In the process, it would require considering multiple LTE CRS rate-matching pattern [1]. In this paper, we share our views on these aspects.

2. Candidate Solutions

In our knowledge, there were a couple of solutions that RAN1 agreed to be considered for Rel-18 DSS enhancements. They can be listed as follows:

- Option 1: DMRS on clean symbol is used for PDCCH channel estimation (DMRS on CRS symbol are kept unchanged).

- Option 2: UE processes PDCCH as if there is no LTE CRS

NR PDCCH DMRS

NR PDCCH

LTE CRS

Fig. 1: An example of option 1 where NR PDCCH is overlapped with LTE CRS symbols.

Fig. 2: An example of option 2 assuming no LTE CRS

We note that option 1 allows PDCCH channel estimation on the clean DMRS symbol only while with option 2 UE assumes there is no LTE CRS. Having assumed no LTE CRS, as expected, the performance of option 2 would be far worse compared to option 1. Therefore, we prefer defining performance requirements with option 1 only.

Observation 1: Performance of option 2 would be much worse compared to option 1.

Proposal 1: Define NR PDCCH performance requirement for option 1 only with PDCCH channel estimation on the clean DMRS symbol.

Proposal 2: Assume 2-symbol coreset for NR PDCCH configuration with option 1.

We also note there is no impact to the NR PDSCH performance compared to Rel-17 NR DSS, where interference from LTE CRS to NR PDSCH was considered. Therefore, we think that there is no need to define NR PDSCH performance requirement in Rel-18 DSS.

Proposal 3: Do not define NR PDSCH performance requirement in Rel-18 DSS.

3. Conclusions

In this paper, we share our views on the enhancement of the Rel-18 DSS. The following have been proposed.

Observation 1: Performance of option 2 would be much worse compared to option 1.

Proposal 1: Define NR PDCCH performance requirement for option 1 only with PDCCH channel estimation on the clean DMRS symbol.

Proposal 2: Assume 2-symbol coreset for NR PDCCH configuration with option 1.

Proposal 3: Do not define NR PDSCH performance requirement in Rel-18 DSS.

4. References

[1] R4-221622, “Revised WID on Enhancement of NR Dynamic spectrum sharing (DSS),” 3GPP TSG RAN WG Meeting #96, Budapest, Hungary, June 6 – 9, 2022.