3GPP TSG-RAN WG4 Meeting#108-bis	R4-2315352

Xiamen, China 9th Oct – 13th Oct, 2023

Source: 	Qualcomm Incorporated

Title: 	Simulation Results on PDSCH Performance Requirements for FR2 Multi-Rx

Agenda item:	5.7.4.2

Document for:	Discussion

# 1. Introduction

Based on the agree correlation model during RAN4#107 and tentative simulation assumptions discussed in RAN4#108, in this paper we share our simulation results.

2. Simulation Results for PDSCH Requirements

We consider the following simulation assumptions for these initial results.

| Transmission Scheme | Multi-DCI fully overlapping |
| --- | --- |
| Correlation model | FR2 spatial correlation model [1] |
| SCS/BW | 120KHz, 100MHz |
| Layer combination | 2L+2L, 1L+1L |
| MCS | 11, 13 (2L+2L), 17 (1L+1L) |
| Channel model | TDLA-30ns-75Hz (statistically independent) |
| Precoding | Random across TRPs |
| DMRS port index | 7 and 8 for TRP1 and TRP2, respectively |
| No. of PDSCH DMRS symbols | 2 (1+1) |
| No. of PDSCH symbols | 10 |
| Cross-polarization coefficient | 0.0625 |
| Carrier frequency | 28GHz |

Below we share our results at -12dB and -15dB cross-talk power values for both independent per TRP and joint processing in the context of mTRP fully overlapping case at 70% peak throughput.

Table 1: PDSCH performance results with joint decoding for mDCI fully overlapping case

| Joint Processing (2L+2L) |  |  |  |
| --- | --- | --- | --- |
| -12dB |  | -15dB |  |
| MCS13 | MCS11 | MCS13 | MCS11 |
| 11.55 | 9.66 | 11.5 | 9.68 |

| Joint Processing (1L+1L) |  |
| --- | --- |
| -12dB | -15dB |
| MCS17 | MCS17 |
| 9.8 | 9.8 |

Table 2: PDSCH performance results with independent per TRP processing for mDCI fully overlapping case

| Independent Per TRP Processing (2L+2L) |  |  |  |
| --- | --- | --- | --- |
| -12dB |  | -15dB |  |
| MCS13 | MCS11 | MCS13 | MCS11 |
| N/A* | 12 | 13.25 | 11.1 |

*Peak throughput can’t be reached for MCS13 at -12 dB cross-talk

| Independent Per TRP Processing (1L+1L) |  |
| --- | --- |
| -12dB | -15dB |
| MCS17 | MCS17 |
| 10 | 11.77 |

3. Conclusions

In this paper, we share our simulation results for PDSCH demodulation requirements in the context of FR2 multi-Rx mDCI scenario.

4. References

[1] R4-2309824, “WF on UE demodulation and CSI requirements for FR2 multi-Rx DL reception,” 3GPP TSG RAN WG4 Meeting #107, Incheon, KR, May 22 – 26, 2023.

[2] R4-2313977, “WF on UE demodulation and CSI requirements for FR2 multi-Rx DL reception,” 3GPP TSG RAN WG4 Meeting #108, Toulouse, FR, Aug 21 – 25, 2023.