3GPP RAN WG4 Meeting #108bis		R4-2315364

Xiamen, China, October 9 – 13th, 2023

Agenda item:	4.31.3

Source:	Apple, Globalstar

Title:	RF requirements for the NTN L-/S-band

WI/SI:	NR_NTN_LSband

Release:	Rel-18

Document for:	Discussion

# 1	Introduction

After the RAN#86 meeting, the Rel-17 WI was approved that aimed at enabling the 5G/NR radio access technology for non-terrestrial satellite deployments [1]. One of the WI objectives was to define and introduce the corresponding bands into the 3GPP specifications. The outcome of these discussions was two NTN bands – L-band and S-band – which were added as bands n255 and n256, respectively, to the Rel-17 core specifications. However, already that time it was noted that there exist other satellite deployments that use mixed L-/S-band pairing, whereupon the UL part is one the L-band, while the DL part is one the S-band. To limit the overall workload in Rel-17, it was decided by RAN WG4 to focus on L- and S-bands with the common understanding that other potential NTN bands should follow the "normal" process of submitting a new spectrum WI. So, during the RAN#98 meeting a new Rel-18 NTN spectrum WI was agreed with the objective of adding support for a new NTN band where the UL is on the L-band and the DL is on the S-band [2].

During the previous RAN WG4 meetings several agreements for the RF requirements were made, such as the maximum Tx power, basic MPR requirements, etc. In this paper we present our further proposals for the RF requirements of the new NTN band.

# 2	RF requirements for the NTN L-/S-band

## 2.1	TX requirements

### 2.1.1	NS values

As already presented during previous meeting and captured in TR 38.741, there are different regulations for this NTN band, for which several NS flags are needed summary for which is presented in Table 2.1.1-1 below. As can be seen from the table, the main differentiating factor is FCC and ETSI regulatory domains. During the previous RAN4#108 meeting a new NS_03N flag was already agreed for the FCC regulatory domain. And even though FCC and ETSI out-of-band emission requirements are similar, they are not the same because ETSI regulations have stricter in-band emission requirements. Furthermore, we need two additional NS flags to support ETSI regulations for the following reasons:

a)	ETSI regulations have different in-band emission masks depending on the operating sub-range, so two NS flags are needed to differentiate between them.

b)	NS_04N applicable range is limited to 1610-1618.25MHz according to the ETSI requirements. And since the total spectrum block size is 8.25MHz, it can accommodate only 5MHz standard NR channel, i.e. the applicable NR channel size is 5MHz.

c)	NS_05N is for the ETSI range 1618.25-1626.5MHz, which can also accommodate only 5MHz standard NR channel. However, ETSI document also clarifies that the same in-band emission requirements will apply if the channel is partially configured in that range. It means that any 10 and 15MHz channel configured in 1610-1626.5MHz range will inevitably fall into the 1618.25-1626.5MHz range and thus they will follow NS_04N requirements.

Table 2.1.1-1: Summary of the proposed NS values.

| Regulatory domain | NS value | Applicable range (MHz) | Applicable channel (MHz) |
| --- | --- | --- | --- |
| FCC | NS_03N | 1610-1626.5 | 5, 10, 15 |
| ETSI | NS_04N | 1610-1618.25 | 5 |
|  | NS_05N | 1618.25-1626.5 | 5 |
|  |  | 1610-1626.5 | 10, 15 |

Proposal 1:	Specify two NS flags, NS_04N and NS_05N, to support ETSI regulations for the NTN L-/S-band.

### 2.1.2	A-MPR

During the previous RAN4#108 meeting we brought a set of comprehensive power back-off simulations for 5, 10 and 15MHz channels covering both FCC and ETSI regulations [6]. As summarised that time, edge channels will mostly require some power back-off to meet strict regulatory requirements for the out-of-band emissions. On the contrary to it, channels residing in the middle of the band will not require any power back-off. What remains unanswered last time is the minimum distance from the lower/upper edge of the band at which the configured channel will have no or minimal power back-off at least for small RB allocations.

Thus, in addition to the configurations presented at the RAN4#108 meeting, there are two additional cases for the 5MHz channel:

-	NR 5MHz channel at the following centre frequencies:

a)	Fc=1613.9MHz (1.4MHz offset from the band lower edge)

b)	Fc=1622.4MHz (1.6MHz offset from the band upper edge)

As with the previous power back-off simulations, all simulations are run with the same common parameters as follows:

-	Tx power: PC3 +23dBm

-	LO placement: always in the centre of the carrier

-	Regulations: ETSI and FCC

For the sake of completeness, Figure 2.1.2-1 below shows all configuration cases for the 5, 10 and 15MHz channels with the corresponding centre frequencies used for the power back-off simulations.

Figure 2.1.2-1: Considered channels for power back-off simulations.

Appendix A1 contains the corresponding power back-off figures for additional cases of the 5MHz channel, short summary for which is presented below:

-	Referring to Figure A1-1, if the 5MHz channel is shifted by 1.4MHz from the band lower edge, then small RB allocations will not require any power back-off at all. Some of the channel edge allocations might need power back-off, but it does not exceed baseline MPR. Only large, full channel, allocations will have power back-off exceeding A-MPR, but it is not likely that the NTN base station will allocate all RBs to a UE because it will result in a very low PSD.

-	If the 5MHz channel is shifted by 1.6MHz from the band upper edge, then required power back-off values also go down. As can be seen from Figure A1-2, irrespective of the fact whether the network allocates small or large RB blocks, required power back-off stays within the allowed MPR limits.

Proposal 2:	Capture additional power back-off simulation results in the NTN L-/S-band TR.

If an operator decides to ensure lower power back-off by configuring the 5MHz channel with offsets from the lower/upper band edges, then to avoid wasting 1.4/1.6MHz at the edges of the band it is possible to deploy other technologies, such as LTE CatM and NB-IOT. Figure 2.1.2-2 shows an exemplary allocation of channels, where an operator may consider one LTE CatM channel or seven NB-IOT channels at the lower edge of the band. For the upper edge, either eight NB-IOT channels can be configured, or one LTE CatM channel can be combined with one NB-IOT channel.

Figure 2.1.2-2: Exemplary allocations of channels at the edges of the band.

### 2.1.3	Summarised A-MPR values

In this section we provide a summary of A-MPR values needed for every channel size under the FCC and ETSI regulations accounting for the power back-off values presented during the RAN4#108 and RAN4#108bis meetings.

For the 5MHz channel, Figure 2.1.3-1 below presents a graphical summary of the required power back-off values for different 5MHz channels:

-	The 5MHz channel right at the lower edge of the band requires power back-off even for small RB allocations irrespective of the fact whether it is FCC or ETSI regulations. If the channel is shifted by 1.4MHz, then at least small RB allocations will not require power back-off exceeding baseline MPR requirements. Shifting the 5MHz channel beyond the 1615.7MHz center frequency results in no A-MPR at all.

-	As for the upper edge of the band, the 5MHz channel under the FCC regulations will not require any additional A-MPR requirements even if it is right at the edge because there are no out-of-band emission requirements for the upper frequencies. However, with the ETSI regulations we will need to provide offset of 1.6MHz, upon which we will not be exceeding baseline MPR requirements.

Figure 2.1.3-1: Summary of required power back-off regions for the 5MHz channel for FCC (top) and ETSI (bottom) regulations.

For the 10MHz channel:

-	For the channel right at the lower edge of the band, A-MPR might be needed even for small RB allocations at the channel edges because of the strict out-of-band emission requirements. Shifting the channel farther away from the lower edge basically improves A-MPR with the switching point at the center frequency of around 1620.1MHz (i.e. 1.4MHz offset from the upper edge).

-	For the channel right at the upper edge of the band, A-MPR is generally not needed for small allocations under the FCC regulations. However, with the ETSI regulations some A-MPR still might be required because of the out-of-band emission requirements for upper frequencies.

For the 15MHz channel, more or less same A-MPR is needed irrespective of the fact where exactly the channel is configured and whether it is under the FCC or ETSI regulations.

The corresponding A-MPR values are summarised in Table 2.1.3-1, 2.1.3-2 and 2.1.3-3 below for the corresponding NS values.

Proposal 3:	Capture proposed A-MPR values for NS_03, NS_04 and NS_05.

Table 2.1.3-1: A-MPR values for NS_03 (FCC)

| Channel BW | Carrier Center Frequency | RB_start*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 5MHz | 1612.5 <= fc < 1613.9 | <= 0.36 | <= 0.36 | A1 |
|  |  |  | >= 2.88 | A2 |
|  | 1613.9 <= fc < 1615.7 |  | >= 3.24 | A3 |
| 10MHz | 1615 <= fc < 1620.1 | <= 1.8 | <= 5.04 | A4 |
|  |  | <= 1.8 | > 5.04 | A5 |
|  |  | > 7.2 | > 0 | A6 |
|  |  | > 1.8 | >= 2.88 | A2 |
|  | 1620.1 <= fc < 1621.5 |  | <= 6.48 | A6 |
|  |  | <= 0.36 | <= 0.36 | A1 |
|  | fc = 1621.5 |  | >= 7.2 | A1 |
| 15MHz | all | <= 3.6 | <= 5.04 | A4 |
|  |  | <= 3.6 | > 5.04 | A5 |
|  |  | > 10.44 |  | A6 |
|  |  | > 3.6 | >= 4.32 | A2 |

|  | Modulation | A1 | A2 | A3 | A4 | A5 | A6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK | 2.5 | 5.0 | 1.5 | 6.5 | 12.0 | 3.5 |
|  | QPSK | 2.5 | 5.0 | 1.5 | 6.5 | 12.0 | 3.5 |
|  | 16QAM | 2.5 | 5.0 | 2.0 | 6.5 | 12.0 | 3.5 |
|  | 64QAM |  | 5.0 |  | 6.5 | 12.0 | 3.5 |
|  | 256QAM |  |  |  | 6.5 | 12.0 |  |
| CP-OFDM | QPSK | 3.5 | 6.5 |  | 8.0 | 14.0 | 4.5 |
|  | 16QAM | 3.5 | 6.5 |  | 8.0 | 14.0 | 4.5 |
|  | 64QAM | 3.5 | 6.5 |  | 8.0 | 14.0 | 4.5 |
|  | 256QAM |  | 6.5 |  | 8.0 | 14.0 |  |

Table 2.1.3-2: A-MPR values for NS_04 (ETSI lower sub-range)

| Channel BW | Carrier Center Frequency | RB_start*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 5MHz | 1612.5 <= fc < 1613.9 | <= 0.36 | <= 0.36 | A1 |
|  |  |  | >= 2.88 | A2 |
|  | 1613.9 <= fc < 1615.7 |  | >= 3.24 | A3 |

|  | Modulation | A1 | A2 | A3 |
| --- | --- | --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK | 2.5 | 3.5 | 3.0 |
|  | QPSK | 2.5 | 3.5 | 3.0 |
|  | 16QAM | 2.5 | 3.5 | 3.0 |
|  | 64QAM |  | 3.5 | 3.0 |
|  | 256QAM |  |  |  |
| CP-OFDM | QPSK | 3.5 | 6.5 | 4.5 |
|  | 16QAM | 3.5 | 6.5 | 4.5 |
|  | 64QAM | 3.5 | 6.5 | 4.5 |
|  | 256QAM |  |  |  |

Table 2.1.3-3: A-MPR values for NS_05 (ETSI upper sub-range)

| Channel BW | Carrier Center Frequency | RB_start*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 5MHz | 1622.4 < fc <= 1624 | <= 3.6 | > 0.36 | A3 |
|  |  |  | >= 2.88 | A1 |
| 10MHz | 1615 <= fc < 1620.1 | <= 1.8 | <= 5.04 | A4 |
|  |  | <= 1.8 | > 5.04 | A5 |
|  |  | > 7.2 | > 0 | A6 |
|  |  | > 1.8 | >= 2.88 | A2 |
|  | 1620.1 <= fc <= 1621.5 |  | <= 7.2 | A6 |
|  |  | <= 0.36 | <= 0.36 | A1 |
|  |  | > 7.2 | > 0 | A6 |
| 15MHz | all | <= 3.6 | <= 5.04 | A4 |
|  |  | <= 3.6 | > 5.04 | A5 |
|  |  | > 10.44 |  | A6 |
|  |  | > 3.6 | >= 4.32 | A2 |

|  | Modulation | A1 | A2 | A3 | A4 | A5 | A6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK | 2.5 | 5.0 | 1.5 | 6.5 | 12.0 | 3.5 |
|  | QPSK | 2.5 | 5.0 | 1.5 | 6.5 | 12.0 | 3.5 |
|  | 16QAM | 2.5 | 5.0 | 2.0 | 6.5 | 12.0 | 3.5 |
|  | 64QAM |  | 5.0 |  | 6.5 | 12.0 | 3.5 |
|  | 256QAM |  |  |  | 6.5 | 12.0 |  |
| CP-OFDM | QPSK | 3.5 | 6.5 |  | 8.0 | 14.0 | 4.5 |
|  | 16QAM | 3.5 | 6.5 |  | 8.0 | 14.0 | 4.5 |
|  | 64QAM | 3.5 | 6.5 |  | 8.0 | 14.0 | 4.5 |
|  | 256QAM |  | 6.5 |  | 8.0 | 14.0 |  |

## 2.2	Rx requirements

### 2.2.1	Out-of-band blocking requirements

There already exist out-of-band blocking requirements for the terrestrial band n53 defined for almost the same frequency range as the new NTN band (see Figure 2.2.2-1 below for the graphical representation of these requirements). Since from the viewpoint of the UE implementation, it does not fundamentally matter whether the wanted signal comes from the terrestrial or the satellite base station, it is possible to re-use existing band n53 requirements with the only difference is that the upper band edge is updated to 2500MHz.

Figure 2.2.1-1: Summary of the band n53 out-of-band blocking requirements.

Proposal 4:	Existing band n53 out-of-band blocking requirements (with the changed upper edge to 2500MHz) can be applied to the new NTN band.

The table below is the corresponding excerpt from the draft running CR illustrating how the out-of-band blocking requirements can be implemented in the specification.

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| [n254]2 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| n255 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| n2561 | Finterferer (CW) | MHz | -100 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -145 < f – FDL_low ≤ -100 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 145 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: Band n256 lower frequency ranges are modified to enable specific implementations void NOTE 2: Band [n254] power level of the interferer (Pinterferer) for Range 3 shall be modified to -20 dBm for Finterferer > 2580 MHz and FInterferer < 2775 MHz. NOTE 3: void NOTE 4: void |  |  |  |  |  |

# 3	Conclusions

In this paper we have presented our further considerations on remaining open issues for the RF requirements, such as NS values, A-MPR, and out-of-band blocking requirements. The corresponding changes to TS 38.101-5 can be found in [5].

Proposal 1:	Specify two NS flags, NS_04N and NS_05N, to support ETSI regulations for the NTN L-/S-band.

Proposal 2:	Capture additional power back-off simulation results in the NTN L-/S-band TR.

Proposal 3:	Capture proposed A-MPR values for NS_03, NS_04 and NS_05.

Proposal 4:	Existing band n53 out-of-band blocking requirements (with the changed upper edge to 2500MHz) can be applied to the new NTN band.

# 4	References

RP-193234, "Solutions for NR to support non-terrestrial networks (NTN)", Thales

RP-223431, "New WID on Introduction of the satellite L-/S-band", Globalstar, Apple

ETSI EN 301 441, "Satellite Earth Stations and Systems (SES); Harmonised Standard for Mobile Earth Stations (MES), including handheld earth stations, for Satellite Personal Communications Networks (S-PCN) operating in the 1,6 GHz/2,4 GHz frequency band under the Mobile Satellite Service (MSS) covering the essential requirements
of article 3.2 of the Directive 2014/53/EU", v2.1.1, June 2016

R4-2314710, "WF on NTN L-/S-band", Apple

R4-2315367, "Draft running CR on Introduction of the NTN L-/S-band to TS 38.101-5", Apple, Globalstar

R4-2311216, "RF requirements for the NTN L-/S-band", Apple, Globalstar

Appendix A1: Power back-off simulation results for 5MHz

Figure A1-1: Power back-off values for the 5MHz channel at Fc=1613.9MHz.

Figure A1-2: Power back-off values for the 5MHz channel at Fc=1622.4MHz.