3GPP TSG RAN WG4 Meeting #108bis		R4-2315370

Xiamen, China, October 9 – 13th 2023

Agenda item:	5.4.1.1

Source:	Apple

Title:	On design options for enhanced channel raster

WI/SI:	NR_channel_raster_enh

Release:	Rel-18

Document for:	Decision

# 1	Introduction

During Rel-16 and Rel-17 discussions, several operators expressed an interest in enabling more efficient utilization of "non-standard" channel bandwidths, i.e., the ones which are not present now in TS 38.101 specifications. Referring to the corresponding operator requests, the following channel bandwidths were suggested by operators: 6, 7, 11, 12, 13, 33MHz. As an outcome a new SI was agreed at the RAN#89 meeting aiming to study further which existing solutions can be used and whether new mechanism should be devised [1]. The SI was concluded at RAN#99 meeting with the general conclusions that two methods – overlapping channels from network perspective and the next larger channel – can be used to support irregular channels without requiring changes at the UE side. In addition to that, companies concluded that the existing NR channel raster design is not flexible and does not allow configuring certain NR channel combinations. In response to that RAN#99 agreed a new WI with the only main objective to enable a more flexible raster design [2].

During the previous RAN WG4 several solutions were discussed, which at the end were structured around two major approaches. In this discussion paper we elaborate further on proposed approach suggesting proceeding with an explicit channel raster defined at least for UE side.

# 2	Enhanced channel raster

## 2.1	General considerations

As discussed during previous RAN WG4 meetings, there are two major approaches for implementing enhanced channel raster: a) introduce explicitly new channel raster points with step of e.g. 10kHz, and b) introduce the "offraster" concept in which a UE channel can be on a raster different than the existing 100kHz.

Our view is that from a UE implementation perspective these two approaches are not different if the "offraster" concept still sets some limits or steps in which a UE channel can be offset from the 100kHz raster. It is worth noting that it would not be possible to assume that the "offraster" UE channel location does not have any limits. From the UE implementation perspective, it will mean that the channel can be almost on any raster, which is somewhat against existing RAN WG4 specifications. And once we set limits or constraints to the "offraster" UE channel, it will define a set of applicable channel raster points, which is the same as defining a more granular channel raster at least for the UE side. In general, a UE does not and should not care about the BS channel because the latter can be of the size that is not even supported by the UE. Thus, it is not reasonable to define enhanced channel raster as some offset from 100kHz; instead, it should be specified as an explicit set of more granular channel raster points.

Proposal:	Introduce enhanced channel raster as a more granular step size of the applicable channel raster points.

## 2.2	Specification impact

Referring to the considerations presented above in section 2.1, introduction of the enhanced channel raster to the RAN WG4 specifications should be rather trivial. There is already a table summarising applicable channel raster points (see Appendix A), where for every band we specify the starting and the ending point, and the step size. For all FR1 bands below 3GHz the step size is defined as 20, i.e. 20x5kHz=100kHz. If the intention is to enable a more granular channel raster, e.g. 10kHz, then the corresponding step size should be changed to 2. In fact, there are already bands for which the channel raster in steps of 2 and even 1, so applying a more granular step size should not be an issue from the specification perspective.

# 3	Conclusions

In this discussion paper we have presented our further considerations on the design of the flexible channel raster. As summarized in our paper, the easiest way to enable a more flexible raster is to specify a more granular step size in the table that specifies applicable raster points to the existing bands. In fact, there are already existing bands for which the step size is defined as 1 and 2.

Proposal:	Introduce enhanced channel raster as a more granular step size of the applicable channel raster points.

# 4	References

RP-202103, "New SID: Study on Efficient utilization of licensed 	spectrum that is not aligned with existing NR channel bandwidths", T-Mobile USA, Ericsson

RP-230813, "New WID on Channel raster enhancement", Ericsson

Appendix A: NR bands channel raster points

| NR operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n1 | 100 | 384000 – <20> – 396000 | 422000 – <20> – 434000 |
| n2 | 100 | 370000 – <20> – 382000 | 386000 – <20> – 398000 |
| n3 | 100 | 342000 – <20> – 357000 | 361000 – <20> – 376000 |
| n5 | 100 | 164800 – <20> – 169800 | 173800 – <20> – 178800 |
| n7 | 100 | 500000 – <20> – 514000 | 524000 – <20> – 538000 |
| n8 | 100 | 176000 – <20> – 183000 | 185000 – <20> – 192000 |
| n12 | 100 | 139800 – <20> – 143200 | 145800 – <20> – 149200 |
| n13 | 100 | 155400 – <20> – 157400 | 149200 – <20> – 151200 |
| n14 | 100 | 157600 – <20> – 159600 | 151600 – <20> – 153600 |
| n18 | 100 | 163000 – <20> – 166000 | 172000 – <20> – 175000 |
| n20 | 100 | 166400 – <20> – 172400 | 158200 – <20> – 164200 |
| n24 | 100 | 325300 – <20> – 332100 | 305000 – <20> – 311800 |
| n25 | 100 | 370000 – <20> – 383000 | 386000 – <20> – 399000 |
| n26 | 100 | 162800 – <20> – 169800 | 171800 – <20> – 178800 |
| n28 | 100 | 140600 – <20> – 149600 | 151600 – <20> – 160600 |
| n29 | 100 | N/A | 143400 – <20> – 145600 |
| n30 | 100 | 461000 – <20> – 463000 | 470000 – <20> – 472000 |
| n34 | 100 | 402000 – <20> – 405000 | 402000 – <20> – 405000 |
| n38 | 100 | 514000 – <20> – 524000 | 514000 – <20> – 524000 |
| n39 | 100 | 376000 – <20> – 384000 | 376000 – <20> – 384000 |
| n40 | 100 | 460000 – <20> – 480000 | 460000 – <20> – 480000 |
| n41 | 15 | 499200 – <3> – 537999 | 499200 – <3> – 537999 |
|  | 30 | 499200 – <6> – 537996 | 499200 – <6> – 537996 |
| n462 | 15 | 743334 – <1> – 795000 | 743334 – <1> – 795000 |
| n47 | 15 | 790334 – <1> – 795000 | 790334 – <1> – 795000 |
| n48 | 15 | 636667 – <1> – 646666 | 636667 – <1> – 646666 |
|  | 30 | 636668 – <2> – 646666 | 636668 – <2> – 646666 |
| n50 | 100 | 286400 – <20> – 303400 | 286400 – <20> – 303400 |
| n51 | 100 | 285400 – <20> – 286400 | 285400 – <20> – 286400 |
| n53 | 100 | 496700 – <20> – 499000 | 496700 – <20> – 499000 |
| n54 | 100 | 334000 – <20> – 335000 | 334000 – <20> – 335000 |
| n65 | 100 | 384000 – <20> – 402000 | 422000 – <20> – 440000 |
| n66 | 100 | 342000 – <20> – 356000 | 422000 – <20> – 440000 |
| n67 | 100 | N/A | 147600 – <20> – 151600 |
| n70 | 100 | 339000 – <20> – 342000 | 399000 – <20> – 404000 |
| n71 | 100 | 132600 – <20> – 139600 | 123400 – <20> – 130400 |
| n74 | 100 | 285400 – <20> – 294000 | 295000 – <20> – 303600 |
| n75 | 100 | N/A | 286400 – <20> – 303400 |
| n76 | 100 | N/A | 285400 – <20> – 286400 |
| n77 | 15 | 620000 – <1> – 680000 | 620000 – <1> – 680000 |
|  | 30 | 620000 – <2> – 680000 | 620000 – <2> – 680000 |
| n78 | 15 | 620000 – <1> – 653333 | 620000 – <1> – 653333 |
|  | 30 | 620000 – <2> – 653332 | 620000 – <2> – 653332 |
| n79 | 15 | 693334 – <1> – 733333 | 693334 – <1> – 733333 |
|  | 30 | 693334 – <2> – 733332 | 693334 – <2> – 733332 |
| n80 | 100 | 342000 – <20> – 357000 | N/A |
| n81 | 100 | 176000 – <20> – 183000 | N/A |
| n82 | 100 | 166400 – <20> – 172400 | N/A |
| n83 | 100 | 140600 – <20> –149600 | N/A |
| n84 | 100 | 384000 – <20> – 396000 | N/A |
| n85 | 100 | 139600 – <20> – 143200 | 145600 – <20> – 149200 |
| n86 | 100 | 342000 – <20> – 356000 | N/A |
| n89 | 100 | 164800 – <20> – 169800 | N/A |
| n90 | 15 | 499200 – <3> – 537999 | 499200 – <3> – 537999 |
|  | 30 | 499200 – <6> – 537996 | 499200 – <6> – 537996 |
|  | 100 | 499200 – <20> – 538000 | 499200 – <20> – 538000 |
| n91 | 100 | 166400 – <20> – 172400 | 285400 – <20> – 286400 |
| n92 | 100 | 166400 – <20> – 172400 | 286400 – <20> – 303400 |
| n93 | 100 | 176000 – <20> – 183000 | 285400 – <20> – 286400 |
| n94 | 100 | 176000 – <20> – 183000 | 286400 – <20> – 303400 |
| n95 | 100 | 402000 – <20> – 405000 | N/A |
| n963 | 15 | 795000 – <1> – 875000 | 795000 – <1> – 875000 |
| n97 | 100 | 460000 – <20> – 480000 | N/A |
| n98 | 100 | 376000 – <20> – 384000 | N/A |
| n99 | 100 | 325300 – <20> – 332100 | N/A |
| n100 | 100 | 174880 – <20> – 176000 | 183880 – <20> – 185000 |
| n101 | 100 | 380000 – <20> – 382000 | 380000 – <20> – 382000 |
| n1024 | 15 | 795000 – <1> – 828333 | 795000 – <1> – 828333 |
| n104 | 15 | 828334 – <1> – 875000 | 828334 – <1> – 875000 |
|  | 30 | 828334 – <2> – 875000 | 828334 – <2> – 875000 |
| n105 | 100 | 132600 – <20> – 140600 | 122400 – <20> – 130400 |
| NOTE 1: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. NOTE 2: The following NREF are allowed for operation in Band n46: see Table 5.4.2.3-2. NOTE 3: The following NREF are allowed for operation in Band n96: see Table 5.4.2.3-3. NOTE 4: The following NREF are allowed for operation in Band n102: see Table 5.4.2.3-4. |  |  |  |