3GPP TSG-RAN WG4 Meeting # 108-bis		R4-2315375

Xiamen, China, October 9 – 13, 2023

Agenda item:	4.22.2

Source:	Apple

Title:	A-MPR discussion for FDD HPUE NS_46

WI/SI:	HPUE_NR_FR1_FDD_R18

Release:	Rel-18

Document for:	Approval

# 1	Introduction

The WID [1] aims to improve the uplink coverage by introducing PC2 to several FDD bands. This contribution focuses on the network signalling flag NS_46 for band n7. This contribution provides simulation results and makes proposals based on original proposal from [2].

# 2  Discussion

###### 2.1 Simulation assumptions

The following assumptions and requirements for the simulations are used:

- Power Class 2

- NR Waveform

- Calibration: 1dB MPR: DFT-s-OFDM QPSK 20MHz, 100RB0

- Carrier Leakage: 28dBc

- Image: 28dBc

- CIM3: 60dBc

- CIM5: 70dBc

Special requirements:

###### 2.2 Simulation results

According to our simulation results the original proposed A-MPR regions and values from [2] are mostly fine. There is only one tweak required. The Table 1 and Table 2 are based on the proposals from [2]. Changes made to the original proposal are indicated by red colour.

Table 1: A-MPR regions for NS_46 for PC2 (red colour indicates changes to original proposal from [2])

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz |  |
| 25 MHz | 2512.5 ≤ FC ≤ 2557.5 | < 14.4 | >max (0, 12*SCS*RBend - 2.7) | A5 |
|  |  | ≥ 14.4, <18.9 | > 9.9 | A7 |
|  |  | ≥ 18.9 | ≥ 7.2 | A9 |
|  |  | ≥ 17.64 | < 7.2 | A10 |
| 30 MHz | 2515 ≤ FC ≤ 2555 | ≥0, <1.44 | >0 | A4 |
|  |  | ≥1.44, <13.5 | >max (0, 12*SCS*RBend -1.8) | A5 |
|  |  | ≥13.5, <19.8 | >11.52 | A6 |
|  |  | ≥19.8, <25.92 | >6.3 | A7 |
|  |  | ≥25.92 | >0 | A8 |
|  |  | > 20.7 , < 25.92 | ≤ 6.3 | A10 |
| 35 MHz | 2517.5 ≤ FC ≤ 2552.5 | ≥0, <3.42 | >0 | A4 |
|  |  | ≥3.42, <15.84 | >max (0, 12*SCS*RBend - 3.06) | A5 |
|  |  | ≥15.84, <22.68 | >12.6 | A6 |
|  |  | ≥22.68, <28.8 | >9.0 | A7 |
|  |  | ≥28.8 | >0 | A8 |
|  |  | > 24.3 , < 28.8 | ≤ 9.0 | A10 |
| 40 MHz | 2520 ≤ FC ≤ 2550 | ≥0, <4.5 | >0 | A4 |
|  |  | ≥4.5, <18 | >max (0, 12*SCS*RBend - 4.5) | A5 |
|  |  | ≥18, <25.74 | >13.5 | A6 |
|  |  | ≥25.74, <32.4 | >12.6 | A7 |
|  |  | ≥32.4 | >0 | A8 |
|  |  | > 27.9 , < 32.4 | ≤ 12.6 | A10 |
| 50 MHz | 2525 ≤ FC ≤ 2545 | ≥0, <9 | >0 | A4 |
|  |  | ≥9, <21.6 | >max (0, 12*SCS*RBend - 7.2) | A5 |
|  |  | ≥21.6, <31.5 | >18 | A6 |
|  |  | ≥31.5, <39.6 | >16.2 | A7 |
|  |  | ≥39.6 | >0 | A8 |
|  |  | > 33.84, < 39.6 | ≤ 16.2 | A10 |

Table 2: A-MPR for NS_46 for PC2

| Modulation/Waveform |  | A4 | A5 | A6 | A7 | A8 | A9 | A10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | 6.5 | 2.5 | 3.5 | 6 | 10 | 6.5 | 3.5 |
|  | QPSK | 6.5 | 2.5 | 3.5 | 6 | 10 | 6.5 | 3.5 |
|  | 16 QAM | 6.5 | 2.5 | 3.5 | 6 | 10 | 6.5 | 3.5 |
|  | 64 QAM | 6.5 | 2.5 | 3.5 | 6 | 10 | 6.5 | 3.5 |
|  | 256 QAM | 6.5 |  |  | 6 | 10 | 6.5 |  |
| CP-OFDM | QPSK | 6.5 | 3.5 | 5.5 | 7 | 11 | 8 | 4.5 |
|  | 16 QAM | 6.5 | 3.5 | 5.5 | 7 | 11 | 8 | 4.5 |
|  | 64 QAM | 6.5 | 3.5 | 5.5 | 7 | 11 | 8 | 4.5 |
|  | 256 QAM | 6.5 |  |  | 7 | 11 | 8 |  |

Proposal 1: Consider Table 1 and Table 2 when defining requirements for PC2.

For PC3 special restriction are defined for channel bandwidth sizes of 20MHz and below. The RB size are limited to a maximum of 54 in case the channel is close to the lower band edge. During last RAN4 meeting an idea came up to remove the restriction for PC2 and define A-MPR. While this idea might be useful for deployment as operators could decide on the field whether to use allocation sizes larger than 54RBs it creates issues in case a UE needs to fall back from PC2 to PC3. In this scenario the UE has no A-MPR requirements and is not allowed to transmit with RB size larger than 54. Introducing A-MPR requirements for PC3 could create issues with legacy UEs. For simplicity and to avoid inconsistencies in case of power class fallback re-use the PC3 RB restrictions for PC2.

Proposal 2: For simplicity and to avoid inconsistencies in case of power class fallback re-use the PC3 RB restrictions for PC2.

# 4	Conclusions

This contribution provides A-MPR simulation results. The following proposals are made:

Proposal 1: Consider Table 1 and Table 2 when defining requirements for PC2.

Proposal 2: For simplicity and to avoid inconsistencies in case of power class fallback re-use the PC3 RB restrictions for PC2.

# 4	References

- RP‑231064, “Revised WID: High power UE (power class 2) for NR FR1 FDD single band“, China Unicom, 3GPP TSG RAN meeting #100

- R4-2313548, “PC2 A-MPR and RSD for bands n7 and n70”, Qualcomm Inc., WG4 Meeting#108