3GPP TSG-RAN WG4 Meeting # 108-bis		R4-2315379

Xiamen, China, October 9 – 13, 2023

Agenda item:         4.1.1.1

Source:                  Apple

Title:                      MSD analysis for CA_n5B-n12,  CA_n5B-n14, and CA_n5B-n29 with CA_n5B UL

WI/SI:                    NR_CADC_R18_2BDL_xBUL

Release:                 Rel-18

Document for:      Discussion

# 1	Introduction

In the previous RAN4 meeting (#108, Toulouse, France), discussions were held on specifying the MSD values for the three CA combinations CA_n5B-n12, CA_n5B-14, and CA_n5B-n29 with CA_n5B UL, based on the summitted proposal described in [1]. The resulting WF, as captured in [2] is summarized in Table 1-1 shown below:

<Way forward >: - The following option is presented as a WF. - Option 1: Use UL configuration 1RB+ 1RB (Equal PSD) to align with existing specification in 38.101-1 - Finalize IMD order and DL frequency test point if necessary. Adjust RB start position if necessary to center IMD noise. Determine MSD Band / Channel bandwidth / NRB / Duplex mode; Source of IMD. NR CA band combination; NR band; UL Fc (MHz); UL/DL BW (MHz); UL CLRB; DL Fc (MHz); MSD (dB); Duplex mode. CA_n5-n12; n5; 829; 10; 1 (RBSTART=[0]); 874; N/A; FDD; N/A. 839; 10; 1 (RBSTART=[52]); 884; N/A. n12; N/A; 5; N/A; [743.5]; TBD; FDD; [IMD9]. CA_n5-n14; n5; 829; 10; 1 (RBSTART=[0]); 874; N/A; FDD; N/A. 839; 10; 1 (RBSTART=[52]); 884; N/A. n14; N/A; 5; N/A; [765.5]; TBD; FDD; [IMD7]. CA_n5-n29; n5; 829; 10; 1 (RBSTART=[0]); 874; N/A; FDD; N/A. 839; 10; 1 (RBSTART=[52]); 884; N/A. n29; N/A; 5; N/A; [725.5]; TBD; FDD; [IMD11]. - Option 2: Other options not precluded for further information.

Table 1-1: WF summary for CA_n5B-12, CA_n5B-n14, and CA_n5B-29 from RAN4 # 108 meeting

In this contribution paper, we will use the test points highlighted in the table above to derive the MSD requirements due to the IMDs generated by the intra band CA_n5B UL.

# Discussion

For MSD analysis, we will rely on the 2In the RAN plenary meeting #100 in Taipei, Taiwan, new LBLB inter-band BCS were introduced with CA_n5B as DL and UL configurations. The requested BCS can be seen in Table 2-1 below.

| CA combination | UL CA configuration | NR band | Channel BW (MHz) | Bandwidth combination set ( BCS ) |
| --- | --- | --- | --- | --- |
| CA_n5B-n12A | CA_n5B CA_n5A-n12A | n5 | CA_n5B_BCS0 | 0 |
|  |  | n12 | 5, 10, 15 |  |
| CA_n5B-n14A | CA_n5B CA_n5A-n14A | n5 | CA_n5B_BCS0 | 0 |
|  |  | n14 | 5, 10 |  |
| CA_n5B-n29A | CA_n5B | n5 | 2 | 0 |
|  |  | n29 | 5, 10 |  |

Table 2-1: Requested BCS

The two UL CCs within the C_n5B UL configurations could generate IMDs that may affect the receiver of the victim bands. This therefore requires generating new MSD’s due to those IMDs, which is the goal of this of this discussion paper.

The IMD orders of interest are based on CA_n5B BCS0 request which is 20MHz maximum aggregated bandwidth:

- CA_n5B-n12A: n12 is subject to IMD7 of the two CA_n5B CCs RB allocation

- CA_n5B-n14A: n14 is subject to the end of IMD9 plus IMD11 of the two CA_n5B CCs RB allocation

- CA_n5B-n29A: n29 is subject to IMD11 of the two CA_n5B CCs RB allocation.

  - MSD Analysis

For the MSD analysis, we will rely on the 2-antenna architecture with triplexers which is the best compromise for size and performance, critical for UE implementations as was concluded during the study phase of the 700/800/900 WID [3] [4] [5] [6]. The three RF front-end diagrams for the 3 combos are captured in the figure below.

Figure 2.1-1: Front-end RF architecture for CA_n5B-n12A, CA_n5B-14A, and CA_n5B-n29A

The analysis will be based on the following worst-case assumptions for the RF front-end performance parameters:

- TX to RX isolation: 40dB

- Tx-Antenna Rejection: 35dB

- Antenna coupling: 10dB

- RxBW: 5MHz

- Output Power at antenna (PC3): 23dBm

- Post-antenna front-end loss: 4dB

Given the assumptions above, the MSD analysis is summarized in the table below:

| Parameter | Unit | CA_n5B-n29 |  | CA_n5B-n12 |  | CA_n5B-n14 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Main | Div. | Main | Div. | Main | Div. |
| CBW | MHz | 5 |  | 5 |  | 5 |  |
| Simulated Tx noise @ PA Output | dBm/CBW | -36.0 | -46 | -33 | -43 | -31 | -41 |
| Tx-Rx iso / Tx-Ant rejection | dB | 40 | 37 | 40 | 37 | 40 | 37 |
| Tx noise @ antenna | dBm | -80 | -87 | -77 | -84 | -75 | -82 |
| 3GPP 5MHz CBW REFSENS | dBm/CBW | -97.0 |  | -97.0 |  | -97.0 |  |
| SNR | dB | -1.0 | -1.0 | -1.0 | -1.0 | -1.0 | -1.0 |
| Thermal Noise | dBm | -96 | -96 | -96 | -96 | -96 | -96 |
| Composite Noise | dBm | -79.8 | -86.4 | -76.9 | -83.7 | -74.9 | -81.8 |
| Composite noise after MRC | dBm | -87.2 |  | -84.5 |  | -82.6 |  |
| MSD | dB | 8.8 |  | 11.5 |  | 13.3 |  |

Figure 2.1-1: MSD analysis for CA_n5B-n12A, CA_n5B-14A, and CA_n5B-n29A

# 3	Conclusion

In this contribution, we shared our analysis results of the MSD due to IMDs of CA_n5B UL for the LBLB combinations CA_n5B-n12A, CA_n5B-n14A and CA_n5B-n29A and make the following proposal:

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n5-n12 | n5 | 829 | 10 | 1 (RBSTART=[0]) | 874 | N/A | FDD | N/A |
|  |  | 839 | 10 | 1 (RBSTART=[52]) | 884 | N/A |  |  |
|  | n12 | N/A | 5 | N/A | 743.5 | 11.5 | FDD | IMD9 |
| CA_n5-n14 | n5 | 829 | 10 | 1 (RBSTART=[0]) | 874 | N/A | FDD | N/A |
|  |  | 839 | 10 | 1 (RBSTART=[52]) | 884 | N/A |  |  |
|  | n14 | N/A | 5 | N/A | 765.5 | 13.3 | FDD | IMD7 |
| CA_n5-n29 | n5 | 829 | 10 | 1 (RBSTART=[0]) | 874 | N/A | FDD | N/A |
|  |  | 839 | 10 | 1 (RBSTART=[52]) | 884 | N/A |  |  |
|  | n29 | N/A | 5 | N/A | 725.5 | 8.8 | FDD | IMD11 |

Table 3-1: Proposal for MSD based on 10RB+10RB UL allocation

# 4	References

R4-2311269, CA_n5B ULCA MSD due to IMD for CA_n5B-n12 CA_n5B-n14 and CA_n5B-n29, Skyworks

R4-2314856, WF on CA_n5B-n12 CA_n5B-n14 and CA_n5B-n29 MSDs with CA_n5B UL, Skyworks

R4-2306470 WF on RF architecture and critical issues for CA_n28-n105, Apple

R4-2306469 WF on RF architecture and critical issues for CA_n5-n105, Xiaomi

R4-2306471 WF on RF architecture and critical issues for CA_n26-n28, Huawei

R4-2307546 Big CR for TR 38.872 to introduce outcomes of CA_n5-n105 CA_n28-n105 CA_n26-n28 in sub-1GHz SI, Huawei, Hisilicon