3GPP TSG-RAN WG4 Meeting #108-bis		R4-2315380

Xiamen, China, October 9 – 13, 2023

Agenda item:        4.9.2

Source:                 Apple

Title:                     MSD analysis for CA_n5B

WI/SI:                   NR_CA_R18_intra-Core

Release:                Rel-17

Document for:     Discussion

# 1	Introduction

In the previous RAN4 meeting (#108, Toulouse, France), proposals were made on the MSD value for the CA combo CA_n5B. After discussions, an agreement was made to add only one BCS1 test point as captured in the WF summary below [1]:

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | UL PCC allocation (LCRB) | UL SCC allocation (LCRB) | PCC ΔRIBC (dB) | SCC ΔRIBC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n5B | 15/15 | 10MHz + 10MHz | 10 (RBSTART = 0) | 10 (RBSTART = 42) | 30.8 | 26.1 | FDD |
| CA_n5Bx | 15/15 | 5MHz + 20MHz | 4 (RBSTART = 0) | 16 (RBSTART = 90) | [FFS] | [FFS] | FDD |
| NOTE 1: All combinations of channel bandwidths defined in Table 5.5A.1-1. NOTE 2: The carrier centre frequency of SCC in the UL operating band is configured closer to the DL operating band. NOTE 3: The transmitted power over both PCC and SCC shall be set to PUMAX as defined in subclause 6.2A.4. NOTE 4: The PCC allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.3.2-1. NOTE X: Applicable only to BCS 1. |  |  |  |  |  |  |  |

In this contribution paper, we will use the test points highlighted in the table above  to derive the MSD requirements for the due CA_n5B due  to the IMDs generated by the two intra-band CCs within the CA_n5B UL. As can be seen in the WF, we will focus on the MSD with total aggregated CBW of 26MHz (5MHz +20MHz)

# Discussion

The requested bandwidth classes (BCS) for the CA_n5B is captured in table captured in Table 2-1 below:

| NR CA configuration | Uplink NR CA configurations | channel bandwidths for carrier [MHz] | channel bandwidths for carrier [MHz] | Maximum aggregated bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- |
| CA_n5B | CA_n5B | 5, 10, 15 | 5, 10, 15 | 20 | 0 |
|  |  | 5, 10, 15, 20 | 5, 10, 15, 20 | 25 | 1 |

Table 2-1: CA_n5B BCS

For BCS1 (5MHz+20MHz case),  SCC is victim of IMD3 overlap and PCC is victim of IMD5.

  - MSD Analysis

Noise levels measured in each CC Rx CBW (dBm/measurement bandwidth) and corresponding MSD levels are plotted in Figure 1-left and Figure 1-right respectively. MSD is evaluated using the following parameters:

- Power class 3 operation, Tx power level: 23dBm,

- Tx-Rx duplexer isolation: 50dB,

- Post-PA insertion losses: 4dB

- Primary antenna to diversity antenna isolation: 10dB

- SNR: -1 dB

Using the parameters above, the MSD are computed as numbers shown in the Table 2.1-1 below.

| Parameter | Unit | PCELL |  | SCELL |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Main | Div. | Main | Div. |
| Rx CBW | MHz | 5 |  | 5 |  |
| Simulated Tx noise @ PA Output | dBm/CBW | -1 | -11 | -8 | -18 |
| Tx-Rx isolation | dB | 50 | 50 | 50 | 50 |
| Tx noise @ antenna | dBm/CBW | -55 | -65 | -62 | -72 |
| 3GPP 5MHz CBW REFSENS | dBm/CBW | -97.0 |  | -97.0 |  |
| SNR | dB | -1.0 | -1.0 | -1.0 | -1.0 |
| Thermal Noise | dBm | -96 | -96 | -96 | -96 |
| Composite Noise | dBm | -55 | -65 | -62 | -72 |
| Composite noise after MRC | dBm | -65.4 |  | -72.4 |  |
| MSD | dB | 30.6 |  | 23.6 |  |

Table 2.1-1: CA_n5B IMD-based MSD analysis for PCELL and SCELL

# 3        Conclusions

In this contribution, we presented the results of our MSD analysis for the initial CA_n5B with 5MHz + 20MHz Tx BW combination and propose the following:

Proposal: Adopt CA_n5B MSD Levels proposed in Table 3-1 shown below.

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | UL PCC allocation (LCRB) | UL SCC allocation (LCRB) | PCC ΔRIBC (dB) | SCC ΔRIBC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n5Bx | 15/15 | 5MHz + 20MHz | 4 (RBSTART = 0) | 16 (RBSTART = 90) | 30.6 | 23.6 | FDD |
| NOTE 1: All combinations of channel bandwidths defined in Table 5.5A.1-1. NOTE 2: The carrier centre frequency of SCC in the UL operating band is configured closer to the DL operating band. NOTE 3: The transmitted power over both PCC and SCC shall be set to PUMAX as defined in subclause 6.2A.4. NOTE 4: The PCC allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.3.2-1. NOTE X: Applicable only to BCS 1. |  |  |  |  |  |  |  |

Table 3-1: CA_n5B MSD Levels for 5MHz +20MHz

# 4        References

[1] R4-2314855, WF on CA_n5B DL/UL BCS1 MSD test point, 3GPP TSG-RAN WG4 Meeting # 108. Toulouse, France