# 3GPP TSG-RAN WG4 Meeting # 108bis			    	   				    	  		        	 R4-2315418

# Xiamen, China, October 09 – October 13, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR |  | rev |  | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of interruprion requirements for inter-RAT NR measurement without gap (case a-1) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MG_enh2-Core |  |  |  |  |  | Date: |  |  | 2023-07-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Support for inter-RAT NR measurement without gap in release 18. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce interruprion requirements for inter-RAT NR measurement without gap (case a-1). Change #1: Clarify the applicability of the interruption requirements Change #2: Add the interruption requirement |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RRM requirements for inter-RAT NR measurement without gap will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

=================================Start of Change #1====================================

## 7.8	Interruptions with Carrier Aggregation

### 7.8.1	Introduction

This section contains the requirements related to the interruptions on PCell and activated SCell if configured, when up to six SCells are configured, deconfigured, hibernated, activated, dormant or deactivated, or when SRS carrier based switching is performed between the configured component carriers. Unless explicitly stated otherwise, the requirements in Section 7.8 shall apply for:

-	E-UTRA FDD CA,

-	E-UTRA TDD CA,

-	E-UTRA TDD-FDD CA,

-	inter-band CA where PCell is FDD or TDD and all the SCells are following the frame structure type 3 [16],

-	E-UTRA CA where at least one SCell is FeMBMS/Unicast-mixed SCell.

A UE causing interruptions during measurements on deactivated SCC shall indicate to the network a need for an interruption control pattern.

A UE causing interruptions during measurements on NR SCC shall indicate to the network that it is capable of inter-RAT NR measurements without gap but with interruption for the frequency band of the NR SCC according to the capability [interRAT-NeedForIntrNR-r18].

Note:	interruptions at SCell addition and release, activation, deactivation and hibernation and during measurements on SCC may not be required by all UEs.

Note:	interruptions during SRS carrier based switching between the configured component carriers may not be required by all UEs.

Editor’s Note:	The interruptions shall not interrupt RRC signalling or ACK/NACKs related to RRC reconfiguration procedure [2] for SCell addition, release and hibernation or MAC control signalling [17] for SCell activation, deactivation and hibernation command. How to specify this is FFS.

=================================End of Change #1====================================

=================================Start of Change #2====================================

7.8.2.21	Interruptions during measurements on NR SCC for inter-RAT CA

For a UE indicating ‘no-gap-with-interruption’ with the capability [interRAT-NeedForIntrNR-r18] for inter-RAT NR measurement, the UE is allowed to cause interruptions on PCell or activated SCell(s) due to inter-RAT NR measurements of:

-	up to [2.50%] probability of missed ACK/NACK when 80ms ≤ Tcycle < 160ms, or

-	up to [1.25%] probability of missed ACK/NACK when 160ms ≤ Tcycle < 320ms, or

-	up to [0.625%] probability of missed ACK/NACK when 320ms ≤ Tcycle.

When the measurements are pefermed on single MO, Tcycle is derived based on max(SMTC, 80ms) as specified in sub-clause [TBD]. When the measurements are pefermed on multiple MOs, Tcycle is FFS.

Editor’s Note: Impact from MG on Tcycle is FFS, and the requirements will be updated according to the final agreements.

The interruptions are allowed for all the serving cells in the same FR as NR SCell being measured if UE supports per-FR measurement gaps, and all the serving cells if UE does not support per-FR measurement gaps.

Each interruption shall not exceed X slot as define in table 7.8.2.21-1 and table 7.8.2.21-2:

Table 7.8.2.21-1: Interruption length X at inter-RAT NR measurements in FR1

| NR SCS (KHz) | Interruption length X (slots) |
| --- | --- |
| 15 | [TBD] |
| 30 | [TBD] |
| 60 | [TBD] |

Table 7.8.2.21-2: Interruption length X at inter-RAT NR measurements in FR2

| NR SCS (KHz) | Interruption length X (slots) |
| --- | --- |
| 60 | [TBD] |
| 120 | [TBD] |

=================================End of Change #2====================================